International Union of Geological Sciences

Manual of Standard Methods

for

Establishing the Global Geochemical Reference Network

Annexe A2.1

Geodetic Levelling of Existing Geochemical Data Sets

Michalis Salahoris

GeoSet Ltd., 60 Cyprus Street, GR-156 69 Papagos-Cholargos, Attiki, Hellas

2022

Published by

The International Union of Geological Sciences

Commission on Global Geochemical Baselines

It is recommended that reference to this part of the Manual should be made in the following way:

Salahoris, M., 2022. Geodetic Levelling of Existing Geochemical Data Sets. Annexe A2.1 in Chapter 2 In: Demetriades, A., Johnson, C.C., Smith, D.B., Ladenberger, A., Adánez Sanjuan, P., Argyraki, A., Stouraiti, C., Caritat, P. de, Knights, K.V., Prieto Rincón, G. & Simubali, G.N. (Editors), International Union of Geological Sciences Manual of Standard Methods for Establishing the Global Geochemical Reference Network. IUGS Commission on Global Geochemical Baselines, Athens, Hellenic Republic, Special Publication, 2, 27−39.

Published by

The International Union of Geological Sciences

Commission on Global Geochemical Baselines

P.O.Box 640 37, Zografou

GR-157 10 Athens

Hellenic Republic

http://www.iugs.org/

http://www.globalgeochemicalbaselines.eu/

National Library of Greece Cataloguing in Publication Data

A catalogue record for this electronic book is available from the

National Library of Greece

ISBN: 978-618-85049-1-2

Contents

A2.1.1. Geodetic levelling	31

A2.1.1.1. Main steps	31

A2.1.1.2. Example	31

A2.1.1.3. Comments	37

A2.1.2. Recommendations	37

Supplementary material	38

References	39

Blank back page

## A2.1.1. Geodetic levelling

The purpose of using this geodetic data processing methodology is the normalisation (or levelling) of the regional geochemical data set according to the reference values of the five random points, as defined within each 160x160 km grid cell of the Global Terrestrial Network (GTN) and described in Chapter 2 (refer to the Supplementary material).

The basic concept behind the method is that the 5 random points in the GTN grid cell are at such a density that they form a comparatively smooth field, without local extrema within the area defined by the three neighbouring GTN points and, therefore, it can be approximated as a plane.

The Delaunay triangulation is used in digital terrain models to generate a TIN data structure (Okabe et al., 2000) for levelling topographical survey measurements. This is the method that will be used for the geodetic levelling of regional geochemical data sets by using the GTN random point reference values.

### A2.1.1.1. Main steps

The method consists of the following steps:

- Define the triangulation irregular network from the points that cover the study area to define inclined planes. This is the network of 5 random points within each 160x160 km GTN grid cell.

- Calculate the vertical distance of each regional geochemical data point to the triangulation plane defined by the 5 random points in each 160x160 km grid cell in order to estimate the normalised elemental concentration value of each data point.

### A2.1.1.2. Example

The geodetic method is explained by using the Finnish regional geochemical nickel subsoil data and the FOREGS subsoil GTN reference data (Fig. A2.1.1; Salminen et al., 2005). It has been performed using ESRI’s ArcMap, v10.3.1 with active the 3D Analyst extension (FinlandExample.mxd; refer to Supplementary material).

Figure A2.1.1. The map shows the two Finnish 160x160 km GTN grid cells N34E09 and N34E10, and all neighbouring grid cells in Sweden and Norway, displaying with red triangles the 5 random reference points within each grid cell and their Ni concentrations in subsoil samples (see Table A2.1.1).

- Input data (see Supplementary material): - The file ‘Finland_Subsoil_AR_data_GTN_N43E09_&_N43E10_Nickel.xlsx’ with the Nickel aqua regia subsoil results of the 5 reference random sites within the GTN grid cells N43E09 and N43E10 (see Table A2.1.1). - The file ‘Finland_GTN_N43E09_&_N43E10_Nickel.xlsx’ with the regional Nickel aqua regia subsoil or C horizon till results (Tarvainen, 1995, 1996; refer to Annexe A2.2 for additional information). For every observation point provided, i.e., the PointID (field SampleNo), the following information is required: geographical coordinates in WGS 1984 (Easting and Northing – Longitude and Latitude), the GTN grid cell number that samples are within, the Sample Number, and the measurements of Ni concentrations (field Ni_511p in mg/kg), e.g.: Easting_WGS1984; Northing_WGS1984; GTN_No; SampleNo; Ni_511p. 24.87842726; 63.65765897; N43E09; 68003; 8.57. 25.18322365; 63.66680337; N43E09; 68004; 9.16. 25.21949996; 63.80274602; N43E09; 68005; 11.9. - The file “IUGS-CGGB_GTN_5-random-points_per_160x160km-grid-cell_v2.xlsx” downloadable from https://www.globalgeochemicalbaselines.eu/content/111/sampling-design-/ with the coordinates of each random point (Note: This concerns the levelling of regional geochemical data using new Global Geochemical Reference Network project results. In this example, the Ni reference values of the FOREGS Geochemical Atlas of Europe project (Salminen et al., 2005) have been used for the geodetic levelling of the Finnish subsoil Ni regional geochemical results).

- Data preparation: - Select the GTN random point that overlaps the regional geochemical data (observations). - Assign Ni values to each GTN random point of the above selection. - Create one feature class with all the geochemical data points (observations) to be levelled.

- Procedure: - Open an ArcMap document and define a cartesian coordinate system of the data frame (in our case “FinlanExample.mxd: EUREF FIN TM35FIN”). This is recommended in order to effectively enforce the Delaunay triangulation rule during the next step (https://desktop.arcgis.com/en/arcmap/latest/tools/3d-analyst-toolbox/create-tin.htm ) - Build a TIN using the GTN reference sample points with the Ni concentrations as elevation values. In ArcMap: From toolboxes, 3D Analyst, Create Tin. In our example is the “tinarsub_tt tin”. - Visualise the TIN triangles (Fig. A2.1.2). In our case using the command “TIN Edge” tool (https://desktop.arcgis.com/en/arcmap/latest/tools/3d-analyst-toolbox/tin-edge.htm). Note that the triangles extend beyond the two grid cells. - Add the feature classes with the observation points. In our case, the feature class is “fingtnall”. - Calculate the vertical distance of each observation point from the relevant tin plane. In our case, use the “Add Surface Information Tool” with Linear method, in field Z (Fig. A2.1.3; Tables A2.1.2 & A2.1.3). - Create a new field (in our case dz) and calculate the difference between the measured values (field Ni_511p) and the calculated values (field Z).

Figure A2.1.2. The map shows the TIN created using the 5 random points in GTN grid cells N34E09 and N34E10, Finland (see Tables A2.1.1 to A2.1.3).

Figure A2.1.3. Sample points with levelled (normalised) values. Positive values (red) when the point regional geochemical survey Ni concentration values are greater than the reference plane elevation at each point, and negative values (blue) when they are smaller (see Tables A2.1.1 to A2.1.3).

Table A2.1.1. Nickel concentrations in subsoil samples from the FOREGS N43E09 and N43E10 GTN grid cells, Finland, are used as the reference data set for levelling the regional subsoil geochemical data (File: Finland_Subsoil_AR_data_GTN_N43E09_&_N43E10_Nickel.xlsx).

| GTN: N43E09 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Longitude (o) | Latitude (o) | Sample No. | Country | Lab. No. | Ni (mg/kg) |
| 26.25 | 63.95 | N43E09C1 | FI | 99-000333 | 10.0 |
| 24.82 | 64.42 | N43E09C2 | FI | 99-000334 | 11.0 |
| 24.14 | 63.38 | N43E09C3 | FI | 99-000335 | 6.00 |
| 26.89 | 63.52 | N43E09C4 | FI | 99-000336 | 7.00 |
| 24.84 | 63.14 | N43E09C5 | FI | 99-000338 | 10.0 |
| GTN: N43E10 |  |  |  |  |  |
| 29.91 | 64.47 | N43E10C1 | FI | 99-000339 | 8.00 |
| 27.18 | 64.14 | N43E10C2 | FI | 99-000340 | 8.00 |
| 27.56 | 63.70 | N43E10C3 | FI | 99-000341 | 5.00 |
| 29.65 | 63.07 | N43E10C4 | FI | 99-000342 | 15.0 |
| 27.41 | 64.07 | N43E10C5 | FI | 99-000343 | 8.00 |

Table A2.1.2. Nickel concentrations in subsoil samples from the regional geochemical survey, Finland, in N43E09 GTN grid cell, the estimated Ni levelled (Z) data and the difference (dz) between the original and levelled values.

| Easting or Longitude (o) | Northing or Latitude (o) | GTN grid cell | Sample No. | Ni (mg/kg) | Ni (mg/kg) levelled (Z) | Ni (mg/kg) difference (dz) |
| --- | --- | --- | --- | --- | --- | --- |
| 24.88 | 63.66 | N43E09 | 68003 | 8.57 | 7.73 | 0.84 |
| 25.18 | 63.67 | N43E09 | 68004 | 9.16 | 8.01 | 1.15 |
| 25.22 | 63.80 | N43E09 | 68005 | 11.9 | 8.61 | 3.29 |
| 24.87 | 63.83 | N43E09 | 68006 | 5.79 | 8.48 | -2.69 |
| 25.27 | 63.97 | N43E09 | 68007 | 10.5 | 9.38 | 1.12 |
| 24.84 | 63.94 | N43E09 | 68008 | 10.9 | 8.92 | 1.98 |
| 26.48 | 64.31 | N43E09 | 68060 | 6.66 | 6.73 | -0.07 |
| 26.43 | 64.45 | N43E09 | 68061 | 4.47 | 6.28 | -1.81 |
| 24.25 | 63.25 | N43E09 | 68074 | 5.15 | 6.73 | -1.58 |
| 24.55 | 63.19 | N43E09 | 68075 | 4.64 | 8.39 | -3.75 |
| 24.13 | 63.35 | N43E09 | 68108 | 6.31 | 6.06 | 0.25 |
| 24.59 | 63.35 | N43E09 | 68109 | 7.21 | 7.78 | -0.57 |
| 26.52 | 63.81 | N43E09 | 68256 | 13.2 | 8.84 | 4.36 |
| 26.85 | 63.85 | N43E09 | 68257 | 9.34 | 7.81 | 1.53 |
| 26.85 | 63.96 | N43E09 | 68258 | 5.59 | 8.62 | -3.03 |
| 26.51 | 64.02 | N43E09 | 68259 | 9.64 | 9.31 | 0.33 |
| 26.49 | 64.13 | N43E09 | 68260 | 10.9 | 8.31 | 2.59 |
| 26.83 | 64.12 | N43E09 | 68263 | 7.53 | 8.26 | -0.73 |
| 26.82 | 64.30 | N43E09 | 68264 | 7.03 | 7.19 | -0.16 |
| 26.72 | 64.44 | N43E09 | 68265 | 6.32 | 7.01 | -0.69 |
| 25.28 | 63.18 | N43E09 | 68408 | 4.98 | 9.21 | -4.23 |
| 25.20 | 63.35 | N43E09 | 68409 | 8.27 | 9.99 | -1.72 |
| 24.87 | 63.33 | N43E09 | 68410 | 10.7 | 8.96 | 1.74 |
| 24.91 | 63.16 | N43E09 | 68411 | 8.64 | 9.92 | -1.28 |
| 24.86 | 63.03 | N43E09 | 68412 | 12.5 | 9.50 | 3.00 |
| 25.25 | 63.02 | N43E09 | 68414 | 5.33 | 8.64 | -3.31 |
| 24.60 | 63.04 | N43E09 | 68722 | 7.42 | 8.81 | -1.39 |
| 24.86 | 64.12 | N43E09 | 68723 | 28.7 | 9.73 | 18.97 |
| 25.35 | 64.11 | N43E09 | 68724 | 10.4 | 10.04 | 0.36 |
| 24.85 | 64.30 | N43E09 | 68780 | 10.9 | 10.49 | 0.41 |
| 24.90 | 64.46 | N43E09 | 68781 | 7.6 | 10.30 | -2.70 |
| 25.25 | 64.28 | N43E09 | 68782 | 12 | 10.68 | 1.32 |
| 25.28 | 64.45 | N43E09 | 68783 | 4.35 | 7.95 | -3.60 |
| 25.74 | 63.79 | N43E09 | 68784 | 11.9 | 9.20 | 2.70 |
| 25.71 | 63.95 | N43E09 | 68785 | 8.33 | 9.60 | -1.27 |
| 26.11 | 63.81 | N43E09 | 68786 | 9.6 | 9.74 | -0.14 |
| 26.02 | 63.96 | N43E09 | 68787 | 15.1 | 9.86 | 5.24 |
| 25.64 | 64.13 | N43E09 | 68788 | 10.5 | 10.35 | 0.15 |
| 26.06 | 64.14 | N43E09 | 68789 | 3.96 | 8.39 | -4.43 |
| 25.68 | 64.29 | N43E09 | 68790 | 5.76 | 8.20 | -2.44 |
| 25.61 | 64.45 | N43E09 | 68791 | 3.21 | 6.11 | -2.90 |
| 26.07 | 64.30 | N43E09 | 68792 | 4.75 | 6.88 | -2.13 |
| 26.11 | 64.46 | N43E09 | 68793 | 2.04 | 5.45 | -3.41 |
| 25.60 | 63.00 | N43E09 | 68794 | 3.25 | 7.86 | -4.61 |
| 25.81 | 63.01 | N43E09 | 68795 | 10.3 | 7.44 | 2.86 |
| 25.76 | 63.19 | N43E09 | 68798 | 6.4 | 8.22 | -1.82 |
| 26.07 | 63.21 | N43E09 | 68800 | 5.29 | 7.63 | -2.34 |
| 26.70 | 63.05 | N43E09 | 68803 | 4.37 | 7.29 | -2.92 |
| 26.92 | 63.13 | N43E09 | 68805 | 5.27 | 8.43 | -3.16 |
| 26.41 | 63.22 | N43E09 | 68806 | 4.27 | 6.93 | -2.66 |
| 26.44 | 63.39 | N43E09 | 68807 | 17.2 | 7.49 | 9.71 |
| 26.85 | 63.27 | N43E09 | 68808 | 9.57 | 7.53 | 2.04 |
| 26.79 | 63.41 | N43E09 | 68809 | 9.82 | 6.81 | 3.01 |
| 25.63 | 63.68 | N43E09 | 68811 | 13.2 | 9.52 | 3.68 |
| 26.04 | 63.70 | N43E09 | 68813 | 14.2 | 9.51 | 4.69 |
| 26.44 | 63.70 | N43E09 | 68815 | 11.1 | 8.65 | 2.45 |
| 26.84 | 63.48 | N43E09 | 68816 | 6.97 | 6.96 | 0.01 |

Table A2.1.3. Nickel concentrations in subsoil samples from the regional geochemical survey, Finland, in N43E10 GTN grid cell, the estimated Ni levelled (Z) data and the difference (dz) between the original and levelled values.

| Easting or Longitude (o) | Northing or Latitude (o) | GTN grid cell | Sample No. | Ni (mg/kg) | Ni (mg/kg) levelled (Z) | Ni (mg/kg) difference (dz) |
| --- | --- | --- | --- | --- | --- | --- |
| 27.21 | 63.48 | N43E10 | 68251 | 10.3 | 8.30 | 2.00 |
| 27.28 | 63.66 | N43E10 | 68252 | 13.5 | 5.91 | 7.59 |
| 27.55 | 63.52 | N43E10 | 68253 | 14.1 | 8.41 | 5.69 |
| 27.59 | 63.63 | N43E10 | 68254 | 24.9 | 6.40 | 18.50 |
| 27.26 | 63.84 | N43E10 | 68255 | 11.6 | 6.71 | 4.89 |
| 27.53 | 63.97 | N43E10 | 68261 | 6.45 | 7.10 | -0.65 |
| 27.31 | 63.94 | N43E10 | 68262 | 5.68 | 7.34 | -1.66 |
| 27.92 | 64.12 | N43E10 | 68268 | 17.1 | 7.72 | 9.38 |
| 27.96 | 63.97 | N43E10 | 68269 | 9.96 | 6.51 | 3.45 |
| 27.94 | 63.80 | N43E10 | 68270 | 4.87 | 5.70 | -0.83 |
| 27.61 | 63.84 | N43E10 | 68271 | 7.95 | 5.98 | 1.97 |
| 27.93 | 63.61 | N43E10 | 68272 | 7.52 | 6.63 | 0.89 |
| 27.88 | 63.48 | N43E10 | 68273 | 16.4 | 9.68 | 6.72 |
| 29.37 | 63.96 | N43E10 | 68274 | 8.99 | 9.23 | -0.24 |
| 29.04 | 63.95 | N43E10 | 68275 | 7.98 | 8.29 | -0.31 |
| 29.03 | 63.79 | N43E10 | 68276 | 6.04 | 9.09 | -3.05 |
| 28.63 | 63.78 | N43E10 | 68277 | 8.66 | 7.91 | 0.75 |
| 28.37 | 63.65 | N43E10 | 68278 | 6.91 | 7.82 | -0.91 |
| 28.32 | 63.51 | N43E10 | 68279 | 4.05 | 8.40 | -4.35 |
| 28.67 | 63.65 | N43E10 | 68280 | 19.4 | 8.75 | 10.65 |
| 29.02 | 63.53 | N43E10 | 68281 | 12.8 | 10.48 | 2.32 |
| 28.81 | 63.48 | N43E10 | 68282 | 13.5 | 10.11 | 3.39 |
| 29.04 | 63.63 | N43E10 | 68283 | 12.8 | 10.01 | 2.79 |
| 29.35 | 63.63 | N43E10 | 68284 | 9.73 | 10.95 | -1.22 |
| 29.36 | 63.51 | N43E10 | 68285 | 10.6 | 11.67 | -1.07 |
| 29.79 | 63.50 | N43E10 | 68286 | 8.82 | 12.56 | -3.74 |
| 30.12 | 63.52 | N43E10 | 68287 | 5.84 | 10.95 | -5.11 |
| 29.84 | 63.65 | N43E10 | 68288 | 7.22 | 11.69 | -4.47 |
| 29.85 | 63.82 | N43E10 | 68289 | 8.25 | 10.95 | -2.70 |
| 29.77 | 63.95 | N43E10 | 68290 | 11.1 | 10.50 | 0.60 |
| 30.13 | 64.15 | N43E10 | 68293 | 9.18 | 8.34 | 0.84 |
| 29.76 | 64.10 | N43E10 | 68294 | 10.6 | 9.60 | 1.00 |
| 29.39 | 64.09 | N43E10 | 68295 | 16.3 | 8.54 | 7.76 |
| 29.09 | 64.08 | N43E10 | 68296 | 11.9 | 7.71 | 4.19 |
| 29.45 | 63.81 | N43E10 | 68297 | 10.6 | 10.24 | 0.36 |
| 28.70 | 63.96 | N43E10 | 68298 | 8.13 | 7.14 | 0.99 |
| 28.29 | 63.84 | N43E10 | 68299 | 9.85 | 6.56 | 3.29 |
| 28.36 | 63.97 | N43E10 | 68300 | 12.6 | 6.07 | 6.53 |
| 28.30 | 64.09 | N43E10 | 68301 | 13.6 | 7.01 | 6.59 |
| 28.71 | 64.12 | N43E10 | 68302 | 13.3 | 6.76 | 6.54 |
| 29.02 | 64.26 | N43E10 | 68303 | 4.18 | 7.46 | -3.28 |
| 29.11 | 64.43 | N43E10 | 68304 | 12.5 | 8.13 | 4.37 |
| 29.77 | 64.46 | N43E10 | 68311 | 12.5 | 8.01 | 4.49 |
| 29.82 | 64.27 | N43E10 | 68313 | 9.81 | 8.86 | 0.95 |
| 29.42 | 64.45 | N43E10 | 68314 | 10.6 | 8.08 | 2.52 |
| 29.39 | 64.28 | N43E10 | 68315 | 22.1 | 7.51 | 14.59 |
| 28.66 | 64.26 | N43E10 | 68316 | 10.1 | 7.87 | 2.23 |
| 28.70 | 64.45 | N43E10 | 68317 | 10.4 | 8.29 | 2.11 |
| 27.57 | 63.04 | N43E10 | 68361 | 13.1 | 12.85 | 0.25 |
| 27.84 | 63.04 | N43E10 | 68362 | 16.8 | 14.54 | 2.26 |
| 28.16 | 63.03 | N43E10 | 68363 | 13.8 | 16.60 | -2.80 |
| 29.71 | 63.02 | N43E10 | 68367 | 18.2 | 15.37 | 2.83 |
| 30.15 | 63.03 | N43E10 | 68373 | 8.39 | 12.80 | -4.41 |
| 29.81 | 63.36 | N43E10 | 68385 | 8.42 | 13.03 | -4.61 |
| 29.14 | 63.34 | N43E10 | 68386 | 14.9 | 11.89 | 3.01 |
| 29.46 | 63.02 | N43E10 | 68387 | 8.43 | 17.77 | -9.34 |
| 29.41 | 63.30 | N43E10 | 68389 | 9.63 | 12.97 | -3.34 |
| 29.51 | 63.18 | N43E10 | 68390 | 14.6 | 13.97 | 0.63 |
| 29.77 | 63.17 | N43E10 | 68391 | 16.1 | 14.03 | 2.07 |
| 28.72 | 63.32 | N43E10 | 68392 | 6.82 | 11.59 | -4.77 |
| 28.25 | 63.34 | N43E10 | 68393 | 12.4 | 13.02 | -0.62 |
| 28.15 | 63.18 | N43E10 | 68394 | 24.8 | 15.77 | 9.03 |
| 27.91 | 63.19 | N43E10 | 68395 | 11 | 14.52 | -3.52 |
| 27.92 | 63.36 | N43E10 | 68396 | 15.2 | 12.08 | 3.12 |
| 27.17 | 63.23 | N43E10 | 68398 | 9.96 | 9.68 | 0.28 |
| 27.45 | 63.22 | N43E10 | 68399 | 12.8 | 11.53 | 1.27 |
| 27.26 | 63.04 | N43E10 | 68401 | 13 | 10.88 | 2.12 |
| 27.45 | 63.38 | N43E10 | 68421 | 12.5 | 10.80 | 1.70 |

### A2.1.1.3. Comments

The Ni concentrations of the Finnish FOREGS GTN reference data set are very low and, thus, negative values (dz) result when the levelled Ni (Z) concentrations are subtracted from the original values (Tables A2.1.2 & A2.1.3).

## A2.1.2. Recommendations

During the “Create TIN” procedure the algorithm connects adjacent points to formulate the triangles, using the Delaunay triangulation method. Which adjacent points are connected, which results, and which Delaunay triangles are created, depends on the number of data points and their spatial distribution. In our example from Finland, we have used all the available data points. The triangles might be different if we could use points that are evenly distributed around the GTN grid cells N43E09 and N43E10, or a subset of them. Thus, the normalised values of the observations depend on how the Delaunay triangles are formed according to the spatial distribution of the 5 random reference sample GTN points. Therefore, it is necessary to study the sensitivity of the levelled results with respect to the changes in the definition of the Delaunay triangles. This recommendation is explained in Figure A2.1.4. It is obvious that different outside reference Dalaunay triangles are created and, accordingly, different levelling values are possible. In order to address this issue, the following recommendations are made:

- To establish a permanent reference Delaunay triangle network to be utilised in all cases by all users, or if this is not possible

- To select reference points in the perimeter that are not at a long distance from the GTN grid cells where the regional geochemical survey values are to be levelled with the aim for the calculations not to be affected by distant random GTN reference values.

Finally, it is strongly recommended that in order to generate a better reference “Tin” plane is to select regional geochemical samples very near to the five random reference GTN points.

Figure A2.1.4. Delaunay triangles procedure for levelling existing geochemical results with respect to the values of a reference data set. The black and red lines show the Delaunay triangles formed when all 5 random points in the Finnish and adjoining Swedish GTN grid cells are used. The superimposed red lines show the selection of the 5 random points within GTN grid cells N43E09 and N43E10 and a few points in the perimeter. See also Figure A2.1.3.

## Supplementary material

The following supplementary material is provided in the zip file ‘Annexe_A2.1_Supplementary_material.zip’, which can be downloaded from the Publications web page of the IUGS Commission on Global Geochemical Baselines:

- MicrosoftTM Excel file: ‘Finland_Subsoil_AR_data_GTN_N43E09_&_N43E10_Nickel.xlsx’ with the Nickel aqua regia subsoil values of the 5 reference random sites within the GTN grid cells N43E09 and N43E10 (extracted from the 'C_AR_data_2v5_8Feb06.xlsx’ (see below).

- MicrosoftTM Excel file: ‘Finland_GTN_N43E09_&_N43E10_Nickel.xlsx’ with the regional geochemistry Nickel aqua regia subsoil (C horizon till) results (from Tarvainen, 1995, 1996).

- MicrosoftTM Excel file: 'C_AR_data_2v5_8Feb06.xlsx' with the values of Nickel for the 5 random points of the GTN grid cells in Europe (FOREGS GTN data set from Salminen et al., 2005).

| # | File name |  | Content |
| --- | --- | --- | --- |
|  | FinlandExample.mxd | ESRI ArcMap 10.3.1 map document which contains all the original data and results. 3D Analyst is activated. All data references are relative to this document. |  |
|  | Finland.mdb | ESRI, ver. 10.3.1, Personal Geodatabase (PGDB). Contains: |  |
| - fingtnall |  |  | Feature class with all the regional geochemical samples collected within GTN Grids N43E09 & N43E10. Fields Z and dz have been added during steps (e) and (f) of the process (see ‘3. Process’ in Section §A2.1.1.2). |
| - arsub_tt |  |  | Feature class of all the GTN 5 random points. It contains columns of AS_AR_C_n and NI_AR_C_n values. The feature class has been created from Subsoil_1v0_21June2006\C_AR_data_2v5_8Feb06.xls using the Display X, Y data (ARSUB_TT$Events) and save the results to the PGDB. |
| - Tinsedges |  |  | The edges of TIN data set created during step (c) of the procedure (see ‘3. Procedure’ in Section §A2.1.1.2). |
| 3. | tinarsub_tt | Tin Dataset created using arsub_tt. |  |

## References

Note: All hyperlinks were checked on the 22nd of February 2022.

Darnley, A.G., Björklund, A., Bølviken, B., Gustavsson, N., Koval, P.V., Plant, J.A., Steenfelt, A., Tauchid, M., Xie, X., Garrett, R.G. & Hall, G.E.M., 1995. A Global Geochemical Database for Environmental and Resource Management. Recommendations for International Geochemical Mapping – Final Report of IGCP Project 259. Earth Science Report 19. UNESCO Publishing, Paris, 122 pp; http://globalgeochemicalbaselines.eu.176-31-41-129.hs-servers.gr/datafiles/file/Blue_Book_GGD_IGCP259.pdf.

Okabe, A., Boots, B., Sugihara, K. & Chiu, S.N., 2000. Spatial Tessellations: Concepts and Applications of Voronoi Diagrams. Wiley, 696 pp.

Salminen, R., Batista, M.J., Bidovec, M., Demetriades, A., De Vivo, B., De Vos, W., Duris, M., Gilucis, A., Gregorauskiene, V., Halamic, J., Heitzmann, P., Lima, A., Jordan, G., Klaver, G., Klein, P., Lis, J., Locutura, J., Marsina, K., Mazreku, A., O’Connor, P.J., Olsson, S.Å., Ottesen, R.T., Petersell, V., Plant, J.A., Reeder, S., Salpeteur, I., Sandström, H., Siewers, U., Steenfelt, A. & Tarvainen, T., 2005. FOREGS Geochemical Atlas of Europe, Part 1: Background Information, Methodology and Maps. Geological Survey of Finland, Espoo, 525 pp.; http://weppi.gtk.fi/publ/foregsatlas/.

Tarvainen, T., 1995. The geochemical correlation between coarse and fine fractions of till in southern Finland. Journal of Geochemical Exploration, 54(3), 187−198; https://doi.org/10.1016/0375-6742(95)00041-0.

Tarvainen, T., 1996. Environmental applications of geochemical databases in Finland: synopsis. Geological Survey of Finland, Espoo, 75 pp.; https://tupa.gtk.fi/julkaisu/erikoisjulkaisu/ej_019.pdf.

Blank back page

ESRI ArcGIS Pro: TIN surface https://pro.arcgis.com/en/pro-app/2.7/help/data/tin/create-a-tin-surface.htm; ESRI Desktop 10 https://help.arcgis.com/en/arcgisdesktop/10.0/help/index.html#//00q900000016000000