International Union of Geological Sciences

Manual of Standard Methods

for

Establishing the Global Geochemical Reference Network

Chapter 2

Global Terrestrial Network Grid Cells,

Selection of Sample Sites, and

Sample Types to be Collected

Alecos Demetriades1,5, Christopher C. Johnson2,5, David B. Smith5,

Timo Tarvainen3,5, Maria João Batista4,5

1 Institute of Geology and Mineral Exploration, Athens, Hellenic Republic

2  GeoElementary, Derby, United Kingdom

3 Geological Survey of Finland, Espoo, Finland

4  Laboratório Nacional de Energia e Geologia, Amadora, Portugal

5 IUGS Commission on Global Geochemical Baselines

2022

Published by

The International Union of Geological Sciences

Commission on Global Geochemical Baselines

It is recommended that reference to this part of the Manual should be made in the following way:

Demetriades, A., Johnson, C.C., Smith, D.B. & Batista, M.J., 2022. Global Terrestrial Network Grid Cells, Selection of Sample Sites and Sample Types to be Collected. Chapter 2 In: Demetriades, A., Johnson, C.C., Smith, D.B., Ladenberger, A., Adánez Sanjuan, P., Argyraki, A., Stouraiti, C., Caritat, P. de, Knights, K.V., Prieto Rincón, G. & Simubali, G.N. (Editors), International Union of Geological Sciences Manual of Standard Methods for Establishing the Global Geochemical Reference Network. IUGS Commission on Global Geochemical Baselines, Athens, Hellenic Republic, Special Publication, 2, 11−26.

Published by

The International Union of Geological Sciences

Commission on Global Geochemical Baselines

P.O. Box 640 37, Zografou

GR-157 10 Athens

Hellenic Republic

http://www.iugs.org/

http://www.globalgeochemicalbaselines.eu/

National Library of Greece Cataloguing in Publication Data

A catalogue record for this electronic book is available from the

National Library of Greece

ISBN: 978-618-85049-1-2

Contents

2.1. Introduction	15

2.1.1. Levelling of existing data sets	16

2.2. Global terrestrial network grid cells	16

2.2.1. Identifiers of GTN grid cells	17

2.2.2. GTN grid cell coordination	18

2.3. Sample types to be collected	19

2.3.1. Sample identifiers	19

2.4. Random sampling design	20

2.5. Selection of catchment basins for sampling	21

2.5.1. Selection of small drainage basins in each GTN grid cell	21

2.5.2. Selection of large drainage basins in each GTN grid cell	22

2.6. Example GTN grid cell N26E14	23

2.7. Differences from IGCP 259 recommendations	24

2.8. Differences from FOREGS project	24

Supplementary material	24

References	25

Blank back page

## 2.1. Introduction

Production of coherent and quantitative geochemical maps to portray the chemical composition of the world’s land surface is necessary, and requires the establishment of a suitable multi-media sample reference framework. One may ask the question: How can such a geochemical reference framework of multi-media samples be developed?

To answer this question, let us go back in time, long before the advent of global navigation satellite systems in the 1980s, and consider an analogy with topographical map making. At that time producing accurate topographical maps required the establishment of a 3-dimensional  ground geodetic network. The core of this geodetic network was the ‘triangulation point or trig point’, which was a reference point on the land surface with precise longitude and latitude coordinates and elevation above the datum. The corners of the trig point network were usually located on hilltops, each visible from at least two others. The angles between the lines joining them were measured precisely, and this process was called triangulation (Higgins, 1974; Musseter, 1985; Britannica, 2012, 2016; Simmons, 2014). Mapping the morphology of the land surface, surveyors used triangulation surveying by generating a denser set of intermediate spot levels, depending on the mapping scale. Thus, surveying may be described as the art of making measurements upon the earth’s surface to produce a topographical map. Levelling is combined with surveying when it is required to determine the differences in elevation of points on the earth’s surface to portray the variation of the height of the topography surveyed with either contour lines, or in vertical sections (Strahler, 1969; Higgins, 1974). If this 3-dimensional Geodetic Reference Network on a topographical map is studied, it can be observed that the trig points are randomly distributed.

Similar to topographical mapping, a Global Geochemical Reference Network representing element concentrations generated on a set of multi-media samples collected from randomly selected sites fulfils a comparable function for the geochemical mapping of the Earth’s terrestrial surface (Darnley, 1997). Each Global Geochemical Reference Network sample point has coordinates as the trig point in surveying, and the element concentration of each reference sample type is analogous to the elevation or height of the trig point. As with a geodetic network, a geochemical reference network is not concerned with local detail, but the latter, as it is acquired, should be tied into the fixed points of the network (Darnley et al., 1995). Hence, what is required is the establishment of ‘fixed point element concentrations’ across the earth’s land surface, generated with a harmonised quality-controlled methodology from sampling and sample preparation to chemical analysis. The methodology for the many steps required to establish the global geochemical reference network is the subject of this Manual. With the establishment of such a multi-media 3-dimensional Global Geochemical Reference Network of ‘fixed point element concentrations’ (the geochemical datum), it will be possible to level national geochemical data sets of each sample type, and produce detailed maps, as the surveyors do in the production of topographical maps.

In conclusion, just as the foundation for detailed topographical surveys is provided by a primary 3-dimensional geodetic network, a comparable 3-dimensional reference network of each sample type is required for geochemical surveys. Darnley et al. (1995) proposed that the highest order grid cell in the global geochemical mapping hierarchy should be 160x160 km, an area of 25,600 km2. A collection of standard reference materials of each sample type is required from these grid cells over the entire land surface of the world as the first step in the technical implementation of the global geochemical project. This Global Geochemical Reference Network (GGRN), based on carefully controlled sampling and geoanalytical requirements, as detailed in this Manual, will provide an inter-regional (and intercontinental) framework to which more detailed or more specialised local geochemical surveys can be related.

### 2.1.1. Levelling of existing data sets

Darnley et al. (1995, p.75) discuss the different methods for levelling and normalisation of existing data sets, namely (a) parametric levelling, and (b) non-parametric normalisation (fractile normalisation, Clarke normalisation, quantile regression), and pointed out that such procedures are ‘geochemically blind’ because they simply manipulate numbers. They also state the conditions for the selection of data sets for compilation. For example, data sets for levelling should be comparable, e.g., stream sediment GGRN data with regional stream sediment, and soil GGRN data with regional and detailed soil survey data sets. Further, it is stressed that it would likely be inappropriate to attempt to level data sets of cold partial extraction and total element concentrations, and similarly stream sediment results with heavy mineral concentrate data (see Annexes A2.1 and A2.2 for worked levelling examples).

## 2.2. Global terrestrial network grid cells

To establish the Global Geochemical Reference Network, Darnley et al. (1995) proposed that a variety of sample media should be collected according to grid-based sampling over the entire land surface of the Earth. The computation of the Global Reference Network (GRN) grid cells, covering the whole globe, was carried out in 1994 by Nils Gustavsson (Geological Survey of Finland). In total, there are 19,833 grid cells of 160x160 km (Fig. 2.1). The original Microsoft™ Access file was provided to the IUGS Commission on Global Geochemical Baselines in August 2018 by Robert G. Garrett (Geological Survey of Canada). Both, the generated Microsoft™ Excel file with all 19,833 grid cells, and the Google Earth *.kml file, are available for downloading from the website of the IUGS Commission on Global Geochemical Baselines (refer to the Supplementary material for more details).

Figure 2.1. The Global Reference Network (GRN) grid cells consist of the Global Terrestrial Network (GTN) and Global Marine Network (GMN) grid cells. The nominal size of each 160x160 km grid cell is defined by two parallels of latitude 1⅟2o (approx. 166 km) apart, and two meridians. In order to retain a constant area, the grid cells are systematically displaced in longitude East and West in successive latitudinal bands. Drawn with Golden Software’s Surfer™ v21 by Alecos Demetriades, Hellenic Institute of Geology and Mineral Exploration (IGME) & IUGS Commission on Global Geochemical Baselines (IUGS-CGGB).

The 160x160 km grid cells (for the entire world) have their origin on the equator at the 0o (Greenwich) meridian and they are symmetrical North and South of the equator. For practical convenience, the cells are bounded by lines of latitude 1⅟2o apart (approximately 166 km). To retain a constant area of 25,600 km2 the meridians defining East and West boundaries are systematically displaced in longitude East and West in successive latitudinal bands. Overlap in the vicinity of the international dateline is ignored because it occurs over the Pacific Ocean.

The original Global Terrestrial Network (GTN) grid cells file, covering the land surface of the Earth, used in 1997 for planning the FOREGS Geochemical Atlas of Europe project, comprised 5711 grid cells (Salminen, Tarvainen et al., 1998). Upon plotting the grid cells on a world map, it was discovered that many grid cells near coastal areas and over islands were missing. The reason for the missing grid cells was that their centre fell over a water body (ocean, sea, lake), and were thus deleted because each 160x160 km grid cell is required to have within it a minimum of three random sites for sampling. This is an important requisite of triangulation and levelling the data sets of regional and detailed geochemical surveys to their respective Global Geochemical Reference Network data sets (see Section §2.1 above). Of course, this was discovered during the FOREGS Geochemical Atlas of Europe project and was corrected for countries that started their sampling campaign a little later than the June 1997 kick-off. The reason for making this correction is that geochemical data are required from all grid cells for the compilation of global geochemical baseline maps.

For planning the Global Geochemical Reference Network project for the entire terrestrial surface of the Earth, a new file was generated by adding all the missing coastal grid cells and over islands. This new Microsoft™ Excel file, covering the terrestrial surface of the Earth, consists of 7356 grid cells of 160x160 km (Fig. 2.2), and is available, together with the Google Earth *.kml  file, from the website of the IUGS Commission on Global Geochemical Baselines.

Figure 2.2. The 7356 Global Terrestrial Network (GTN) grid cells cover the entire land surface of the Earth, including islands. The nominal size of each 160x160 km grid cell is defined by two parallels of latitude 1⅟2o (approx. 166 km) apart, and two meridians. For keeping a constant area, the grid cells are systematically displaced in longitude East and West in successive latitudinal bands. Drawn by Alecos Demetriades (IGME/IUGS-CGGB) with Golden Software’s MapViewer™ v8.

### 2.2.1. Identifiers of GTN grid cells

The identifiers of GTN 160x160 km grid cells consist of a four-part label:

- The latitude letter code ‘N’ and ‘S’, representing North and South from the equator, respectively.

- The longitude letter code ‘E’ and ‘W’, representing East and West from the 0o Greenwich meridian, respectively.

- A two-digit number after the latitude letter code (N or S), representing the position of the grid cell North or South from the equator, and

- A two-digit number after the longitude letter code (E or W), representing the position of the grid cell East or West from the 0o Greenwich meridian.

For example, the identifier N34E03 denotes that this 160x160 km GTN grid cell is the 34th cell to the North of the equator, and the 3rd cell to the East of the 0o Greenwich meridian, and covers parts of Belgium, France, Luxemburg and Germany (Fig. 2.3).

### 2.2.2. GTN grid cell coordination

The coordination of sampling within the 160x160 km grid cells falling completely within a country is coordinated by its national organisation. Some grid cells are located in more than one country. In these cases, sampling of that particular cell is coordinated by the national organisation of the country in which the centre of the grid cell is located. For example, grid cell N32E04 falls in Switzerland, France and Germany, and its centre is in Switzerland (Fig. 2.3).

Figure 2.3. Map showing the FOREGS Geochemical Atlas of Europe area and adjacent countries. The coded numbers indicate the North-South and East-West GTN grid cell numbering system used in the Global Geochemical Reference Network project. The red colour lined GTN grid cell N34E03 covers Belgium, Luxembourg (Lx), France and Germany, and the green colour lined GTN grid cell N32E04 covers Switzerland (Sw), France and Germany. In the former case, Belgium is the coordinating country, and in the latter case is Switzerland. Refer to the text for an explanation. Drawn by Alecos Demetriades (IGME/IUGS-CGGB) with Golden Software’s MapViewer™ v8.

Therefore, the sampling in this particular grid cell is coordinated by Switzerland, and the Swiss organisation, responsible for its sampling, should get in touch with the French and German organisations for the sampling of catchment basins falling in their countries.

It is noted that the coordinating countries for the sampling of each 160x160 km grid cell are indicated in the relevant Microsoft™ Excel file, which can be downloaded from the website of the IUGS Commission on Global Geochemical Baselines (refer to the Supplementary material). Hence, the first task of each national organisation is to find out all grid cells that are coordinated by its country, and are its responsibility for their sampling. It is recommended to plot the 160x160 km grid cells on 1:50,000 scale topographical maps, and on a smaller scale topographical map, depending on the size of the country. The latter map provides an overview, and is used for planning purposes.

Important note: A condition that must be observed in the Global Geochemical Reference Network project is that the participating national organisations must not carry out any sampling on the territory of any other country without a previous written agreement with the organisation responsible for sampling in that particular territory. If one or more of the randomly selected catchments are located in a neighbouring country, it is important to contact the organisation responsible for the coordination, and to reach an agreement on their sampling.

## 2.3. Sample types to be collected

All sample types, mentioned below, are mandatory and must be collected from each randomly selected second- and third-order catchment basin, as this is the only way to establish reference sample suites of permanent value with the objective for their results to be used for (a) levelling to a common geochemical datum the results of more detailed geochemical mapping projects, and (b) compilation of a multi-media Global Geochemical Baselines atlas.

The following sample types must be collected from second-order catchment basins of an area of <100 km2:

- Rock

- Residual soil (Top and Bottom)

- Humus (where present)

- Stream water (where present)

- Stream sediment, and

- Overbank sediment (Top and Bottom).

The following sample type must be collected from third-order catchment basins of an area between 1000 and 6000 km2, except for the cases mentioned above:

- Floodplain sediment (Top and Bottom).

### 2.3.1. Sample identifiers

The codes tabulated in Table 2.1 characterise the sample types to be collected from the second- and third-order catchment basins, according to the Strahler (1957, 1969) stream magnitude order classification, and a topographical map scale of 1:50,000.

The following identifier N26E14S3 clarifies the sample identification system where:

| N26E14 | = GTN grid cell identification code |
| --- | --- |
| S | = Sample medium code (in this case is Stream sediment) |
| 3 | = Random catchment (drainage) basin number |

This grid cell is explained in greater detail and presented graphically in Section §2.6.

The field duplicate sample code for all sample types is ‘D’. In this case, for the stream sediment sample it would be N26E14S3D.

The identifier for the blank water (W) and blank solid (T) samples is zero (0), i.e.,

- GTN grid cell/random point number/W/0 (zero), e.g., N26E14W30, and

- GTN grid cell/random point number/T/0 (zero), e.g., and N26E14T30.

Table 2.1. Sample identifier codes, and example using GTN grid cell N26E14 and random site number 3 (see Section §2.6 where there is a detailed explanation).

| Catchment basin | Sample type | Code | Example |
| --- | --- | --- | --- |
| Second-order (<100 km2) | Rock | R | N26E14R3 |
|  | Residual soil – Top | T | N26E14T3 |
|  | Residual soil – Bottom | C | N26E14C3 |
|  | Humus (where present) | H | N26E14H3 |
|  | Stream water (where present) | W | N26E14W3 |
|  | Stream sediment (mineral sediment) | S | N26E14S3 |
|  | Overbank sediment – Top | K | N26E14K3 |
|  | Overbank sediment – Bottom | N | N26E14N3 |
| Third-order (1000-6000 km2) | Floodplain sediment – Top | F | N26E14F3 |
|  | Floodplain sediment – Bottom | L | N26E14L3 |

## 2.4. Random sampling design

The catchment basins for sampling are represented by points in each Global Terrestrial Network (GTN) grid cell of 160x160 km, and are totally randomised. Darnley et al. (1995) recommended either a 5- or an 8-random point sampling scheme for the Global Geochemical Reference Network project (Fig. 2.4), and each country is free to select the scheme it will follow. Of course, the latter scheme, with an additional 3 random points, is better for both levelling the results of more detailed geochemical surveys, and for producing global geochemical baseline maps. However, the 5-random point scheme is more cost- and time-effective, and it has been used successfully for the FOREGS Geochemical Atlas of Europe project (Salminen, Tarvainen et al., 1998; Salminen et al., 2005; De Vos, Tarvainen et al., 2006).

| (a) | (b) |
| --- | --- |

Figure 2.4. Random sampling schemes in each 160x160 km GTN grid cell: (a) 5-random point sampling scheme, and (b) 8-random point sampling scheme. Diagrams plotted by Alecos Demetriades (IGME/IUGS-CGGB) with Microsoft™ PowerPoint.

The 5- and 8-random point sampling schemes were generated by R-scripts written and run by He and Geng (2022). The Microsoft™ Excel and Google Earth *.kml files of both schemes are available for downloading (refer to the Supplementary material). With respect to the 5-random sampling site scheme, point number 1 is located in the NE quadrant of the GTN grid cell, number 2 in the NW quadrant, number 3 in the SW quadrant and number 4 in the SE quadrant (Fig. 2.4a). Point number 5 is randomly located within the 160x160 km grid cell. If a national coordinator is not satisfied with the available random points for his/her country, the R-script, accompanied by instructions, is available and can be run for the generation of a new set of random points for planning the sampling campaign. The R-script for generating new random points can be run either for the GTN grid cells covering the whole country or for just a particular GTN grid cell.

## 2.5. Selection of catchment basins for sampling

The national set of 5-random points in each GTN 160x160 km grid cell should be plotted on 1:50,000 scale topographical maps for planning the sampling campaign. If the 1:50,000 scale topographical maps are not available in digital form, then the 5-random points should be plotted by hand by using the coordinates of each point.

### 2.5.1. Selection of small drainage basins in each GTN grid cell

The 5-random points in each 160x160 km GTN grid cell are used to select the five nearest small second-order drainage basins of <100 km2 in area, according to the Strahler stream magnitude order classification system (Strahler, 1957, 1969). This is the smallest unit where pristine samples could be collected. The criteria that should be used for the selection of the small drainage basin for the collection of rock, residual soil (top and bottom), stream water, stream sediment and overbank sediment (top and bottom) (see Figs. 2.5, 2.6, 2.7 & 2.8) are:

- Rock samples should represent the dominant rock type, and

- Residual soil should represent the dominant soil type within the small catchment basin.

Figure 2.5. Block diagram showing possible sites of GTN sampling media in second- and third-order catchment basins at a mapping scale of 1:50,000. Notation: Numbers 1, 2 and 3 indicate catchment basin order according to the Strahler stream classification magnitude order (after Strahler, 1969, Fig. 27.1E, p.499, with minor modifications). Drawn by Alecos Demetriades (IGME/IUGS-CGGB) with Golden Software’s MapViewer™ v8.

It is, therefore, recommended that geological and soil maps, if available, should be studied during the planning stage of the sampling campaign.

In case the random point happens to fall in the sea or a lake or an unreachable area, the nearest accessible small catchment basin should be used (see Fig. 2.8).

### 2.5.2. Selection of large drainage basins in each GTN grid cell

The large drainage basin with an area of 1000 to 6000 km2 is easy to find because it is physically connected to the selected small catchment basin of <100 km2 in area (see Figs. 2.5 & 2.8).

If no suitable size drainage basin is available, the top and bottom floodplain sediment samples can be taken from a smaller third-order drainage basin (>500 to 1000 km2).

Justified exceptions to these specifications are the collection of samples from small countries and small islands, as in the case shown in Figure 2.8. What is important in such exceptional cases is the collection of floodplain sediment samples from third-order catchment basins at a map scale of 1:50,000.

Figure 2.6. Block diagram showing possible sites of GTN sampling media in a second-order catchment basin at a mapping scale of 1:50,000 (after Strahler, 1969, Fig. 22.3, p.363, with minor modifications). Drawn by Alecos Demetriades (IGME/IUGS-CGGB) with Golden Software’s MapViewer™ v8.

Figure 2.7. Cross-section of second-order stream valley showing different features and the sample sites for collecting rock, residual soil, stream water, stream and overbank sediment samples. Residual soil samples must not be collected from colluvium, as this is transported overburden. Drawn by Alecos Demetriades (IGME/IUGS-CGGB) with Golden Software’s Grapher™ v20.

## 2.6. Example GTN grid cell N26E14

The samples to be collected from the second- and third-order catchment basins according to the Strahler (1957, 1969) stream magnitude order classification are indicated in Figure 2.8. The plan view shows random point number 3 second- and third-order catchment basins of GTN grid cell N26E14, and the possible sites for the collection of all specified sample types. This GTN grid cell was selected to show the problems of the random point generation algorithm, which generates random points indiscriminately whether they fall on land or water. During the planning stage of the sampling campaign, the random points are plotted on 1:50,000 topographical maps, and any problems such as random point 4 falling in the sea is moved to the nearest land. Possible sites for sampling are:

- On the island of Kea sample site 4a with an area of 132 km2 where a second-order catchment basin with an area of <100 km2 can be found.

- The second option is site 4b on the southern part of Euboea island, and

- The third option is site 4c on the island of Andros with an area of 380 km2.

In all three cases, there are no third-order catchment basins with an area of 1000-6000 km2. It is noted that there is no suitable catchment basin on mainland Hellas, and even if there was this will extend into the south-west quadrant of the N26E14 grid cell, which is covered by site number 3. Hence, a decision should be taken from which island the samples will be collected, with the knowledge that the large third-order catchment basin does not meet the specifications. However, in these special cases, it is more important to collect samples from each quadrant of the 160x160 km grid cell.

Figure 2.8. Example of a GTN grid cell N26E14 with 5 random points, and a schematic diagram showing the possible sample sites of rock, residual soil, humus, stream water and sediment, overbank sediment, and floodplain sediment from the catchment basin representing random point number 3. Two random points fall on mainland Hellas (3 & 5), two on Euboea Island (1 & 2), and one in the sea (4), which should be moved to a land site. See text for explanation. Drawn by Alecos Demetriades (IGME/IUGS-CGGB) with Golden Software’s MapViewer™ v8.

## 2.7. Differences from IGCP 259 recommendations

The differences between the IGCP 259 recommendations of Darnley et al. (1995), and this Manual of Standard Methods for Establishing the Global Geochemical Reference Network are:

- All sample types are mandatory because the purpose of the Global Geochemical Reference Network project is to provide sufficient reference data points to level geochemical mapping results at all scales.

- An important sample type that is now included is rock.

- The shorter sample identifiers of the FOREGS Geochemical Atlas of Europe project (Salminen, Tarvainen et al., 1998) are used.

- Stream sediment is the only sample type that is collected as a composite from about 10 sub-sites along a stream stretch of 500 metres.

- Samples of residual soil are horizon-based, i.e., A and C horizon samples. Similarly, overbank and floodplain sediments are collected from single top and bottom layers. They are all collected from a SINGLE pit or vertical section.

- No compositing of samples within each GTN grid cell for reducing analytical costs and, in fact, destroying the integrity and characteristics of individual samples, and the establishment of a primary Global Geochemical Reference Network similar to a geodetic grid (Darnley et al., 1995; Darnley, 1997). This procedure is unacceptable and MUST NOT be used (see Demetriades et al., 2018, p.167−170).

- The grain size fraction of residual soil and overbank sediment samples for analysis is <2 mm, and NOT the fine-grained fraction of <0.150 mm.

## 2.8. Differences from FOREGS project

The FOREGS Geochemical Atlas of Europe project was the first multinational and multi-media sampling project to follow closely the IGCP 259 recommendations of Darnley et al. (1995). The differences between the FOREGS Geochemical Atlas of Europe project (Salminen, Tarvainen et al., 1998), and this Manual of Standard Geochemical Methods are:

- All sample types are mandatory.

- Rock is included as a mandatory sample medium.

- Samples of residual soil are horizon-based, i.e., A and C horizon samples. Similarly, overbank and floodplain sediments are collected from a single top and a single bottom layer. They are all taken from a SINGLE pit or vertical section.

- The analysed grain size fraction for overbank sediment samples is <2 mm, and NOT the fine-grained fraction of <0.150 mm.

## Supplementary material

- The files described below can be downloaded from the ‘Sampling/GTN 160x160 km’ web page of the IUGS Commission on Global Geochemical Baselines.

- Global Reference Network grid cells of 160x160 km files:

- IUGS-CGGB_19833_GRN_160x160km_grid_cells.xlsx (Microsoft™ Excel workbook file with the 19,833 grid cells of the Global Reference Network each of 160x160 km).

- IUGS-CGGB_GRN_19833_160x160km_grid_cells.zip (same as above but in a Google Earth kml file format). When unzipped the file name is: IUGS-CGGB_19833_GRN_160x160km_grid_cells.kml.

- Global Terrestrial Network grid cells of 160x160 km files:

- IUGS-CGGB_7356_GTN_160x160km_grid_cells.xlsx (Microsoft™ Excel workbook file with the 7356 grid cells of the Global Terrestrial Network each of 160x160 km).

- IUGS-CGGB_7356_GTN_grid_cells_160x160km.zip (same as above but in a Google Earth kml file format). When unzipped the file name is: IUGS-CGGB_7356_GTN_grid_cells_160x160km.kml.

- The following six files can be downloaded from the ‘Sampling/Sampling Design’ web page of the IUGS Commission on Global Geochemical Baselines.

- Randomly selected points in each Global Terrestrial Network grid cell:

- IUGS-CGGB_5-random-points_per_GTN_160x160km-grid-cell.xlsx (Microsoft™ Excel workbook file with 5 random points in each of the 7356 grid cells of 160x160 km totalling 36,780 random sampling points).

- IUGS-CGGB_5-random-points_per_GTN_160x160km-grid-cell.zip (same as above but in a Google Earth kml file format). When unzipped the file name is: IUGS-CGGB_5-random-points_per_GTN_160x160km-grid-cell.kml.

- IUGS-CGGB_8-random-points_per_GTN_160x160km-grid-cell.xlsx (Microsoft™ Excel workbook file with 8 random points in each of the 7356 grid cells of 160x160 km totalling 58,848 random sampling points).

- IUGS-CGGB_8-random-points_per_GTN_160x160km-grid-cell.zip (same as above but in a Google Earth kml file format). When unzipped the file name is: IUGS-CGGB_8-random-points_per_GTN_160x160km-grid-cell.kml.

- IUGS-CGGB_16-random-points_per_GTN_160x160km-grid-cell.xlsx (Microsoft™ Excel workbook file with 16 random points in each of the 7356 grid cells of 160x160 km totalling 117,696 random sampling points). It is noted that this sampling density is not recommended for the Global Geochemical Reference Network.

- IUGS-CGGB_16-random-points_per_GTN_160x160km-grid-cell.zip (same as above but in a Google Earth kml file format). When unzipped the file name is: IUGS-CGGB_16-random-points_per_GTN_160x160km-grid-cell.kml.

## References

Note: All hyperlinks were checked on the 15th of December 2021.

Britannica, The Editors of Encyclopӕdia, 2012. Modern surveying. Wright, J.W. (article Editor), Ultimate Reference Suite DVD. Encyclopӕdia Britannica, Inc., United Kingdom.

Britannica, The Editors of Encyclopӕdia, 2016. Modern surveying. Encyclopӕdia Britannica, Inc., 6 Jun. 2016; https://www.britannica.com/technology/surveying/Modern-surveying.

Darnley, A.G., 1997. A global geochemical reference network: the foundation for geochemical baselines. Journal of Geochemical Exploration, 60(1),1−5; https://doi.org/10.1016/S0375-6742(97)00020-4.

Darnley, A.G., Björklund, A., Bølviken, B., Gustavsson, N., Koval, P.V., Plant, J.A., Steenfelt, A., Tauchid, M., Xie., X., Garrett, R.G. & Hall, G.E.M., 1995. A Global Geochemical Database for Environmental and Resource Management. Recommendations for International Geochemical Mapping – Final Report of IGCP Project 259. Earth Science Report 19. UNESCO Publishing, Paris, 122 pp; http://globalgeochemicalbaselines.eu.176-31-41-129.hs-servers.gr/datafiles/file/Blue_Book_GGD_IGCP259.pdf.

De Vos, W., Tarvainen, T., Salminen, R., Reeder, S., De Vivo, B., Demetriades, A., Pirc, S., Batista, M.J., Marsina, K., Ottesen, R.T., O’Connor, P.J., Bidovec, M., Lima, A., Siewers, U., Smith, B., Taylor, H., Shaw, R., Salpeteur, I., Gregorauskiene, V., Halamic, J., Slaninka, I., Lax, K., Gravesen, P., Birke, M., Breward, N., Ander, E.L., Jordan, G., Duris, M., Klein, P., Locutura, J., Bel-lan, A., Pasieczna, A., Lis, J., Mazreku, A., Gilucis, A., Heitzmann, P., Klaver, G. & Petersell, V., 2006. Geochemical Atlas of Europe. Part 2 – Interpretation of Geochemical Maps, Additional Tables, Figures, Maps, and Related Publications. Geological Survey of Finland, Espoo, 690 pp.; http://weppi.gtk.fi/publ/foregsatlas/.

He, J. & Geng, X., 2022. R-scripts for generation of 5, 8 and 16 random sampling points within predefined rectangles. International Union of Geological Sciences, Commission on Global Geochemical Baselines, Special Publication No. 3, Athens, Hellenic Republic, 20 pp.; available from the Publications web page of the IUGS Commission on Global Geochemical Baselines: http://www.globalgeochemicalbaselines.eu/content/91/publications-/.

Higgins, A.L., 1974. Elementary surveying. Longman Group Ltd., London, 197 pp.

Musseter, W., 1985. Manual of Reconnaissance for Triangulation. Special Publication 225, U.S. Department of Commerce, National Oceanic and Atmospheric Administration, National Ocean Survey, 100 pp.; https://www.ngs.noaa.gov/PUBS_LIB/ManualOfReconnaissanceForTriangulation_NOS_Special_Pub_225.pdf.

Salminen, R., Tarvainen, T., Demetriades, A., Duris, M., Fordyce, F.M., Gregorauskiene, V., Kahelin, H., Kivisilla, J., Klaver, G., Klein, P., Larson, J.O., Lis, J., Locutura, J., Marsina, K., Mjartanova, H., Mouvet, C., O’Connor, P., Odor, L., Ottonello, G., Paukola, T., Plant, J.A., Reimann, C., Schermann, O., Siewers, U., Steenfelt, A., Van Der Sluys, J. & Williams, L., 1998. FOREGS Geochemical Mapping Field Manual. Geological Survey of Finland, Espoo, Guide 47, 36 pp.; http://tupa.gtk.fi/julkaisu/opas/op_047.pdf.

Salminen, R., Batista, M.J., Bidovec, M., Demetriades, A., De Vivo, B., De Vos, W., Duris, M., Gilucis, A., Gregorauskiene, V., Halamic, J., Heitzmann, P., Lima, A., Jordan, G., Klaver, G., Klein, P., Lis, J., Locutura, J., Marsina, K., Mazreku, A., O’Connor, P.J., Olsson, S.Å., Ottesen, R.T., Petersell, V., Plant, J.A., Reeder, S., Salpeteur, I., Sandström, H., Siewers, U., Steenfelt, A. & Tarvainen, T., 2005. FOREGS Geochemical Atlas of Europe, Part 1: Background Information, Methodology and Maps. Geological Survey of Finland, Espoo, 526 pp.; http://weppi.gtk.fi/publ/foregsatlas/.

Simmons, K.B., 2014. The triangulation method of stadia transit topographic surveying adapted to landscape architecture. University of Massachusetts Amherst, Master Theses 1911, 104 pp.; https://scholarworks.umass.edu/theses/1965.

Strahler, A.N., 1957. Quantitative analysis of watershed geomorphology. American Geophysical Union Transactions, 38(6), 912−920; https://doi.org/10.1029/TR038i006p00913.

Strahler, A.N., 1969. Physical Geography. Wiley & Sons, Inc., New York and London, 733 pp.

Triangulation station or Trigonometrical point or Trig point or Trig pillar: A survey point with known coordinates and elevation, which is marked on the ground by a concrete pillar with an engraved brass plate on its top, providing a mounting base for a theodolite. Trig points are usually situated on hilltops for the purposes of visibility. On a topographical map trig points are indicated by a small triangle, and its precise elevation recorded besides it.