International Union of Geological Sciences

Manual of Standard Methods

for

Establishing the Global Geochemical Reference Network

Chapter 3.2

Residual Soil and Humus Sampling

Maria João Batista1,10, Alecos Demetriades2,10, Christopher C. Johnson3,10, Timo Tarvainen4,10, Igor Savin5, David B. Smith10, Edith Haslinger6,10, Reijo Salminen4,

Virgilija Gregorauskienė7,10, Zomenia Zomeni8,10, Juan Pablo Lacassie Reyes9,10,

Felipe Astudillo9,10, Pablo Sebastian Oliva Vicentelo9,10

1   Laboratório Nacional de Energia e Geologia, Amadora, Portugal

2  Institute of Geology and Mineral Exploration, Athens, Hellenic Republic

3   GeoElementary, Derby, United Kingdom

4   Geological Survey of Finland, Espoo, Finland

5   V.V. Dokuchaev Soil Science Institute, People’s Friendship University of Russia, Moscow, Russian Federation

6  Austrian Institute of Technology GmbH, Centre for Energy, Vienna, Austria

7   Geological Survey of Lithuania, Vilnius, Lithuania

8   Geological Survey Department, Lefkosia, Cyprus

9  Servicio Nacional de Geología y Minería, Valdivia, Chile

10 IUGS Commission on Global Geochemical Baselines

2022

Published by

The International Union of Geological Sciences

Commission on Global Geochemical Baselines

It is recommended that reference to this part of the Manual should be made in the following way:

Batista, M.J., Demetriades, A., Johnson, C.C., Tarvainen, T., Savin, I., Smith, D.B., Haslinger, E., Salminen, R., Gregorauskienė, V., Zomeni, Z., Lacassie Reyes, J.P., Astudillo, F. & Oliva Vicentelo, P.S, 2022. Residual Soil and Humus Sampling. Chapter 3.2 In: Demetriades, A., Johnson, C.C., Smith, D.B., Ladenberger, A., Adánez Sanjuan, P., Argyraki, A., Stouraiti, C., Caritat, P. de, Knights, K.V., Prieto Rincón, G. & Simubali, G.N. (Editors), International Union of Geological Sciences Manual of Standard Methods for Establishing the Global Geochemical Reference Network. IUGS Commission on Global Geochemical Baselines, Athens, Hellenic Republic, Special Publication, 2, 91−119.

Published by

The International Union of Geological Sciences

Commission on Global Geochemical Baselines

P.O. Box 640 37, Zografou

GR-157 10 Athens

Hellenic Republic

http://www.iugs.org/

http://www.globalgeochemicalbaselines.eu/

National Library of Greece Cataloguing in Publication Data

A catalogue record for this electronic book is available from the

National Library of Greece

ISBN: 978-618-85049-1-2

Contents

3.2.1. Introduction	95

3.2.1.1. Polar and subpolar regions	99

3.2.1.2. Temperate regions	100

3.2.1.3. Mediterranean regions	100

3.2.1.4. Desert regions (arid and semi-arid)	100

3.2.1.5. Tropical regions	101

3.2.2. Residual soil and humus samples to be taken	101

3.2.2.1. Identifiers of residual soil and humus samples	102

3.2.3. Equipment for residual soil and humus sampling	103

3.2.3.1. Equipment to be provided by the Project Coordinator	103

3.2.3.2. Equipment to be purchased by each participating country	103

3.2.4. Sampling procedure for residual soil and humus	105

3.2.4.1. Sample site selection	105

3.2.4.2. Sampling procedure for residual soil	107

3.2.4.3. Sampling procedure for humus	109

3.2.4.4. Photographs to be taken at each sample site	110

3.2.4.5. Photographic documentation of residual soil sampling procedure	112

3.2.4.6. Photographic documentation of humus sampling procedure	116

References	117

Residual soil and humus samples are collected from second-order catchment basins

Blank back page

## 3.2.1. Introduction

Soil is a complex natural sampling medium. It is composed of individual mineral particles (e.g., quartz, feldspar, clay minerals, etc.) of widely varying composition, and these minerals are generally unevenly distributed throughout a given soil horizon. The nature of the soil is determined by the interaction of lithology, climate (e.g., temperature, precipitation), vegetation, relief (topography), biological activity and time (Dokuchaev, 1883; Jenny, 1941; Bridges, 1970; Buckman and Brady, 1970). All soil-forming factors have great spatial variability. This leads to considerable inhomogeneity even within the area of a single sampling site. Climate is considered to be one of the most important soil-forming factors, and forms the basis for the five regional classifications described below, namely polar and subpolar, temperate, Mediterranean, desert and tropical. Figure 3.2.1 shows the effects of climate (mainly temperature and precipitation) on soil formation from the North Pole to the Equator. Further, with increasing elevation, there are climatic changes, which affect in an analogous way soil development (Fig. 3.2.2; Strahler, 1969; Bridges, 1970; Badía et al., 2013, 2016). It is, therefore, quite apparent that soil differs in chemical, physical and biological characteristics in both space and time.

Figure 3.2.1. The effects of climate on soil worldwide (Britannica, 2012).

Figure 3.2.2. Schematic diagram showing the gradation of soil types from a dry steppe-climate basin (left) to a cool, humid climate (right) as one ascends the west slope of the Bighorn Mountains, Wyoming, USA (after Strahler, 1969, Fig. 19.6, p.317; redrawn with minor modifications by Alecos Demetriades, Hellenic Institute of Geology and Mineral Exploration (IGME) & IUGS Commission on Global Geochemical Baselines (IUGS-CGGB) with Golden Software’s MapViewerTM v8).

Haslinger et al. (Annexe A3.2.1 of this Chapter) describe the characteristics, geographical distribution, parent material, usage, and geochemical processes of the 28 major soil units, according to the classification of the “Harmonised World Soil Database” (FAO/IIASA/ISRIC/ISS-CAS/JRC, 2012), i.e., Regosols, Leptosols, Gleysols, Cambisols, Podzols, Arenosols, Calcisols, Histosols, Luvisols, Fluvisols, Podzoluvisols, Acrisols, Ferralsols, Kastanozems, Chernozems, Phaeozems, Solonetz, Vertisols, Lixisols, Solonchaks, Gypsisols, Greyzems, Andosols, Planosols, Nitisols, Plinthosols, Anthrosols, and Alisols. Further to the geochemical characteristics of the 28 soil units given by Haslinger et al. (Annexe A3.2.1 of this Chapter), a most useful classification for applied geochemists is that of Büdel (1982), which has been used as a basis for understanding the geochemistry of tropical and subtropical terrains in a definitive text by Butt and Zeegers (1992). The classification (Fig. 3.2.3) considers the extensive tropical weathering, which extended into high latitudes during the Cretaceous and Tertiary times, and the modification of large areas as a result of Pleistocene glaciation (Darnley et al., 1995). It does not, however, consider mountain belts, although changes with increasing altitude generally correspond to those associated with increasing latitude.

The variations in soil, types of duricrust, the thickness of weathering zone and their relationship to mean annual temperature and rainfall are shown in Figure 3.2.4. Variations in Eh, pH, electrical conductivity and the activity of some major cationic and anionic species have been added (Darnley et al., 1995). Apparently there is little chemical modification of bedrock chemistry in polar, sub-polar and cold temperate regions, in contrast to arid, tropical and equatorial terrains, which are typically affected by prolonged and deep weathering.

In some environments, for example, in lateritised terrains, considerable expertise is needed to interpret soil geochemical data (Blanchard, 1968; Butt and Zeegers, 1992).

Figure 3.2.3. World morphoclimatic regions, present climatic zones, and simple geomorphology (From Giles et al., 2017, Fig. 4, p.371; after Fookes, 1991, 1997). Note: The map is similar to that of Büdel (1982), which is displayed by Darnley et al. (1995, Fig. 5-2, p.47).

Figure 3.2.4. Generalised relationships between surface/groundwater chemistry and climate (From Darnley et al., 1995, Fig. 5-3, p. 48, based on Pédro, 1984). Notation: TM = Transition metals. Redrawn with minor modifications by Alecos Demetriades (IGME/IUGS-CGGB) with Golden Software’s Grapher™ v20.

In the Global Geochemical Reference Network project, two horizon-based samples of residual mineral soil shall be collected from the second-order drainage basin with an area of <100 km2 (refer to Section §2.5 in Chapter 2 of this Manual), as the aim is to collect samples unaffected by human activities and to determine the natural variation worldwide. A Τopsoil sample is collected representing the uppermost mineral soil A horizon (most often occurring at the air/earth interface or directly below the litter and/or organic material horizons), and a deeper Bottomsoil sample is collected from the soil C horizon (generally partially weathered parent material) from the same vertical profile (see Fig. 3.2.5).

The Topsoil A horizon (non-humic or humic) sample could vary in thickness from site to site and the depth range should be recorded on the field observations sheet, i.e., the top boundary is between the litter and/or organic horizon and the A horizon, and the bottom boundary is between the A and either the E or B soil horizons, depending on soil type. If, however, the thickness of the residual A soil horizon is greater than 20 cm, then the top 20-cm is arbitrarily selected as the targeted sample. This is consistent with the sampling thickness of ploughed agricultural fields on the one hand, if a qualitative comparison between the two data sets needs to be made, and on the other, a greater thickness will most likely not show human influences. Further, the experience from the FOREGS Geochemical Atlas of Europe project (Salminen et al., 2005) has shown that the residual A soil horizon from second-order catchment basins rarely exceeds 20-cm in thickness.

The Bottomsoil C horizon sample should be up to 20-cm in thickness beginning at the top of the soil C horizon, i.e., from the boundary between the B and C horizons. If the Bottomsoil C

Figure 3.2.5. Defining criteria of the two residual soil samples for the horizon-based sampling method. Drawn by Christopher C. Johnson, GeoElementary, Derby, United Kingdom, with Microsoft’s PowerPoint.

horizon is less than 20 cm in thickness, then the entire horizon should be sampled. In either case, the depth range of the upper and lower limits of the sampled horizon should be noted on the field observations sheet.

Residual soil is formed in-situ by the weathering and disintegration of parent materials (e.g., rock, till, loess) followed by pedological processes that lead to the formation of soil horizons (Fig. 3.2.6). The collected samples should reflect the geochemical variation in the underlying soil parent materials. It is important to avoid sampling at locations that have visible or known contamination (Salminen, Tarvainen et al., 1998). Comparison between the geochemical results of Topsoil and Bottomsoil samples will provide information about enrichment or depletion processes between the two soil horizons. One such process is anthropogenic contamination of the Topsoil horizon.

Figure 3.2.6. The soil profile shows the major layers from the O horizon (organic material) to the R horizon (consolidated rock). A pedon is the smallest unit of the land surface that can be used to study the characteristic soil profile of a landscape. The top and bottom residual soil samples are collected from the A and C soil horizons, respectively, and the humus sample from the highly decomposed organic material (Oa) horizon. Source: Britannica (2012).

Recent highly decomposed organic matter (Oa horizon), derived from an assemblage of vegetation, which is referred to as humus in this Manual (Fig. 3.2.6), can be used to determine the atmospheric (anthropogenic) input of chemical elements to the ecosystem (Salminen, Tarvainen et al., 1998). Further, as suggested by Darnley et al. (1995), the collection and analysis of surface humus, provides a pointer to the bioavailability of elements. Although element concentrations in the humus samples are generally a good indication of long term atmospheric deposition, some elements (e.g., Ag, Cd, Ge, Hg, Pb) have a natural tendency to enrich humus samples (Reimann et al., 2007, 2015; Flem et al., 2018).

The collection of a separate recently formed humus sample, normally occurring directly above the soil A humic mineral horizon, is carried out only at sites where highly decomposed organic material is developed (Fig. 3.2.6). Its thickness is normally less than 5-10 cm. The highly decomposed organic matter horizon (Oa) is developed under specific conditions, namely:

- Availability of biomass, biological activity, temperature, moisture, water saturation, topography, and

- In colder climate regimes, production exceeds the destruction of organic matter and humus accumulates. As a rule of thumb, above a mean annual temperature of 25oC, destruction exceeds production and humus is absent (Strahler, 1963, 1969).

These conditions depict that highly decomposed organic matter cannot be found at every residual soil sampling site. It is, therefore, worth mentioning that in the FOREGS Geochemical Atlas of Europe project, out of the 837 residual soil sampling sites, located in second-order catchment basins, only 367 humus samples were collected (Salminen et al., 2005), i.e., a proportion of about 44%, which is expected to be much lower in the global project.

### 3.2.1.1. Polar and subpolar regions

The polar regions are to the north and south of the 66.5⁰ parallels (Arctic and Antarctic Circles), and the subpolar regions are between 50⁰ and 70⁰ latitude north and south of the equator. These regions are presently glaciated or were glaciated during the Pleistocene. The major processes of physical weathering and disintegration of rocks are the action of (i) glaciers, (ii) water in various forms (liquid, ice, vapour), (iii) salt weathering, (iv) insolation (i.e., exposure to the sun’s rays), and (v) wind (Campbell and Claridge, 1987). Chemical weathering is relatively insignificant, and normally glacial action removes chemical weathering residua in most areas, leaving bare rock pavements or exotic morainic and other types of clastic deposits. Organic matter is not readily oxidised in the low ambient temperatures of such environments and thick accumulations of peat or bogland (muskeg) may give rise to surface conditions characterised by low Eh/pH (Darnley et al., 1995).

The most common soil type in polar and subpolar regions is Podzol, which is characteristic of humid regions and strongly leached. Other soil types are Histosols (an organic rather than a mineral soil), Luvisols (iron-depleted and extremely low in organic content), Gleysols (water-saturated), and Cambisols (soil with incipient soil formation) - (refer to Annexe A3.2.1 of this Chapter to see examples of these sample types).

Another soil type occurring in polar and subpolar regions, which is not included in the Harmonized World Soil Database (FAO/IIASA/ISRIC/ISS-CAS/JRC, 2012), is Cryosol (also referred to as Gelisol, Cryozem, Cryomorphic soil and Polar Desert soil). Cryosols are characterised by frozen soil within 1 metre of the land surface (permafrost), and by waterlogging during periods of thaw (Jones et al., 2009). They often show disrupted soil layers, cracks, or patterned surface features such as frost mounds, caused by the physical actions of ice formation and melting (Britannica, 2016). Cryosols may be either mineral soil or humus-rich material.

Iron and Mn may be mobilised in the mainly podzolic soil profiles and redeposited in lower horizons or stream channels, giving rise to anomalously high values for the first-row transition elements, and also Ba and Ag (Darnley et al., 1995).

### 3.2.1.2. Temperate regions

The north and south temperate regions extend from latitude 23.5⁰ to 50⁰ in both hemispheres. In general, temperate regions are characterised by increased rainfall and temperature relative to polar and subpolar regions. The characteristic soil types are Cambisols and Chernozems (refer to Annexe A3.2.1 of this Chapter). The pH/Eh and conductivity of surface water generally increase relative to polar regions (Fig. 3.2.4; Darnley et al., 1995). Temperate regions are characterised by having four seasons in the year, depending on rainfall and temperature. Therefore, chemical weathering and leaching of chemical elements through the soil profile are the most crucial factors in soil development. The weathering in this case forms iron hydroxides such as amorphous hydroxides, goethite Fe(OH)3, oxyhydroxides FeOOH, and also clay minerals derived from muscovite, such as illite by the loss of K+ ions. The formation of iron oxides, such as goethite, gives the soil a yellowish-brown colour. In temperate climatic regions with distinct dry summers, the formation of haematite (Fe2O3) leads to more red colours in soil. When the soil parent material is calcareous, chemical weathering results in dissolution of carbonates and, subsequently, the previously mentioned processes may also occur (Spaargaren and Deckers, 2005). Temperate regions correspond to 7% of the surface land area of the earth, where approximately 40% of the global population lives.

### 3.2.1.3. Mediterranean regions

Mediterranean climatic regions occur in both hemispheres between 30⁰ and 45⁰ latitude. They are found around the Mediterranean Sea, and on the western side of the other continents, i.e., California, Chile, the Western Cape Province of South Africa, and West and South Australia (Fig. 3.2.3). The Mediterranean regions are characterised by warm to hot, dry summers, and cool to mild, wet winters. The most common soil types in the Mediterranean regions are, according to the Harmonised World Soil Database (FAO/IIASA/ISRIC/ISS-CAS/JRC, 2012), Cambisols, Calcisols, Leptosols, Luvisols and Regosols (see Annexe A3.2.1 of this Chapter). The best-known example is ‘terra rossa’ or Rhodic and Chromic Luvisol developed on carbonate rocks (Zdruli et al., 2010). The climatic conditions are such for effective the dissolution and leaching of CaCO3 and other easily soluble salts, as well as the migration of clay (Verheye and De la Rosa, 2005). During the hot and dry summer months, the soil desiccates, causing the development of red dehydrated oxidised iron compounds (e.g., haematite, goethite) within the profile.

### 3.2.1.4. Desert regions (arid and semi-arid)

Desert regions are a difficult terrain type to define. Their common characteristic is that they are arid as they receive very little precipitation (<250 mm/annum). Meigs (1953) divided desert regions into three categories according to the amount of precipitation they receive. In this widely accepted system, extremely arid lands have at least 12 consecutive months without rainfall, arid lands have <250 mm annual rainfall, and semi-arid lands have mean annual precipitation between 250 and 500 mm; arid and extremely arid lands are deserts, and semi-arid grasslands generally are referred to as steppes and prairies. More recently, National Geographic classified the world’s deserts into five types according to the causes of their dryness, i.e., (i) subtropical (Sahara, Kalahari, Tanami), (ii) coastal (Atacama), (iii) rain shadow (Death Valley), (iv) interior (Gobi), and (v) polar (parts of the Arctic and the Antarctic).

Subtropical or hot deserts are characterised by exceptionally low rainfall with high evaporation rates as a result of high temperatures and/or strong winds, and they are affected by temperature extremes (Darnley et al., 1995). The most extensive and well-documented deserts are hot deserts characterised by extremely high temperatures, such as those of North Africa, Saudi Arabia and Western Australia. Evidence exists that some hot deserts were formerly subjected to extreme leaching under tropical conditions during the Cretaceous and Tertiary. In Western Australia, it has been suggested that the landscape was formed initially during the Proterozoic (Butt and Zeegers, 1992).

Desert soil is predominantly immature mineral soil, weakly developed in terms of its vertical profile, and mostly alkaline (Walker, 1992), and occurs in most cases below a stone pavement (McAuliffe et al., 2018). Sands, sandy or gravelly loam, shallow stony soil, and alluvium (deposited by ephemeral rivers and streams) and scree-derived deposits predominate (Rafferty, 2011). The repeated accumulation of water in some desert soil causes distinct salt layers to form. Calcium carbonate precipitated from solution may cement sand and gravel into hard layers called ‘calcrete’, forming in some cases layers up to 50-m thick. Caliche is a reddish-brown to white layer found in many desert soils. Caliche commonly occurs as nodules or as coatings on mineral grains formed by the complicated interaction between water and carbon dioxide released by plant roots or by decaying organic material.

The usual topographic sequence (catena) of soil in arid regions begins with shallow rocky soil on the barren mountains and hills, then progresses downslope to coarse-textured and deeper soil on the dissected upper alluvial fans, and is followed on the lower fans and plains by finer-textured and deeper soil with more well-defined carbonate and clay horizons (Dregne, 1976).

Important constituents in desert soil are resistate detrital quartz and zircon, and secondary clay minerals. Heavy minerals such as zircon and magnetite, may become enriched in these soil types due to the removal of lower density minerals (Petrov, 1976). Calcareous soil is enriched in elements that are mobile in conditions of high pH, a feature of inland drainage basins. Soluble salts (halite, gypsum) may be present in quantities sufficient to alter the mobility of trace elements in the surficial environment (Berkel, 1982). Biological activity and, therefore, organic matter in desert soil is minimal or non-existent.

### 3.2.1.5. Tropical regions

Surface soil in tropical environments may have been deeply leached under a range of climatic conditions, including lengthy periods of high rainfall and high temperature, to which they have been subjected from Tertiary times or earlier, in some areas as far back as the late Proterozoic (Butt and Zeegers, 1992).

Surface conditions vary widely but there is generally intense oxidation, a lack of organic debris (which is generally stored in the biota rather than in soil), and the formation of stable insoluble secondary minerals such as clays (especially kaolinite) which increase the Al/Si ratio of the surface material (Darnley et al., 1995).

In some tropical environments, laterite is common and may extend tens or hundreds of metres downwards before the fresh rock is encountered. The surface may be extremely complex, and vertical soil profiles may be truncated and/or buried by later detritus (Butt and Zeegers, 1992).

## 3.2.2. Residual soil and humus samples to be taken

The following horizon-based residual soil and humus samples are collected:

| - Routine sample site of a normal 160x160 km grid cell: - 1 – 2 litres of Humus where it is present (highly decomposed organic material). - 2 – 3 kg of residual Topsoil (Top) sample representing the soil A non-humic or humic horizon, which may occur at the air/earth interface or below the litter or organic material horizon, where present. The maximum sampling range (thickness) of the Topsoil sample should not exceed 20 cm, i.e., the upper 20 cm of the A mineral soil horizon are sampled. If the soil A horizon is less than 20-cm thick, then the Topsoil sample is collected from the entire horizon. The depth range of the sampled Topsoil A non-humic or humic horizon should be noted on the field observations sheet. - 2 – 3 kg of residual Bottomsoil (Bottom) sample. This sample should be collected from the soil C horizon, which consists, generally, of the partially weathered parent material. A 20-cm thick section should be sampled beginning at the top of the C horizon, i.e., below the B-C boundary. In some cases, the C horizon may be at a depth of more than 100 cm, and in such cases, the pit should be dug until it is reached. However, if the C horizon is deeper than 200 cm, then a 20-cm thick sample is collected from a depth of 200 to 220 cm (or variants of 180 to 200 cm or 190 to 210 cm - depending on local conditions), and always from the same horizon. The depth range is noted on the field observations sheet. If the thickness of the soil C horizon is less than 20 cm, then the entire horizon is sampled, and the depth range is recorded. - Duplicate field sample site: At least one site out of twenty is selected for the collection of a set of duplicate field samples (i.e., ≈5% duplication of the sample sites) in each country. However, countries with less than 20 GTN grid cells should collect field duplicate residual soil samples from at least one random site. The field duplicate sample site is selected at a distance of 5 to 50 metres away from the routine sampling site. Additional Top and Bottom residual soil samples, and an additional recently formed Humus sample (where present) are collected, according to the sampling scheme summarised in (a) above. |  |
| --- | --- |

Enough material must be collected to yield a minimum of 1 kg of <2 mm grain size for each residual Top and Bottom soil sample. Larger sample quantities can be taken and stored separately in each country.

Refer to Annexe A3.2.1 of this Chapter for a description of the characteristics, geographical distribution, parent material, usage, and geochemical processes of the 28 major soil units, and Annexe A3.2.2 where the sampling scheme is shown using selected examples of diverse types of soil profiles.

### 3.2.2.1. Identifiers of residual soil and humus samples

The identifiers of the Top and Bottom residual soil samples are ‘T’ and ‘C’, respectively, and for Humus samples (where present) the identifier is ‘H’:

- Routine sample site (e.g., GTN grid cell N26E14): - Topsoil sample: N26E14T1 - Bottomsoil sample: N26E14C1 - Humus sample (where present): N26E14H1 Note: Number ‘1’ represents the 1st random sample site in GTN grid cell N26E14.

- Duplicate field sample site (e.g., GTN grid cell N27E12): - Routine Topsoil sample – Routine: N27E12T3 - Duplicate Topsoil sample – Duplicate: N27E12T3D - Routine Bottomsoil sample – Routine: N27E12C3 - Duplicate Bottomsoil sample – Duplicate: N27E12C3D Note: Number ‘3’ represents the 3rd random sample site in GTN grid cell N27E12, and ‘D’ denotes the duplicate Top and Bottom residual soil samples.

## 3.2.3. Equipment for residual soil and humus sampling

### 3.2.3.1. Equipment to be provided by the Project Coordinator

The following equipment must be purchased or made centrally, and provided to all sampling teams in each participating country:

  - 300x600x0.04 mm strong certified trace-element free Rilsan® plastic or similar bags for packing residual soil samples (such bags are also suitable for organic compounds analysis).

  - 450x750 mm white cotton bags with drawstring free of chemical impregnation for packing humus samples.

  - Plastic strip locks for securing the sample bags (attention: the plastic strip locks cannot be opened once closed; this is a safety precaution for checking that the samples have not been tampered with from the time of sampling until they reach the sample preparation laboratory).

  - 6x10 cm white cards for writing the sample number on both sides.

  - 7.5x11.5 cm zip-lock plastic bags for holding the 6x10 cm white cards.

  - Plastic laminated scalebar for ‘photographs’ (see Fig. 3.3. in Chapter 3), and

  - Black permanent drawing ink markers (ONLY black coloured allowed).

Instead of the strong certified trace-element free plastic bags for packing the residual soil samples, two other types of bags can be used, and purchased by the Project Coordinator and distributed to all participating countries. These are:

- 125x250 mm wet-strength Kraft paper bags with a plastic-coated aluminium tin-tie, which can be twisted to seal the bag, or

- 254x431 mm polyester bags with nylon cord drawstring.

Whatever the decision, the same type of sample bags must be used by all countries throughout the Global Geochemical Reference Network project.

### 3.2.3.2. Equipment to be purchased by each participating country

Each participating country must purchase the following equipment, and for all its sampling teams:

  - Stainless-steel spade with an unpainted wooden handle is preferred. If this is not available, then an unpainted steel spade with an unpainted wooden handle is the next option. However, if these cannot be found and painted spade and varnished wooden handle are purchased, the paint/varnish must be removed by sandblasting before sampling (see Fig. 3.1 in Chapter 3); sandpapering either the spade or wooden handle will not remove completely the paint/varnish.

  - Unpainted mattock cutter (if mattock cutter is painted, the paint must be removed by sandblasting prior to sampling); the wood handle must be unpainted (if varnished, it must be removed by sandblasting – not sandpapering – see Figure 3.1 in Chapter 3).

  - Cylindrical sampler for sampling humus.

  - Hard white plastic spatula for removing mineral soil from humus samples.

  - Powder-free disposable gloves for sampling humus.

  - Stainless steel knife.

  - Metal-free white or colourless plastic scoop or stainless-steel scoop.

  - Stainless-steel geological hammer either pointed-tip or chisel-end (preferable).

  - Leather gloves.

  - 10x magnifying lens (loupe).

  - Hard bristle brush for cleaning plastic scoop, and geological hammer.

  - Cotton-lint or white cotton rags for cleaning sampling equipment.

  - Wooden folding 2-m long measure (alternate colours every 10 or 20 cm); or plastic tape with alternate colours every 10 cm.

  - 30x60 cm sturdy plastic bags for packing sample bags, as a safety precaution during transportation to the sample preparation laboratory.

  - Plastic strip locks (or plastic cable ties) for securing the outside plastic sample bags.

  - Plastic or carton boxes for packing sample bags in the field, and subsequent transportation to the sample preparation laboratory.

  - Strong PVC packing tape.

  - 6 mm natural sisal rope.

  - Heavy-duty cutter knife with replacement blades.

  - Global Positioning System (GPS) for recording sample site coordinates, or a tablet with a GPS and digital topographical maps; or a mobile telephone with GPS; it is strongly recommended to have a backup GPS receiver.

  - Extra batteries for GPS or battery charger with rechargeable batteries.

  - Topographical maps, preferred scale 1:50,000 (a must in case electronic digital devices fail, e.g., GPS).

  - A plastic roamer or map ruler for extracting coordinates from 1:50,000 topographical maps in case GPS fails.

  - An orienteering compass (a must in case GPS fails).

  - Digital camera or a mobile telephone with a macro facility for field documentation (minimum 5 megapixels).

  - Extra batteries for the digital camera.

  - Field-ruggedised notebook or laptop computer with extra charger and spare batteries.

  - Car adapter for charging notebook or laptop computer.

  - Portable storage device (USB memory stick or external hard drive) for backup of field data and digital photographs.

  - USB cable to download photographs to a laptop computer daily.

  - Threshold scintillometer to measure natural radioactivity (Total, Th, U, K); the scintillometer must be calibrated at least once a year before the field campaign at a national facility that is certified.

  - Extra batteries for the threshold scintillometer.

  - Field observations sheets.

  - Waterproof case to hold field observation sheets.

  - Writing pens.

  - HB pencils, pencil sharpener and rubber (back-up in case the pens fail to write in the field).

  - First-aid kit, and

  - Mobile telephone or other communication equipment like CB radios and satellite telephone (the latter may be needed in remote areas), or emergency position-indicating radio beacon (EPIRB).

Important Notes for the use of an auger in soil sampling: - Global Geochemical Reference Network project: The use of an auger for sampling residual soil is NOT recommended because three-dimensional information about the soil sampling site can be obtained only from a soil pit.

- Exploration Geochemistry projects: The use of an auger for soil sampling does have a place in applied geochemistry when a large number of samples are collected over a small area; for example, along traverse lines in exploration geochemistry.

## 3.2.4. Sampling procedure for residual soil and humus

### 3.2.4.1. Sample site selection

During the desktop study, the second-order drainage basin (<100 km2) is selected after studying the 1:50,000 scale topographical, geological and soil maps (refer to Section §2.5 of Chapter 2 in this Manual). It is stressed that the residual soil samples should represent the dominant residual soil type of the second-order catchment basin (Strahler, 1957, 1969). Hence, if soil maps are available, they should be studied in order to identify the dominant residual soil type. If not, then the geological map should be studied, and in this case the residual soil to be taken over the dominant rock type of the second-order catchment basin.

The exact position of the residual soil sampling site is located in the field, after a careful study of the area covered by the dominant residual soil (or rock) type. The sample site must be above the alluvial plain and base-of-slope of the second-order stream, where alluvium and colluvium are respectively deposited (see Annexe A3.2.2 of this Chapter). The sample site must be situated on the dominant relief position, avoiding slopes with erosion and transport (see Chapter 2 and Annexe A3.2.1 of this Chapter). Residual soil may be developed either directly on bedrock or on till, as is the case in glaciated terrains, or on loess.

It is stressed that the aim of the Global Geochemical Reference Network project is to sample natural residual soil, meaning soil that is not affected by any anthropogenic activities. Such natural sampling sites in the small second-order drainage basin of <100 km2 are usually found in grasslands and forested areas.

The residual soil sampling site is selected according to the following criteria: -

- Avoid possible contamination by selecting a sample site that is at least:

- 100 m away from asphalted roads (particularly major roads/highways), railway lines, bridges, buildings, and dams.

- 50 m away from rural or dirt roads.

- 50 m away from ditches.

- 100 m away from buildings.

- 25 m away from fences.

- 100 m from high-power electric lines, and

- 2 km away from active major industrial activity, such as electric power plants or smelters.

- Avoid agricultural fields as the surface soil down to a depth of 20 cm (normal ploughing depth) to over 50 cm (deep ploughing depth) has been disturbed by human activities.

- Avoid sites that are disturbed by human activities such as camping sites (e.g., presence of fire-places, cans and/or bottles), graded land (e.g., areas from which soil has been removed and the remaining soil cannot be classified in any soil type), levelled fields (for irrigation), mines (active or abandoned), landfills, and rehabilitated sites. It is stressed that every single contaminated site will seriously influence the generated geochemical results.

- Avoid locally atypical sites.

- Avoid, if possible, collecting soil containing large quantities of organic material or rich in Fe or Mn oxides.

Important note for selecting residual soil sampling site: As the purpose of the wide-spaced geochemical survey is the establishment of a Global Geochemical Reference Network and the 21st-century geochemical baseline of residual soil against which future changes may be recognised and quantified, even a single contaminated site will seriously influence the geochemical results. Hence, the project aims to collect the ‘most representative natural’ residual soil sample, and not the ‘most unusual’ residual soil sample. It is, therefore, of paramount importance to ensure that the material collected is ‘residual’, and not transported such as colluvium or alluvium, and is not affected by human activities.

Important note for selecting a sampling site with recent highly decomposed organic material (humus): The sampling site should be as flat-lying as possible, unless the sample is collected from a mountainous area (forest or grassland). Local depressions with humus must be avoided. If the sample site is in a forested area, all humus sampling locations should be selected at a sufficient distance from the nearest trees to avoid ‘throughfall’ precipitation from the trees. A minimum distance of 5 m from the nearest tree, and 3 m from the nearest bush is recommended. However, each sampling team is free to use its discretion in the selection of humus subsites in forested areas. It is noted that these conditions apply also to the selection of the residual soil sampling site.

Annexes A3.2.1 and A3.2.2 of this Chapter demonstrate the complexity of soil types and profiles from different morphoclimatic environments around the globe, yet in this chapter, the best effort of a single method for collecting residual soil samples from anywhere in the world is provided. With the aid of Annexe A3.2.2, clear pictorial help is given to define where in the soil profile samples of Top and Bottom residual soil, as well as Humus (where present), are to be collected by following the standardised method presented in this chapter for their collection in a consistent and representative manner.

The method requires that samplers use good observational skills at sampling locations. Such skills should be acquired from a soil sampling training course. The procedures for describing the soil at location (including photographs - Fig. 3.2.7) are a vital part of providing information and a context within which users of the data generated can interpret results.

### 3.2.4.2. Sampling procedure for residual soil

Two different horizon-based residual soil samples are taken at each site:

- A Topsoil sample (abbreviated Top) is collected, representing the soil A non-humic or humic horizon, and which occurs either at the air/earth interface, or directly below the surface litter and/or highly decomposed organic (Oa) material horizon (humus), where present. As noted above, the maximum sampling thickness range of the Topsoil sample should not exceed 20 cm, i.e., the upper 20 cm of the soil A non-humic or humic horizon are sampled. If the soil A non-humic or humic horizon is less than 20 cm thick, then the Topsoil sample is collected from the entire thickness of the horizon. The depth range of the sampled soil A non-humic or humic horizon should be noted on the field observations sheet. Although the measure is placed at the air/earth interface of the profile, the 0 cm depth essentially starts from the upper mineral soil surface, i.e., the thickness of the litter or organic material (Oa) horizon, where present, is not considered, as these parts are removed before sampling the soil A non-humic or humic and C horizons. Hence, the A non-humic or humic mineral soil surface at the air/earth interface of the profile is considered to be the top boundary of the first mineral soil horizon that can support plant/root growth. This equates to for:

- Bare mineral soil: the air/earth interface, and

- Vegetated mineral soil: the upper boundary of the first horizon that supports root growth, excluding both freshly fallen plant litter, and litter that has compacted and begun to decompose but remains somewhat recognisable, and includes slightly and moderately decomposed organic material – see Fig. 3.2.7); and

- A Bottomsoil sample (abbreviated Bottom) is collected from the soil C horizon (generally partially weathered parent material). The soil C horizon sample should be approximately 20 cm in thickness beginning at the top of the C horizon, i.e., the B-C boundary. If the soil C horizon is less than 20-cm thick, then the entire horizon should be sampled, and the depth range recorded on the field observations sheet.

At each residual soil sample site:

- Switch on the GPS (Fig. 3.2.8a), or notebook or laptop computer with GPS (Fig. 3.2.8b), to obtain the WGS1984 decimal degree coordinates of the sample site. It is important to switch on the GPS upon arrival at the site, and to allow enough time for the signal to settle.

- Write the sample numbers of Top and Bottom residual soil samples with a black permanent ink marker on a plastic trace element free plastic Rilsan® bag (or Kraft paper or cotton bag; Figs. 3.2.9a-b).

- Write the sample numbers of Top and Bottom residual soil samples on both sides of a small card, and place each card in a small plastic zip-lock bag and seal it (Figs. 3.2.9c-d).

- Clean spade and mattock cutter by sticking it several times into the soil at each new sample site.

- Remove surface litter and grass to begin with a knife or spade, and a mattock cutter if necessary.

- Dig a pit down to the C horizon with a stainless-steel or an unpainted steel spade and mattock cutter, thus uncovering a clean vertical surface for sampling (Fig. 3.2.10). If it is at all possible, the pit should be rectangular in plan-view with the vertical face for sampling and description to be facing, preferably, the sun or the direction of maximum light; when marking out the pit, allowance should be made for the movement of the sun, which will occur during digging, so that there are no shadows in the corners of the pit during the taking of photographs (Hodgson, 1976).

- Mark the soil horizons with the aid of a stainless-steel knife (Figs. 3.2.11a).

- Place an alternate coloured-section 2-m measure on the vertical face of the pit and secure it (Fig. 3.2.11b).

- Photographic documentation of sample site: At this stage, ensure that the alternate coloured-section wooden 2-m measure is on the face of the pit, and then take at least six digital photographs (see Section §3.2.4.4 & Fig. 3.2.7).

Additional photographs can be taken to show the textural characteristics of both the Top and Bottom residual soil horizons.

IMPORTANT CAUTION: As a safety precaution, always photograph first the sample number of the Top residual soil sample (Fig. 3.2.7a). Thus, the succeeding photographs are classified to belong to this particular sampling site.

- Mark the location of the sample site on the 1:50,000 topographical map or digital map; this is a safety precaution to ensure that the GPS coordinates are correct.

- Record sample site coordinates and general observations on the field observations sheet (refer to Appendix 1 of this Manual), leaving the grain size to be completed after the collection of the Top and Bottom soil samples.

- First, collect the Bottom C horizon residual soil sample (Figs. 3.2.11c-f) using a geological hammer to excavate the soil from the pit face, and a white plastic scoop to collect each soil aliquot, which is placed in a strong certified trace-element free plastic Rilsan® bag (or Kraft paper bag; Figs. 3.2.11d-f) until the required amount is collected. This procedure avoids cleaning the surface of the Bottom soil horizon from fallen Topsoil sample material, if the latter is taken first. Handpick and remove any large rock fragments.

- Upon collecting the Bottom soil (C horizon) sample of about 2–3 kg weight, the numbered small card in the plastic zip-lock bag is placed on top of the sample (Fig. 3.2.11e).

- Twist the top of the sample bag, and seal it securely with a plastic strip lock (Fig. 3.2.11f).

- For safety during transportation of the sample, place the sample bag in a strong ordinary plastic bag, and seal it securely with a plastic strip lock (Fig. 3.2.13b).

- Clean thoroughly the sampling equipment with a hard bristle brush and cotton lint.

- Second, collect the Top residual soil sample from the A horizon using the same procedure as that for the Bottom sample (refer to steps (xii) to (xvi); Figs. 3.2.12a-f & Figs. 3.2.13a-b). Handpick and remove any large rock fragments.

- Store the two samples in different strong carton or plastic boxes.

- Switch on the portable threshold scintillometer, hold it at knee height and record the count readings of total radiation, K, U, and Th on the field observations sheet (Figs. 3.2.14a-b).

- Record the grain size of Top and Bottom residual soil samples, and digital photograph numbers on the field observations sheet (Fig. 3.2.14b; refer to Appendix 1 of this Manual).

- Take a photograph of the field observations sheet as a safety precaution in case the original sheet is somehow lost or becomes illegible.

Duplicate field samples: Duplicate Top and Bottom residual soil samples are collected randomly at least at every 20th sampling site (i.e., ≈5% duplication of the sample sites) in each country. However, countries with less than 20 GTN grid cells should collect duplicate Top and Bottom residual soil samples from at least one random site. The duplicate sample site is selected at a distance of 5 to 50 metres away from the routine sampling site following the same procedure as for collecting the routine Top and Bottom residual soil samples.

Important Note 1: After collecting the Top and Bottom residual soil samples, using the procedure described above, the dug-up soil is returned to the pit, the two samples are placed on the surface together with the sample number and GPS, and the last site digital photograph is taken to show that the pit was filled-in, and the landscape returned to its original state (Fig. 3.2.7g).

Important Note 2: The soil C horizon is especially important, and must be reached at all sampling sites because it is the reference horizon, i.e., the C horizon is composed of unconsolidated parent material, which may be either relatively unweathered or deeply weathered. Samples collected at sites where the C horizon is unweathered or only slightly weathered represent very closely the chemical composition of the parent material. The upper 20-cm thick section of the C horizon is sampled, and the depth range is noted on the field observations sheet. In case the C horizon has a thickness of less than 20 cm, then the entire horizon is sampled, and the thickness range is recorded on the field observations sheet. It is anticipated that the C horizon will be reached in most cases at a depth of less than 100 cm, or in the worst-case scenario is less than 200 cm. If it is deeper, then the pit should be dug until the C horizon is reached. In extreme cases, where the C horizon is deeper than 200 cm, the Bottom sample is collected from a depth range of 200 to 220 cm, and always from the same horizon, which means that this depth range, depending on local conditions, maybe slightly changed to either 180 to 200 cm or 190 to 210 cm or similar thickness variants below a depth of 180 cm (e.g., 185-205 cm).

Important Note 3: The Top and Bottom residual soil samples are always collected from a SINGLE site, and a SINGLE soil profile.

### 3.2.4.3. Sampling procedure for humus

If there is a recognisable well-developed highly decomposed organic material (humus) above the A humic residual mineral soil horizon, collect this sample first by using either a cylindrical steel sampler or a spade (Salminen, Tarvainen et al., 1998), and before digging the pit deeper to expose the other horizons. Each humus sample should be a composite sample from at least five (5) locations within a 50x50 m area around the residual soil sampling site, and the uppermost three (3) cm of humus should be sampled. The reason for collecting a composite humus sample is that from a single site it is not possible to collect the required 1 to 2 litres. Hence, the first humus sub-sample is collected from the site of the pit that will be dug for the collection of the Top and Bottom residual soil samples. The additional four sub-sites (at least) are located around this location. Refer to “Important note for selecting sampling site with recent highly decomposed organic material (humus)” (Section §3.2.4.1) before starting the sampling of humus. Collect the humus samples using the following sampling procedure:

- Write the humus sample number with a black permanent ink marker on a white cotton bag with a drawstring free of chemical impregnation or a certified trace-element free plastic Rilsan® bag (Fig. 3.2.15a).

- Write the humus sample number on both sides of a small card, place the card in a small plastic zip-lock bag and seal it (Fig. 3.2.15b).

- Wear on both hands powder-free disposable gloves (Figs. 3.2.16a-d).

- Remove carefully from each location the living surface vegetation, fresh litter, big roots and rock fragments.

- Drive the cylindrical steel sampler into the humus and down to a depth of 3 cm, twist it to cut the highly decomposed organic material, and then lift it up. If a spade is used for the collection of the humus sample, cut with the spade square blocks down to the boundary with the A humic residual soil horizon, and sample the upper 3-cm of the highly decomposed organic material.

- Check if mineral soil is attached to the bottom of the retrieved humus aliquot; if mineral soil is attached, remove it very carefully using a hard plastic spatula (Fig. 3.2.16b).

- Place each humus aliquot in the white cotton bag (or certified trace-element free Rilsan® bag), and press it down hard.

- Repeat steps (iv) to (vii) at other locations within the 50x50 m area until 2 litres of compacted humus are collected.

- Upon collecting the humus sample, the numbered small card in the plastic zip-lock bag (Fig. 3.2.15b) is placed on top of the sample.

- Twist the top of the cotton bag, and tie it securely with a drawstring (or secure the certified trace-element free Rilsan® bag with a plastic strip lock).

- Record observations on the field observations sheet (Fig. 3.2.14b; refer to Appendix 1 of this Manual).

- Upon collecting each humus sample, clean thoroughly the sampling equipment.

- After collecting the Humus sample, the pit is dug further down to reach the C horizon for the collection of the Bottom C horizon sample first, and the Top A humic mineral soil sample second by following the procedure described in Section §3.2.4.2.

Important cautionary note: Do not collect raw humus because it is extremely difficult to define an exact boundary between litter and raw humus.

### 3.2.4.4. Photographs to be taken at each sample site

At each residual soil sampling site at least 6 digital photographs (>5 megapixels) are taken (Figs. 3.2.7a-g):

- First photograph: Top residual soil sample site number (Fig. 3.2.7a).

- Second photograph: General landscape photograph of the sampling site (Fig. 3.2.7b).

- Third photograph: Soil surface photograph taken from a height of about 1 m from the ground surface (Fig. 3.2.7c); either the Top residual soil sample site number or plastic laminated scalebar should be placed on the ground surface.

- Fourth photograph: Close-up of sample pit with natural light. Before taking this photograph mark with a knife the soil horizons, if they can be distinguished, and place an alternate coloured-section wooden measure on the face of the pit (Figs. 3.2.7d-e).

- Fifth photograph: Close-up of sample pit using fill-in flash for it is important to show the horizons and textural characteristics of the soil profile (Fig. 3.2.7f), and

- Sixth photograph: This is an important photograph as proof that dug-up soil has been returned to the pit, and the landscape to its original state. Place on top of the fill-in pit (a) sample number, (b) sample bags, and (c) GPS and then take the photograph (Fig. 3.2.7g).

| (a) |  |  |
| --- | --- | --- |
| (b) | (c) |  |
| (d) | (e) | (f) |
| (g) |  |  |

Figure 3.2.7. Photographs (a) Residual soil sample site number. (b) General landscape. (c) Surface of sample site taken from a height of 1 m. (d) Pit showing residual soil profile. (e) Close-up of the residual soil profile with natural light. (f) Close-up with fill-in flash showing textural characteristics of soil horizons, and (g) Last photograph of bagged Top and Bottom residual soil samples with sample number and GPS on top of fill-in pit, as evidence that dug-up soil has been returned to the pit, and the land to its original state. Photographs: Alecos Demetriades (IGME/IUGS-CGGB).

### 3.2.4.5. Photographic documentation of residual soil sampling procedure

The following photographs show the residual soil sampling procedure.

| (a) | (b) | (c) |
| --- | --- | --- |

Figure 3.2.8. Photographs showing (a) GPS instrument and (b) Laptop with GPS for recording sample site coordinates − Step (i) of sampling procedure (see Section §3.2.4.2). The reason for switching on the GPS upon reaching the sample site is that in a few remote and forested areas it takes some time to locate the satellites. If the GPS is struggling to find the three satellites to triangulate the sample site coordinates without success, as in this case, move it a few metres to an open space or the nearest forest track road (c). Photographs: Alecos Demetriades  (IGME/IUGS-CGGB).

| (a) | (b) |
| --- | --- |
| (c) | (d) |

Figure 3.2.9. Photographs showing Steps (ii) and (iii) of the sampling procedure (see Section §3.2.4.2): (a & b) Write sample numbers of Top and Bottom residual soil samples with a black permanent ink marker on a white Kraft paper bag (this case) or plastic trace element free Rilsan® bag, and (c & d) write sample numbers of Top and Bottom residual soil samples on both sides of a small card, and place each card in a small plastic zip-lock bag and seal it. Photographs: Alecos Demetriades (IGME/IUGS-CGGB).

| (a) | (b) |
| --- | --- |

Figure 3.2.10. Photographs showing the procedure of digging a pit with (a) Mattock cutter, and (b) Spade (Step (vi) of sampling procedure – see Section §3.2.4.2). Photographs: Alecos Demetriades (IGME/IUGS-CGGB).

| (a) | (b) |
| --- | --- |
| (c) | (d) |
| (e) | (f) |

Figure 3.2.11. Photographs showing (a) Marking of soil horizons with the stainless-steel knife (Step (vii) of sampling procedure − see Section §3.2.4.2). (b) Placing and securing of 2-m measure on vertical pit phase (Step (viii)). (c) Sampling of Bottom residual soil sample using a geological hammer for excavating soil, which is then collected in a white plastic scoop (Step (xii)). (d) Bagging each Bottom soil aliquot in white Kraft bag (Step (xiii)). (e) Small, numbered card in a plastic zip-lock bag is placed on top of Bottom soil sample in white Kraft bag (Step (xiii)), and (f) Sealing white Kraft bag by twisting the aluminium tin tie (Step (xiv)). Photographs: Alecos Demetriades (IGME/IUGS-CGGB).

| (a) | (b) |
| --- | --- |
| (c) | (d) |
| (e) | (f) |

Figure 3.2.12. Photographs showing (a) Removal of grass and litter with the aid of a spade. (b) Sampling of Topsoil sample using a geological hammer for excavating soil, which is then collected in a white plastic scoop (Step (xii) of sampling procedure – see Section §3.2.4.2). (c) Breaking-up soil lumps with the hammer. (d) Bagging each Topsoil aliquot in white Kraft bag (Step (xii)). (e) Small, numbered card in the plastic zip-lock bag is placed on top of Topsoil sample in white Kraft bag (Step (xiii)), and (f) Sealing white Kraft bag by twisting the aluminium tin tie (Step (xiv)). Photographs: Alecos Demetriades (IGME/IUGS-CGGB).

| (a) | (b) |
| --- | --- |

Figure 3.2.13. Photographs showing placement of (a) Topsoil and (b) Bottomsoil sample bags in a sturdy plastic bag for storage and transportation (Step (xv) of sampling procedure – see Section §3.2.4.2). Photographs: Alecos Demetriades (IGME/IUGS-CGGB).

| (a) | (b) |
| --- | --- |

Figure 3.2.14. (a) Taking scintillometer readings at knee height (Step (xix) of sampling procedure − see Section §3.2.4.2), and (b) Recording field observations on the field observations sheet (Steps (xi, xix & xx)). Photographs: Alecos Demetriades (IGME/IUGS-CGGB).

### 3.2.4.6. Photographic documentation of humus sampling procedure

The following photographs show the humus sampling procedure.

| (a) | (b) |
| --- | --- |

Figure 3.2.15. Photographs showing numbered (a) Certified trace-element free plastic Rilsan® bag with a plastic strip lock for packing the humus sample (Step (i) of humus sampling procedure – see Section §3.2.4.3), and (b) Small card in a plastic zip-lock bag (Step (ii) of the sampling procedure for Humus, see Section §3.2.4.3). Photographs: Alecos Demetriades (IGME/IUGS-CGGB).

| (a) | (b) |
| --- | --- |
| (c) | (d) |

Figure 3.2.16. (a) Humus sampling in Finland using cylindrical sampler. (b) Removal of all mineral soil with a plastic scoop. (c) Pull apart Humus (on left) and roots (on right), and (d) Final Humus sample for bagging in either certified trace element free Rilsan® bag or white cotton bag free of chemical impregnation; when full the small-numbered card in a plastic zip-lock bag is placed on top, and the plastic bag is secured with the plastic strip lock or the cotton bag is securely tied with the drawstring. Photographs: Timo Tarvainen, Geological Survey of Finland (Salminen, Tarvainen et al., 1998, Figs. 8a to 8d, p.23-24).

## References

Note: All hyperlinks were checked on the 13th of February 2022.

Badía, D., Martí, C., Aznar, J.M., León, J., 2013. Influence of slope and parent rock on soil genesis and classification in semiarid mountainous environments. Geoderma, 193-194, 13–21; https://doi.org/10.1016/j.geoderma.2012.10.020.

Badía, D., Ruiz, A., Girona, A., Martí, C., Casanova, J., Ibarra, P. & Zufiaurre, R., 2016. The influence of elevation on soil properties and forest litter in the Siliceous Moncayo Massif, SW Europe. Journal of Mountain Science, 13, 2155–2169; https://doi.org/10.1007/s11629-015-3773-6.

Berkel, F., 1982. Geochemical Exploration In Arid And Semi-Arid Environments. Dissertation submitted for the Degree of Master of Science (Mineral Exploration), Rhodes University, South Africa, 159 pp.; https://core.ac.uk/download/pdf/145055433.pdf.

Blanchard, R., 1968. Interpretation of Leached Outcrops. Mackay School of Mines, University of Nevada, Nevada Bureau of Mines Bulletin 66, 196 pp.

Bridges, E.M., 1970. World Soils. Cambridge University Press, London, 89 pp.

Britannica, The Editors of Encyclopӕdia, 2012. Ultimate Reference Suite DVD. Encyclopӕdia Britannica, Inc., United Kingdom.

Britannica, The Editors of Encyclopӕdia, 2016. Cryosol. Encyclopӕdia Britannica, Inc., 6 Jun. 2016, https://www.britannica.com/science/Cryosol.

Buckman, H.O. & Brady, N.C., 1970. The nature and properties of soils. Macmillan Co., New York, 653 pp.

Büdel, J., 1982. Climatic Geomorphology (Trans. L. Fischer and D. Busch). Princeton University Press, Princeton, 443 pp.

Butt, C.R.M. & Zeegers, H. (Editors), 1992. Regolith Exploration Geochemistry in Tropical and Subtropical Terrains. Volume 4 In: G.J.S. Govett (Series Editor), Handbook of Exploration Geochemistry. Elsevier, Amsterdam, 607 pp.

Campbell, I.B. & Claridge, G.G.C., 1987. Physical Weathering and Rock Disintegration. Chapter 5 In: I.B. Campbell & G.G.C. Claridge (Editors), Antarctica: Soils, Weathering Processes and Environment. Developments in Soil Science, 16, 97–122; https://doi.org/10.1016/S0166-2481(08)70153-3.

Darnley, A.G., Björklund, A., Bølviken, B., Gustavsson, N., Koval, P.V., Plant, J.A., Steenfelt, A., Tauchid, M., Xuejing, Xie., Garrett, R.G. & Hall, G.E.M., 1995. A Global Geochemical Database for Environmental and Resource Management. Recommendations for International Geochemical Mapping – Final Report of IGCP Project 259. Earth Science Report 19. UNESCO Publishing, Paris, 122 pp; http://globalgeochemicalbaselines.eu.176-31-41-129.hs-servers.gr/datafiles/file/Blue_Book_GGD_IGCP259.pdf.

Dregne, H.E. (Editor), 1976. The Arid Regions. Chapter 6 In: Developments in Soil Science Book Series. Elsevier, 1-12; https://doi.org/10.1016/S0166-2481(08)70094-1.

Dokuchaev, V.V., 1883. Russian Chernozem: Report to the Free Economic Society. Decleron and Evdokimov, St. Petersburg, 376 pp. (In Russian; see concise description in English by Kyuma, K., 2018. Bibliographic Introduction and Explanatory Notes on “Russian Chernozem” published in 1883 by Vasilii V. Dokuchaev. Pedologist, 62(1), 3-10; https://doi.org/10.18920/pedologist.62.1_3).

FAO, 2006. Guidelines for soil description. Land and Water Division, Food and Agricultural Organization of the United Nations, Rome, Italy, 110 pp.; https://www.fao.org/3/a0541e/a0541e.pdf.

FAO/IIASA/ISRIC/ISS-CAS/JRC, 2012. Harmonized World Soil Database (Version 1.2). Food and Agriculture Organization of the United Nations (FAO), International Institute for Applied Systems Analysis (IIASA), ISRIC-World Soil Information, Institute of Soil Science – Chinese Academy of Sciences (ISSCAS), Joint Research Centre of the European Commission (JRC). FAO, Rome, Italy and IIASA, Laxenburg, Austria, 42 pp.; https://esdac.jrc.ec.europa.eu/ESDB_Archive/Soil_Data/Docs_GlobalData/Harmonized_World_Soi_Database_v1.2.pdf.

Flem, B., Torgersen, E., Englmaier, P., Andersson, M., Finne, T.E., Eggen, O. & Reimann, C., 2018. Response of soil C- and O-horizon and terrestrial moss samples to various lithological units and mineralization in southern Norway. Geochemistry: Exploration, Environment, Analysis (GEEA), 18(3), 252–262; https://doi.org/10.1144/geochem2017-087.

Fookes, P.G., 1991. Quaternary engineering geology. In: A. Forster, M.G. Culshaw, J.C. Cripps, J.A. Little, & C.F. Moon (Editors), Quaternary Engineering Geology. Geological Society, London, Engineering Geology Special Publications, 7, 73–98; https://doi.org/10.1144/GSL.ENG.1991.007.01.04.

Fookes, P.G., 1997. Geology for Engineers: The Geological Model, Prediction and Performance. Quarterly Journal of Engineering Geology, 30, 293–424; https://doi.org/10.1144/GSL.QJEG.1997.030.P4.02.

Giles, D., Martin, C. & Williams, R., 2017. Engineering geology of the Quaternary. Quarterly Journal of Engineering Geology and Hydrogeology, 50, 369–378; https://doi.org/10.1144/qjegh2017-104.

Haslinger, E., Reimann, C. & Birke, M., 2014. Short Guide to Soil Formation and the Soils of Europe.  Chapter 1 In: C. Reimann, M. Birke, A. Demetriades, P. Filzmoser & P. O’Connor (Editors), Chemistry of Europe's Agricultural Soils – Part B: General Background Information and Further Analysis of the GEMAS Data Set. Geologisches Jahrbuch (Reihe B103), Schweizerbarth, Hannover, 21–46; https://www.schweizerbart.de/publications/detail/isbn/9783510968473.

Hodgson, J.M. (Editor), 1976. Soil Survey Field Handbook – Describing and Sampling Soil Profiles. Soil Survey, Technical Monograph No. 5, Rothamsted Experimental Station, Harpenden, Herts, England, 99 pp.

Jenny, H., 1941. Factors of Soil Formation. A System of Quantitative Pedology. McGraw Hill Book Company, New York, NY, 281 pp.

Jones, A., Stolbovoy, V., Tarnocai, C., Broll, G., Spaargaren, O. & Montanarella, L. (Editors), 2009. Soil Atlas of the Northern Circumpolar Region. European Commission, Office for Official Publications of the European Communities, Luxembourg, 142 pp.; https://esdac.jrc.ec.europa.eu/Library/Maps/Circumpolar/Documents/Circumpolar_atlas.pdf.

McAuliffe, J.R., McFadden, L.D. & Timm Hoffman, M., 2018. Role of aeolian dust in shaping landscapes and soils of arid and semi-arid South Africa. Geosciences, 8, 171, 34 pp.; https://doi.org/10.3390/geosciences8050171.

Meigs, P., 1953. World distribution of arid and semi-arid homoclimates. In: Reviews of research on arid zone hydrology. Arid Zone Programme-1. United Nations Educational, Scientific, and Cultural Organization, Paris, 203-209.

Pédro, G., 1984. La genèse des argiles pédologiques. Ses implications minéralogiques, physico-chimiques et hydriques (Clay genesis in pedological conditions. Mineralogical, physico-chemical and hydric implications). Sciences Géologiques Bulletin, 37(4), 333–347; https://doi.org/10.3406/sgeol.1984.1677.

Petrov, M.P., 1976. Deserts of the World. John Wiley and Sons, New York, 447 pp.

Rafferty, J.P. (Editor), 2011. The Living Earth: Deserts and Steppes. Britannica Educational Publishing, 234 pp.

Reimann, C., Arnoldussen, A., Englmaier, P., Filzmoser, P., Finne, T.E., Garrett, R.G., Koller, F. & Øystein, N., 2007. Element concentrations and variations along a 120-km transect in southern Norway - Anthropogenic vs. geogenic vs. biogenic element sources and cycles. Applied Geochemistry, 22, 851–871; https://doi.org/10.1016/j.apgeochem.2006.12.019.

Reimann, C., Fabian, K., Schilling, J., Roberts, D. & Englmaier, P., 2015. A strong enrichment of potentially toxic elements (PTEs) in Nord-Trøndelag (central Norway) forest soil. Science of the Total Environment, 536, 130–141; https://doi.org/10.1016/j.scitotenv.2015.07.032.

Salminen, R., Tarvainen, T., Demetriades, A., Duris, M., Fordyce, F.M., Gregorauskiene, V., Kahelin, H., Kivisilla, J., Klaver, G., Klein, P., Larson, J.O., Lis, J., Locutura, J., Marsina, K., Mjartanova, H., Mouvet, C., O’Connor, P., Odor, L., Ottonello, G., Paukola, T., Plant, J.A., Reimann, C., Schermann, O., Siewers, U., Steenfelt, A., Van Der Sluys, J. & Williams, L., 1998. FOREGS Geochemical Mapping Field Manual. Geological Survey of Finland, Espoo, Guide 47, 36 pp.; http://tupa.gtk.fi/julkaisu/opas/op_047.pdf.

Salminen, R., Batista, M.J., Bidovec, M., Demetriades, A., De Vivo, B., De Vos, W., Duris, M., Gilucis, A., Gregorauskiene, V., Halamic, J., Heitzmann, P., Lima, A., Jordan, G., Klaver, G., Klein, P., Lis, J., Locutura, J., Marsina, K., Mazreku, A., O’Connor, P.J., Olsson, S.Å., Ottesen, R.T., Petersell, V., Plant, J.A., Reeder, S., Salpeteur, I., Sandström, H., Siewers, U., Steenfelt, A. & Tarvainen, T., 2005. FOREGS Geochemical Atlas of Europe, Part 1: Background Information, Methodology and Maps. Geological Survey of Finland, Espoo, 526 pp.; http://weppi.gtk.fi/publ/foregsatlas/.

Spaargaren, O.C. & Deckers, J.A, 2005. Factors of soil formation, Climate. Encyclopedia of Soils in the Environment. Academic Press, Oxford, 512–520.

Strahler, A.N., 1957. Quantitative analysis of watershed geomorphology. American Geophysical Union Transactions, 38(6), 912-920; https://doi.org/10.1029/TR038i006p00913.

Strahler, A.N., 1963. The Earth Sciences. Harper and Row, New York, 681 pp.

Strahler, A.N., 1969. Physical Geography. Wiley & Sons, Inc., New York and London, 733 pp.

Verheye, W. & De la Rosa, D., 2005. Mediterranean soils. In: Land Use and Land Cover, Encyclopedia of Life Support Systems (IOLSS), UNESCO, Eolss Publishers, Oxford, UK, 26 pp.; https://digital.csic.es/bitstream/10261/38282/1/Mediterranean%20soils.pdf.

Walker, A.S., 1992. Deserts: Geology and Resources. U.S. Department of the Interior/U.S. Geological Survey, U.S. Government Printing Office 0-305-515, 61 pp.; https://pubs.usgs.gov/gip/7000004/report.pdf.

Zdruli, P., Kapur, S. & Çelik, I., 2010. Soils of the Mediterranean region, their characteristics, management and sustainable use. In: S. Kapur, H. Eswaran & W. Blum (Editors), Sustainable Land Management. Springer, Berlin, Heidelberg, 128−142; https://doi.org/10.1007/978-3-642-14782-1_4.

Blank bank page

As the top soil is referred to as ‘Topsoil’ by the same token the bottom soil is referred to henceforth ‘Bottomsoil’.

An A humic soil horizon has ≥1% organic carbon in the fine earth fraction as a weighted average to a depth of 50 cm from the mineral soil surface (FAO/IIASA/ISRIC/ISS-CAS/JRC, 2012). However, to distinguish in the field the proportion of ≥1% soil organic carbon is difficult. So, a qualitative field classification of the A residual soil horizon as either non-humic or humic can be made by using colour, namely lighter or darker, respectively (see Annexe A3.2.2 of this Chapter). Hence, the field classification is crude, and after the total organic carbon is determined in the laboratory, the classification can be corrected.

In glacial and periglacial terrains, till and loess are considered to be the parent materials of residual soil.

It is noted that these terrain lines, as well the subsequent lines of the other regions, are without value because geographical boundaries are not keyed to the nature of the terrain. As no dividing line is completely definitive, a generally useful guide is, in the case of the Polar region, the irregular line marking the northernmost limit of the stands of trees.

https://www.nationalgeographic.org/encyclopedia/desert/

TUB-EX: http://www.tub-ex.com − sales@tub-ex.com

It is known that some cotton bag manufacturers impregnate the cotton cloth with some chemicals. As the chemicals are never mentioned, and the soil samples must not be contaminated with unknown chemicals, it is important to ensure that the white cotton bags are not impregnated with any chemicals.

The Kraft paper bags need to be constructed using waterproof glue, otherwise the bags will fall apart when wet.

If topographical maps are not available, orthophotographs or satellite photographs can be used.

Note: It is not necessary to take composite soil samples when a pit has been dug, as enough sample can be collected from the target horizon, provided the horizon is not too thin. In cases where the horizons are thin the pit is extended sideways in order to collect the required sample weight from the same Top and Bottom horizons.