International Union of Geological Sciences

Manual of Standard Methods

for

Establishing the Global Geochemical Reference Network

Chapter 3.3

Stream Water Sampling

Timo Tarvainen1,4, Tarja Hatakka1, Mikael Eklund1, Reijo Salminen1,

Christopher C. Johnson2,4, Alecos Demetriades3,4

1 Geological Survey of Finland, Espoo, Finland

2 GeoElementary, Derby, United Kingdom,

3 Institute of Geology and Mineral Exploration (retired), Athens, Hellenic Republic

4 IUGS Commission on Global Geochemical Baselines

2022

Published by

The International Union of Geological Sciences

Commission on Global Geochemical Baselines

It is recommended that reference to this part of the Manual should be made in the following way:

Tarvainen, T., Hatakka, T., Eklund, M., Salminen, R., Johnson, C.C. & Demetriades, A., 2022. Stream Water Sampling. Chapter 3.3 In: Demetriades, A., Johnson, C.C., Smith, D.B., Ladenberger, A., Adánez Sanjuan, P., Argyraki, A., Stouraiti, C., Caritat, P. de, Knights, K.V., Prieto Rincón, G. & Simubali, G.N. (Editors), International Union of Geological Sciences Manual of Standard Methods for Establishing the Global Geochemical Reference Network. IUGS Commission on Global Geochemical Baselines, Athens, Hellenic Republic, Special Publication, 2, 207−227.

Published by

The International Union of Geological Sciences

Commission on Global Geochemical Baselines

P.O. Box 640 37, Zografou

GR-157 10 Athens

Hellenic Republic

http://www.iugs.org/

http://www.globalgeochemicalbaselines.eu/

National Library of Greece Cataloguing in Publication Data

A catalogue record for this electronic book is available from the

National Library of Greece

ISBN: 978-618-85049-1-2

Contents

3.3.1. Introduction	211

3.3.2. Stream water sampling	211

3.3.2.1. Stream water samples to be collected	212

3.3.2.2. Identifiers of stream water samples	212

3.3.3. Equipment for stream water sampling	213

3.3.3.1. Equipment to be provided by the Project Coordinator	213

3.3.3.2. Equipment to be purchased by each participant country	213

3.3.4. Stream water sampling procedure	216

3.3.4.1. Photographs to be taken at each sampling site	220

3.3.5. Treatment of stream water samples	220

3.3.6. Storage and despatch of stream water samples to the laboratory	221

3.3.7. Anticipated problems at remote locations	221

3.3.8. Photographic documentation of stream water sampling procedure	222

3.3.9. Photographs to be taken at each stream/water sediment sampling site	226

References	226

Stream water samples are collected from second-order catchment basins

Blank back page

## 3.3.1. Introduction

Water is a precious natural resource, and dependence on clean water for human consumption, agriculture and animal husbandry requires that the reliability of data collected from water sampling campaigns is carefully monitored. According to objective 18.36 of Agenda 21 of the United Nations Programme of Action, as stated in the 1992 Earth Summit in Rio de Janeiro (UN, 1992): “The complex interconnectedness of freshwater systems demands that freshwater management be holistic (taking a catchment management approach) and based on a balanced consideration of the needs of people and the environment. The Mar del Plata Action Plan has already recognised the intrinsic linkage between water resource development projects and their significant physical, chemical, biological, health and socio-economic repercussions. …”.

In this manual, a catchment basin approach is used, as stipulated in objective 18.36 of Agenda 21, by the collection of stream water samples from second-order streams with a catchment basin of <100 km2 in area. The reason for selecting second-order streams is that the aim of this continental-scale survey, apart from the establishment of the Global Geochemical Reference Network, is to determine the geochemical baseline of the drainage system, and to define the geochemical background levels in stream water. Completely pristine stream water is difficult to find because of atmospheric transport of contaminants, and their subsequent deposition at locations distant from their source. Hence, streams that are normally least disturbed by human activities are selected for sampling (refer to Stream Sediment Sampling Chapter 3.4 for more details).

According to Simpson et al. (1993, p.63): “... bedrock geochemistry and mineralisation are the most important variables which influence the surface water geochemistry. The primary control by geological parameters is variously modified by secondary influences which include geomorphological factors (especially altitude) atmospheric (climatic and coastal effects), and anthropogenic (agriculture, urban and industrial developments).” This statement has been challenged by the FOREGS Geochemical Atlas of Europe project (Salminen et al., 2005; De Vos, Tarvainen et al., 2006).  In this continental-scale stream water sampling project, with an average density of 1 sample/4800 km2, “Most patterns appear to be controlled less directly by geology, but more by climate-related parameters such as rainfall, temperature, acidity and organic matter, in combination with topography and distance from the ocean, i.e., the source of precipitation” (De Vivo et al., 2006, p.33). The difference in the conclusions reached by Simpson et al. (1993) versus De Vivo et al. (2006) may be due to the average density of sampling, i.e., 1 sample/km2 and 1 sample/4800 km2, respectively.

In the compilation of these stream water sampling instructions, the main source is the FOREGS Geochemical Mapping Field Manual (Salminen, Tarvainen et al., 1998). However, other sampling manuals have been consulted (e.g., Bartram and Ballance, 1996; Chapman, 1996; US EPA, 1996, 2013; Gregorauskiene et al., 2000; Myumbilwa et al., 2007; Musselman, 2012; Danielson, 2014).

## 3.3.2. Stream water sampling

Flowing stream water is collected from a small, second-order, drainage basin (<100 km2; see Figs. 2.5 & 2.8 in Chapter 2) at the same site as active stream sediment (see Chapter 3.4 and Fig. 3.4.1). In semi-dry and dry terrains, such as southern Europe, North and South Africa, Namibia, parts of North, Central and South America, Australia, and other countries with dry late spring, summer and early autumn seasons, streams may not have flowing water if sampling is carried out during these periods, yet ponds with stagnant water may be found. No samples should be collected from such ponds, since the geochemistry of stagnant water is normally different from that of flowing stream water.

In true humid tropical and temperate climates, streams will rarely run dry and there is always an abundance of water. However, the wettest time of the year should be avoided in order not to sample during periods of high rainfall and flood events when the stream water is diluted with substantial surface water run-off. Ideally, stream water samples should not be collected within forty-eight hours of a rainfall event in the selected second-order catchment basin. If this is not possible, the time of the rainfall event should be recorded on the field observations sheet.

If the sampling campaign is to be carried out over several years, it is important to collect the stream water samples during the same climatic season. Other parameters to be considered are the transient character of stream water chemical composition, and that element concentrations are susceptible to various sources of temporal variation. Thus, the generated data are ‘snapshots’ of the chemical state of stream water at the time of sampling (Lark et al., 2016). These authors recommend setting up monitoring sites on selected streams, where daily stream water samples are collected over variable time intervals. The statistical analyses of water geochemical data from these monitoring sites can supplement the results from the analysis of routine stream water samples, collected in a once-off sampling campaign to predict temporal effects on water geochemistry.

Stream water, as stream sediment, is always sampled upstream from bridges, artificial channels (culverts, flumes), and settlements (refer to Stream Sediment Sampling Chapter 3.4 and Sections §3.4.1 and §3.4.4 for information about selecting a suitable sampling site on a second-order stream).

### 3.3.2.1. Stream water samples to be collected

The following stream water samples are collected from a second-order stream:

(a) Routine sample site of a normal GTN grid cell: - 1 x 500 ml bottle for unfiltered stream water for major IC ion analysis. - 1 x 100 ml bottle for filtered stream water for ICP-MS and ICP-AES analysis. - 1 x 100 ml bottle for mercury analysis, and - 1 x 60 ml bottle for Dissolved Organic Carbon (DOC) analysis. (b) Duplicate field site of a duplicate GTN grid cell (at least one in each country): - The same stream water samples, and field measurements, are taken at the duplicate field site (see below). Sampling quantities: See bottle sizes above. However, before starting the stream water sampling campaign, the Project Manager should check with the laboratory the required volume for analysis.

Duplicate stream water sample: Duplicate stream water samples are collected randomly at least at every 20th sampling site (i.e., ≈5% duplication of the sample sites) in each country. However, countries with less than 6 GTN grid cells should collect field duplicate stream water samples from at least one random site. The field duplicate stream water sample is collected from the same site less than five (<5) minutes after taking all field measurements (pH, electrical conductivity (EC), alkalinity).

### 3.3.2.2. Identifiers of stream water samples

The identifier of stream water samples is ‘W’:

(a) Routine sample site (e.g., GTN grid cell N26E14): - Stream water sample: N26E14W1 Note: Number ‘1’ represents the 1st sample site in GTN grid cell N26E14. (b) Duplicate field site (e.g., GTN grid cell N27E12): - Stream water sample: N27E12W3 - Stream water sample – Duplicate: N27E12W3D Note: Number ‘3’ represents the 3rd sample site in GTN grid cell N27E12, and ‘D’ denotes the duplicate stream water sample. (c) Blank water sample: The identifier for a filtered blank water sample is zero (0), i.e., GTN grid cell / W / sample no. / 0; for example, N27E12W30.

## 3.3.3. Equipment for stream water sampling

### 3.3.3.1. Equipment to be provided by the Project Coordinator

- 500 ml new certified trace element free Nalgene® bottles or comparable (for non-filtered water samples for major IC ion analysis).

- 100 ml new certified trace element free Nalgene® bottles or comparable (for ICP-MS analysis) for mercury analysis. It is proposed, however, to use instead new 100 ml borosilicate glass bottles for mercury analysis by ICP-MS, as these are recommended by US EPA (1996).

- 60 ml new certified trace element free Nalgene® bottles or comparable (for filtered, unacidified samples for Dissolved Organic Carbon (DOC) analysis.

- Disposable white-colour powder-free vinyl gloves (e.g., Medi-Point or Kingfisher vinyl gloves or comparable).

- Disposable 50 ml syringes (e.g., Becton & Dickinson Plastipak™, or HSW® Norm-Ject® or comparable).

- Disposable syringe filters 0.45 μm (e.g., Schleicher & Schuell pyrogen-free, or Corning® or Whatman® or comparable), and

- Droplet bottles made of Teflon FEP (fluorinated ethylene propylene).

- Black permanent drawing ink markers (ONLY black coloured allowed).

Whatever the decision, the same type of materials must be used by all countries throughout the Global Geochemical Reference Network project. The same applies to national stream water geochemical projects.

Important note: The volume of stream water samples needed for analysis should be confirmed with the laboratory before purchasing the bottles. This is an issue that must be examined by the Laboratory Committee..

### 3.3.3.2. Equipment to be purchased by each participant country

Each participating country must purchase the following equipment, and for all its sampling teams:

- pH-meter (e.g., WTW pH3310 or HACH® HQ11D or comparable).

- EC-meter (e.g., WTW Cond3310 or HACH® HQ14D or comparable).

- Buffer solutions for calibration of pH- and EC-meters, as specified by instrument manufacturers, i.e., pH buffer solution at pH 4, pH 7 and pH 10.

- Class 1 water at 18.2 MΩ/cm (<5 μg/l TOC), and a washing bottle.

- Class 1 water or sterile water for injection (purchased from pharmacy or drugstore) to be used as a field blank sample (Sterile water for injection is clear, colourless, odourless, sterile, hypotonic, nonpyrogenic, and contains no bacteriostatic or antimicrobial agents).

- Concentrated nitric acid (HNO3) 65%, density 1.40 kg/l (Merck Suprapur® 100441 or equivalent).

- Teflon dark bottle for storing potassium dichromate solution for the preservation of stream water sample taken for Hg analysis.

- Potassium dichromate solution for Hg preservation. Prepare in the laboratory: 0.2 grams of K2Cr2O7 (Pro analysis, PA, quality) / 100 ml nitric acid HNO3 (Merck Suprapur® quality).

- Plastic disposable pipettes of 1 ml capacity for acidifying stream water samples with concentrated nitric acid.

- Plastic disposable pipettes of 3 ml capacity for adding acidified potassium dichromate solution to stream water sample bottles for Hg preservation.

- 2 polyethylene decanters of 1 litre capacity for sample water to measure pH and EC.

- Plastic 100 ml measuring cylinder (for alkalinity measurements, methods A and B).

- 250 ml plastic conical flask (for alkalinity measurements, methods A and B).

- Plastic zip-lock bags (10 x 20 cm) for packing used water filters.

- Transparent waterproof adhesive tape for protecting the bottle labels, which are written with the black permanent drawing ink markers.

- Bailer device for water sample collection from deep and fast-flowing streams.

- Lint-free disposable wipes, such as those made by Kimtech (Kimwipe paper) or by Bounty (kitchen paper rolls), both do not leave fuzz or comparable lint-free paper.

- Car refrigerator or battery-powered cool box or cool boxes with ice packs.

- Strong carton or plastic boxes for packing samples for despatch to the laboratory.

- Insulating material boards to line the carton boxes to retain a low temperature until samples reach the laboratory.

- Paper rubbish bags.

- Global Positioning System (GPS) for recording sample site coordinates, or a tablet with a GPS and digital topographical maps, or a mobile telephone with GPS; it is strongly recommended to have a backup GPS receiver.

- Extra batteries for GPS or battery charger with rechargeable batteries.

- Topographical maps, preferred scale 1:50,000 (a must in case electronic digital positioning devices fail, e.g., GPS failure).

- A plastic roamer or map ruler for extracting coordinates from 1:50,000 topographical maps in case GPS fails.

- An orienteering compass (a must in case GPS fails).

- Digital camera or a mobile telephone with a macro facility for field documentation (minimum 5 megapixels).

- Extra batteries for the digital camera.

- Field-ruggedised notebook or laptop computer with extra charger and spare batteries.

- Car adapter for charging notebook or laptop computer.

- Portable storage device (USB memory stick or external hard drive) for backup of field data and digital photographs.

- USB cable to download photographs to a laptop computer daily.

- Threshold scintillometer to measure natural radioactivity (Total, Th, U, K); the scintillometer must be calibrated at least once a year before the field campaign at a national facility that is certified.

- Extra batteries for the threshold scintillometer.

- Field observations sheets.

- Waterproof case to hold field observation sheets.

- Writing pens.

- HB pencils and pencil sharpener (back-up in case the pens fail to write in the field).

- First-aid kit.

- Rope or wading pole as river conditions are not the safest, and

- Mobile telephone or other communication equipment like CB radios and satellite telephone (the latter may be needed in remote areas), or emergency position-indicating radio beacon (EPIRB).

If alkalinity is measured using method A (see below), the following equipment is required:

- HACH® Model 16900-01 digital titrator or equivalent with solution delivery straws.

- HACH® digital titration cartridges sulphuric acid 1.6 N.

- HACH® digital titration cartridges sulphuric acid 0.16 N, and

- Bromocresol green acid-base indicator solution to be prepared in the laboratory before starting the field sampling campaign: Accurately weigh 0.1 g of bromocresol green into a brown container flask. Add 14.3 ml of 0.01 M NaOH + 235.7 ml distilled and deionised water and swirl to dissolve (Lide, 1991). Note: HACH® provides ready-prepared sachets of bromocresol green, which can be added directly to the stream water sample for titration.

If alkalinity is measured using method B (see below), the following materials are required:

- 1.6 N H2SO4: Dilute 44.4 ml of concentrated H2SO4 (mass-% = 96; density = 1840 g/l) to 1000 ml with deionised water.

- 0.16 N H2SO4: Dilute 10 ml of 1.6 N H2SO4 to 100 ml with deionised water.

- Bromocresol green indicator solution: Accurately weigh 0.1 g of bromocresol green into a brown container flask. Add 14.3 ml of 0.01 M NaOH + 235.7 ml distilled and deionised water and swirl to dissolve (Lide, 1991).

- Volumetric flasks of 100 and 1000 ml capacity.

- Burette or equivalent equipment, 10 ml capacity, graduated in divisions of 0.02 ml, and

- Disposable Pasteur-pipettes.

Field observation sheets are included in this manual (see Appendix 1).

## 3.3.4. Stream water sampling procedure

It would be best if water sampling is carried out by persons who have practical experience in this procedure. Individuals, who are inexperienced in stream water sampling, should be trained for one to two weeks, and closely supervised for at least 10 days after the training programme to ensure that they follow the field sampling procedure, and safety precautions.

Calibration: The pH and electrical conductivity meters must be calibrated, according to the manufacturer’s instructions, at the start of each day.

Location of stream water sample: As the stream sediment sample is composited from 5 to 10 subsamples in the field (see Chapter 3.4), the stream water sample should be taken from the first, lowermost stream sediment sampling point.

CAUTIONS: - All hand jewellery must be removed. - Samplers must not use insect repellent before sampling even when wearing gloves. - All tools and containers must be free of contaminants. - Note that the samplers are not allowed to smoke, or have the vehicle running when stream water samples are taken. - Avoid sampling during rainy periods and flood events. - The stream water sample must be taken before the stream sediment sample, for obvious reasons, i.e., during the collection of the stream sediment, fine-grained material is agitated and transported in suspension.

Stream water samples at each sampling site should be taken according to the following procedure:

- Switch on the GPS unit, or notebook or laptop computer with GPS, to obtain the WGS1984 decimal degree coordinates of the sample site. It is important to switch on the global positioning system upon arrival at the site, and to allow enough time for the signal to settle.

- Write the sample number on the 60 ml, 100 ml and 500 ml bottles (Fig. 3.3.1); distinguish the stream water samples for determination of mercury (Hg – Fig. 3.3.1b) and dissolved organic carbon (DOC) by writing ‘Hg’ on 100 ml and ‘DOC’ on 60 ml sample bottles, respectively. The marked labels should be protected with transparent waterproof adhesive tape.

- Write sample number on field observations sheet, and coordinates of sample site as recorded by GPS unit.

- Mark sample site position on the 1:50,000 topographical map. Note: Even if you have a notebook with a GPS and digital topographical map, where the position is recorded and saved digitally, it is strongly recommended, as a safety precaution, to mark the sample site position on the paper topographical map.

- Important: Before collecting stream water samples, wear disposable powder-free gloves on both hands (Fig. 3.3.2), and keep them on until all stream water samples are collected and stored safely in a portable refrigerator. One pair of powder-free disposable gloves should be used at each sampling site.

Important caution: Do not touch anything except the stream water sample bottles upon wearing the powder-free disposable gloves. Avoid glove and sample bottle contact with the ground or with human skin or clothing, and avoid breathing directly in the bottle or cap during sample collection.

- Unfiltered stream water sample for major IC analysis: Take a 500 ml sample bottle and rinse it twice with stream water (Figs. 3.3.3a-b), i.e., fill up the bottle with 50 to 100 ml of stream water, close the bottle with a cap and shake it vigorously; remove the cap and decant stream water. Repeat this procedure a second time.

- submerge 500 ml sample bottle completely, with bottle mouth facing upstream.  Fill up the bottle up to its brim (i.e., completely full) with stream water so that no air bubbles are left in the bottle (Fig. 3.3.3c).  Close the bottle by screwing the cap tightly below stream water level (Fig. 3.3.3d).

Cautions: - If the stream is too shallow to immerse the bottle fully, submerge it at a low angle with the bottle mouth facing upstream, and with care not to touch the bottom sediment. If the stream is too shallow even for this procedure, then use the disposable syringe to fill up the bottle with stream water. - If the stream is too deep and fast flowing the water sample can be collected with a bailer device (Fig. 3.3.4). - This procedure can potentially agitate bottom sediment by stepping on a bank composed of loose material or loose rocks. So, extra care is necessary not to agitate bottom sediments during stream water sampling. It is, therefore, safer to use a bailer device for all occasions (Fig. 3.3.4), as the stream water samples can be collected by standing on the stream bank.

- Unfiltered stream water sample for Hg analysis: Take a marked 100 ml Hg sample bottle and rinse it with stream water twice, i.e., fill up the bottle with 10 to 20 ml of stream water, close the bottle with cap and shake it vigorously; remove cap and decant stream water. Repeat the procedure a second time.

- Fill up the 100 ml Hg sample bottle with stream water to almost its neck and close it by screwing the cap tightly. Note: Remember to leave room for the addition of 5 ml acidified potassium dichromate (see Section §3.3.6).

- Filtered stream water for ICP-MS and ICP-AES analysis: Filter one 100 ml stream water sample (Fig. 3.3.5) by using the following procedure:

- Rinse a disposable syringe two times with stream water, and each time dispose of the water either downstream from the sampling point or on land (Figs. 3.3.5a-b).

- Fill up disposable syringe with stream water (Fig. 3.3.5c).

- Place 0.45 μm filter on the disposable syringe (Fig. 3.3.5d).

- Discard the first 10 ml of filtered stream water from each new filter unit used (Fig. 3.3.5e).

- Take a 100 ml marked sample bottle for the acidified sample and rinse the bottle twice with filtered sample water, i.e., fill up the bottle with 10 to 20 ml of filtered stream water (Fig. 3.3.5f), close the bottle with the cap and shake it vigorously (Fig. 3.3.5g); remove the cap and decant filtered stream water (Fig. 3.3.5h); repeat the procedure a second time.

- Fill up 100 ml bottle up to its neck with filtered stream water (Fig. 3.3.5i; change the filter if needed), and close it tightly with screw cap (Fig. 3.3.5j). Care should be taken for the filtered stream water sample to go straight into the bottle without contact with your hands. Note: Remember to leave room for the addition of 1 ml of concentrated ultra-pure nitric acid (see Section §3.3.5).

- It is recommended to preserve the used filter unit in a labelled zip-lock bag for later analysis of the filter for suspended sediment chemistry. The suffix for used filter units is ‘F’, for example, N43E09W4F.

- Filtered stream water for DOC analysis: Take 60 ml marked sample bottle for DOC determination and rinse it twice with filtered stream water, i.e., fill up the bottle with 10 to 20 ml of filtered stream water, close the bottle with cap and shake it vigorously; remove cap and decant filtered stream water. Repeat the procedure a second time.

- Fill up the 60 ml DOC bottle up to its brim with filtered stream water (change the filter if needed) and close the bottle tightly with screw cap.

- Place the filter unit in the same labelled zip-lock bag that the filters of filtered stream water for ICP-MS and ICP-AES analysis are kept.

- Directly after sampling, the filled stream water sample bottles are placed into a cool box or car refrigerator (Fig. 3.3.6a).

- In the evening, store all stream water samples in the field camp or hotel refrigerator at a temperature of <4oC (Fig. 3.3.6b).

- Measurement of pH and Electrical conductivity: Mark with a permanent drawing ink marker two 1 litre polyethylene decanters, one for pH (write pH) and another for electrical conductivity (write EC). Rinse the two decanters twice with stream water, and then fill them up with stream water. Place the electrodes in water and measure the pH and electrical conductivity with calibrated meters.

- Record the pH (with 1 decimal figure) and EC value (mS/m) on the field observations sheet.

- Rinse the electrodes and decanters with distilled and deionised water and keep the electrodes in their measuring decanters until all daily sampling sites are visited.

- At the end of the day, place the meters in their cases.

- As already cautioned, the pH and Conductivity meters are calibrated every day before the first measurements are taken.

- Total alkalinity measurements: Alkalinity means the ability of water to neutralise acid. It is defined as the quantity of ions, such as CO32-, HCO3-, OH-, HSiO3-, H2BO3-, HPO42- and H2PO4-, in water that will neutralise hydrogen ions. However, in most waters, HCO3- is the dominant ion between pH 4.5 and 8.3. The alkalinity is often expressed as milligrams per litre of CaCO3. This means that the determined total amount of all ions neutralising acid is transformed to an equivalent concentration of CaCO3 by calculation. Many digital titrators and standard acid cartridges are already calibrated to read in mg/l CaCO3.

The determination of alkalinity is made by titrating a known amount of stream water with acid (e.g., H2SO4 or HCl) to pH 4.5. At this point, all the ions which neutralise acid are used and the colour of the indicator (e.g., phenolphthalein or bromocresol green) will change.

There are two methods to determine the alkalinity. Follow either method (A) or method (B) in the field.

Method A:  Using HACH® digital titrator and standard acid cartridges:

  - Select a sulphuric acid cartridge of 1.6 N or 0.16 N, according to the expected alkalinity of stream water samples. This can be judged from the pH and conductivity of the stream water. Specifically, use the 0.16 N cartridge if the conductivity is below 5 mS/m (50 μS/cm), and the 1.6 N cartridge if the conductivity is above this level.

  - Fit the cartridge to the hand-held digital titrator and push the titrator piston down until it meets the top of the cartridge.

  - Remove the cap from the cartridge and fit a feeder straw into the end of the cartridge.

  - Wind up the large wheel of the titrator until all air is removed from the cartridge and the straw, and a drop of acid leaves the end of the straw.

  - Wipe the end of the straw to remove excess acid with a lint-free paper towel.

  - Reset the titrator scale to zero by winding the small wheel to the left of the scale forwards.

  - Rinse the measuring cylinder and conical flask with distilled and deionised water, and directly afterwards rinse them with a small amount of stream water.

  - Using the measuring cylinder, measure 100 ml of stream water, and decant it into the conical flask.

  - Add a few (two) drops of bromocresol green indicator using a small pipette (or add the content of bromocresol green sachet).

  - Add the sulphuric acid using the large wheel on the titrator until the solution changes from blue to green-yellow, and note the reading on the titrator scale when this occurs.

  - When the 0.16 N cartridge is used the readings should be multiplied by 0.1.

  - The reading is the total alkalinity expressed as mg/l CaCO3.

  - Record the measured alkalinity on the field observations sheet.

  - The digital titrator should be reset to zero, and the conical flask and measuring cylinder rinsed with deionised water before leaving the sampling site.

  - Important Note: Titrations should be carried out as quickly as possible to reduce degassing effects.

Method B: Using normal burette and non-standard acid:

  - Transfer 100 ml of stream water to a 250 ml conical flask with a measuring cylinder.

  - Add two drops of bromocresol green indicator with a disposable Pasteur-pipette (or add the content of bromocresol green sachet).

  - Select a sulphuric acid cartridge either 1.6 N or 0.16 N, according to the expected alkalinity of the stream water sample.

  - Mix and titrate immediately.

  - Add the H2SO4 dropwise from a burette, while continuously stirring the stream water sample.

  - The end-point is reached when the colour has changed to yellow.

  - Write down the volume of sulphuric acid used on the field observations sheet.

  - Note: If sample water is very coloured and turbid the end-point is difficult to observe, and in such a case use a pH meter to determine the end-point. Titrate solution as above to pH 4.5.

  - After titration, wash the pH meter electrode with distilled and deionised water and calibrate the pH meter.

  - If 0.16 N H2SO4 is used for titration, the alkalinity of the sample, expressed as milligrams of CaCO3 per litre, is given by equation (1):

Alkalinity (mg/l CaCO3) = 80 * V0.16N			(1)

where V0.16N = the volume of 0.16 N H2SO4 (ml) used in the titration.

  - If 1.6 N H2SO4 is used for titration, the alkalinity of the sample, expressed as milligrams of CaCO3 per litre, is given by equation (2):

Alkalinity (mg/l CaCO3) = 800 * V1.6N			(2)

where V1.6N = the volume of 1.6 N H2SO4 (ml) used in the titration.

- Field water blank sample: At every 20th stream water sample (and at least once in every country) the field water blank sample is filtered:

- Filter distilled and deionised water in a 60 ml bottle in the same manner as the normal stream water sample (Sterile water for injection is recommended instead of distilled and deionised water).

- This sample is treated (acidified and handled) like the normal stream water samples.

- The suffix for a filtered field blank water sample is zero (0) and the full identifier is GTN grid cell / W / sample number / 0; for example, N43E09W40.

### 3.3.4.1. Photographs to be taken at each sampling site

At each stream sediment/water sample site at least three digital photographs should be taken (refer to Chapter 3.4, Section §3.4.4.4 and Figs. 3.4.20a-c).

## 3.3.5. Treatment of stream water samples

- In laboratory or comparable conditions, soon after sampling (at least on the same day) add to a 100 ml filtered stream water sample bottle 1.0 ml of ultrapure concentrated HNO3 acid with a disposable single-use 1 ml plastic pipette or droplet bottle.

- Close the bottle tightly and shake it to thoroughly mix the acid with the stream water sample.

- Mark the top of the bottle cap with a black cross as a clear sign that the sample has been acidified.

Important cautions: - Use disposable clean powder-free vinyl gloves and wear safety glasses, because the acid is very corrosive. - Do not let the droplet bottle or the tip of the pipette touch the sample of stream water in the bottle.

- Do not add acid to the 60 ml DOC stream water sample bottle!

- Add nitric acid and potassium dichromate to stream water samples in Hg sample bottles. A volume of 5 ml HNO3-K2Cr2O7 to each 100 ml stream water bottle is added with disposable single-use 3 ml pipettes.

- Close the bottle tightly and shake it thoroughly.

Caution: - Remember that this solution is extremely aggressive and extra care must be exercised. Safety glasses and clean powder-free vinyl gloves are essential.

## 3.3.6. Storage and despatch of stream water samples to the laboratory

- As noted above, place the stream water sample bottles in the field camp or hotel refrigerator at a temperature of <4oC (Fig. 3.3.6b).

- All stream water samples should be stored in the dark.

- Packed the stream water samples in strong carton boxes lined with insulating material boards (blue) to retain a low temperature until samples reach the laboratory (Fig. 3.3.7).

- Send stream water samples to the laboratory soon after sampling.

## 3.3.7. Anticipated problems at remote locations

Treatment of stream water samples with acidification and storage in a refrigerator can be problematic in remote locations. Planning of logistics should try and ensure acidification of water samples and storage in cooled containers to be done as soon as possible after collection. In the worst case, it may be possible to obtain ice cubes at petrol stations or even carry a large bottle of CO2 and make ice on the spot.

## 3.3.8. Photographic documentation of stream water sampling procedure

|  | Figure 3.3.1. Labelling bottles for stream water sampling by writing the sample number on each bottle with a black permanent ink marker, e.g., (a) Bottle for ICP-MS and ICP-AES analysis, and (b) bottle for Hg analysis; similarly, the DOC is marked. As the labels with permanent ink may be damaged during transport, it is recommended to be covered by transparent waterproof adhesive tape. Photographs: Alecos Demetriades (IGME/IUGS-CGGB). |
| --- | --- |
| (a) |  |
| (b) |  |
|  | Figure 3.3.2. Wearing disposable powder-free gloves on both hands before the start of stream water sampling. Photographs: Alecos Demetriades (IGME/IUGS-CGGB). |
| (a) |  |
| (b) |  |
|  | Figure 3.3.3. Sampling unfiltered stream water sample for major IC ion analysis: (a) Rinse 500 ml bottle with 50 to 100 ml of stream water, cap it and shake it, and (b) Decant the stream water downstream of the sampling point; repeat the rinsing procedure a second time. (c) Submerge a 500 ml sample bottle completely, with its mouth facing upstream, and fill it up to its brim (i.e., completely full) with stream water in such a way that no air bubbles are left in the bottle, and (d) Close the bottle by screwing the cap tightly below stream water level. Almost the same procedure is followed by rinsing and filling up with unfiltered stream water the 100 ml bottle for Hg analysis, except that in this case the bottle is filled up to its neck leaving space for the addition of 5 ml HNO3-K2Cr2O7 stabilising solution. Photographs: Alecos Demetriades (IGME/IUGS-CGGB). |
| (a) |  |
| (b) |  |
| (c) |  |
| (d) |  |
| (a) | (b) |
| (c) | (d) |

Figure 3.3.4. (a) Fast flowing stream, Britannia Creek, British Columbia, Canada. (b-d) Stream water sampling with a bailer by Ray Lett. Photographs: Alecos Demetriades (IGME/IUGS-CGGB).

| (a) | (b) |
| --- | --- |
| (c) | (d) |
| Figure 3.3.5. Continued on next page. |  |
| (e) | (f) |
| (g) | (h) |
| (i) | (j) |

Figure 3.3.5. Filtering stream water for ICP-MS and ICP-AES analysis (100 ml bottle). Rinse the disposable syringe two times with stream water: (a) Fill up the disposable syringe with stream water, and (b) Eject stream water downstream; repeat the rinsing procedure a second time. (c) Fill up the disposable syringe with stream water and (d) Add 0.45 μm filter. (e) Discard downstream the first 10 ml of filtered stream water from each new filter used; rinse the bottle two times with filtered water by (f) Adding 10 to 20 ml of filtered water in the bottle. (g) Close the bottle with cap and shake it rigorously, and (h) Decanting the filtered water downstream. (i) Fill-up the bottle with filtered stream water, taking care for the filtered stream water to go straight into the bottle without any contact with your hands, and (j) Close the bottle tightly with the screw cap. Note: The bottle is filled up to its neck and room is allowed for the addition of 1.0 ml ultrapure nitric acid. Almost the same procedure is used for the collection of the filtered stream water for DOC analysis, but this time the 60 ml bottle is filled up to its brim. Photographs: Alecos Demetriades (IGME/IUGS-CGGB).

| (a) | (b) |
| --- | --- |

Figure 3.3.6. In the field, the stream water samples are stored in (a) A car refrigerator, and in the evening are stored in (b) A refrigerator at a temperature <4oC. Photographs: Alecos Demetriades (IGME/IUGS-CGGB).

Figure 3.3.7. Samples of stream water are packed in a strong carton box with insulating material boards (blue) in order to retain a low temperature during the transportation of the samples to the laboratory. Note: Addition of dry ice or ice cubes may be necessary, especially under hot environmental conditions. Photograph: Alecos Demetriades (IGME/IUGS-CGGB).

## 3.3.9. Photographs to be taken at each stream/water sediment sampling site

Refer to Section §3.4.4.4. in Chapter 3.4 ‘Stream Sediment Sampling’ and Figure 3.4.20.

## References

Note: All hyperlinks were checked on the 19th of February 2022.

Bartram, J. & Ballance, R. (Editors), 1996. Water quality monitoring – A Practical guide to the design and implementation of freshwater quality studies and monitoring programmes. United Nations Environment Programme (UNEP), and the World Health Organization (WHO), Geneva, 348 pp.; https://apps.who.int/iris/bitstream/handle/10665/41851/0419217304_eng.pdf?sequence=1&isAllowed=y.

Chapman, D.V. (Editor), 1996. Water quality assessments:  a guide to the use of biota, sediments and water in environmental monitoring. World Health Organization (WHO), United Nations Educational, Scientific and Cultural Organization (UNESCO) & United Nations Environment Programme, E & FN Spon, London, 609 pp.; https://apps.who.int/iris/bitstream/handle/10665/41850/0419216006_eng.pdf?sequence=1&isAllowed=y.

Danielson, Th.J., 2014. Protocols for collecting water grab samples in rivers, streams, and freshwater wetlands. State of Maine, Department of Environmental Protection, Division of Environmental Assessment, Biological Monitoring Program, DEPLW0637A-2014, 4 pp.; https://www.maine.gov/dep/water/monitoring/biomonitoring/materials/sop_watergrab.pdf.

De Vivo, B., Ander, E.L., Bidovec, M., Lima, A., Pirc, S., Reeder, S., Siewers, U., Smith, B., Albanese, S., Batista, M., Bel-Ian, A., Birke, M., Breward, N., Demetriades, A., De Vos, W., Duris, M., Gravesen, P., Gregorauskiene, V., Halamic, J., Jordan, G., Lax, K., Locutura, J., O’Connor, P.J., Pasieczna, A., Slaninka, I. & Tarvainen, T., 2006. Distribution of elements in stream water. In: W. De Vos, T. Tarvainen (Chief Editors), R. Salminen, S. Reeder, B. De Vivo, A. Demetriades, S. Pirc, M.J. Batista, K. Marsina, R.T. Ottesen, P.J. O’Connor, M. Bidovec, A. Lima, U. Siewers, B. Smith, H. Taylor, R. Shaw, I. Salpeteur, V. Gregorauskiene, J. Halamic, I. Slaninka, K. Lax, P. Gravesen, M. Birke, N. Breward, E.L. Ander, G. Jordan, M. Duris, P. Klein, J. Locutura, A. Bel-lan, A. Pasieczna, J. Lis, A. Mazreku, A. Gilucis, P. Heitzmann, G. Klaver & V. Petersell, Geochemical Atlas of Europe.  Part 2 - Interpretation of Geochemical Maps, Additional Tables, Figures, Maps, and Related Publications. Geological Survey of Finland, Espoo, 33−35; http://weppi.gtk.fi/publ/foregsatlas/articles/water.pdf.

De Vos, W., Tarvainen, T., Salminen, R., Reeder, S., De Vivo, B., Demetriades, A., Pirc, S., Batista, M.J., Marsina, K., Ottesen, R.T., O’Connor, P.J., Bidovec, M., Lima, A., Siewers, U., Smith, B., Taylor, H., Shaw, R., Salpeteur, I., Gregorauskiene, V., Halamic, J., Slaninka, I., Lax, K., Gravesen, P., Birke, M., Breward, N., Ander, E.L., Jordan, G., Duris, M., Klein, P., Locutura, J., Bel-lan, A., Pasieczna, A., Lis, J., Mazreku, A., Gilucis, A., Heitzmann, P., Klaver, G. & Petersell, V., 2006. Geochemical Atlas of Europe. Part 2 - Interpretation of Geochemical Maps, Additional Tables, Figures, Maps, and Related Publications.  Geological Survey of Finland, Espoo, 692 pp.; http://weppi.gtk.fi/publ/foregsatlas/.

Gregorauskiene, V., Salminen, R., Reimann, C. & Chekushin, V., 2000. Field manual for Barents Ecogeochemistry project. Geological Survey of Finland, Report No. S/44/0000/2/2000, Espoo, 60 pp.; http://newprojects.gtk.fi/export/sites/projects/barents/images/BARENTS_Fieldmanual_full.pdf.

Lark, R.M., Bearcock, J.M. & Ander, E.L., 2016. How does temporal variation affect the value of stream water as a medium for regional geochemical survey? Journal of Geochemical Exploration, 169, 211−233; https://doi.org/10.1016/j.gexplo.2016.07.020.

Musselman, R., 2012. Sampling procedure for lake or stream surface water chemistry. United States Department of Agriculture, Forest Service, Rocky Mountain Research Station, Research Note RMRS-RN-49, 11 pp.; https://www.fs.fed.us/rm/pubs/rmrs_rn049.pdf.

Myumbilwa, Y., Salminen, R., Malisa, E., Kashabano, J., Petro, F.N., Momburi, P., 2007. Field manual for geochemical surveys project. Geological Survey of Tanzania, Dodoma, 33 pp.

Lide, D.R. (Ed.), 1991. CRC Handbook of chemistry and physics. 72nd edition, CRC Press Inc., Boston, 8−15.

Salminen, R., Tarvainen, T., Demetriades, A., Duris, M., Fordyce, F.M., Gregorauskiene, V., Kahelin, H., Kivisilla, J., Klaver, G., Klein, P., Larson, J.O., Lis, J., Locutura, J., Marsina, K., Mjartanova, H., Mouvet, C., O’Connor, P., Odor, L., Ottonello, G., Paukola, T., Plant, J.A., Reimann, C., Schermann, O., Siewers, U., Steenfelt, A., Van Der Sluys, J. & Williams, L., 1998. FOREGS Geochemical Mapping Field Manual. Geological Survey of Finland, Espoo, Guide 47, 36 pp.; http://tupa.gtk.fi/julkaisu/opas/op_047.pdf.

Salminen, R., Batista, M.J., Bidovec, M., Demetriades, A., De Vivo, B., De Vos, W., Duris, M., Gilucis, A., Gregorauskiene, V., Halamic, J., Heitzmann, P., Lima, A., Jordan, G., Klaver, G., Klein, P., Lis, J., Locutura, J., Marsina, K., Mazreku, A., O’Connor, P.J., Olsson, S.Å., Ottesen, R.T., Petersell, V., Plant, J.A., Reeder, S., Salpeteur, I., Sandström, H., Siewers, U., Steenfelt, A. & Tarvainen, T., 2005. FOREGS Geochemical Atlas of Europe, Part 1:  Background Information, Methodology and Maps. Geological Survey of Finland, Espoo, 526 pp.; http://weppi.gtk.fi/publ/foregsatlas/.

Simpson, P.R, Edmunds, W.M., Breward, N., Cook, J.M., Flight, D., Hall, G.E.M. and Lister, T.R., 1993. Geochemical mapping of stream water for environmental studies and mineral exploration in the UK. Journal of Geochemical Exploration, 49, 63−87; https://doi.org/10.1016/0375-6742(93)90039-O.

US EPA, 1996. Method 1669: Sampling ambient water for trace metals at EPA water quality criteria levels. U.S. Environmental Protection Agency, Office of Water Engineering and Analysis Division, Washington, D.C., 35 pp.; https://www.epa.gov/sites/production/files/2015-10/documents/method_1669_1996.pdf.

US EPA, 2013. Operating procedure:  Surface water sampling.  U.S. Environmental Protection Agency, Science and Ecosystem Support Division, Region 4, Athens, Georgia, SESDPROC-201-R3, 22 pp.; https://www.epa.gov/sites/production/files/2015-06/documents/Surfacewater-Sampling.pdf.

UN, 1992. Agenda 21. United Nations Conference on Environment & Development, Rio de Janeiro, Brazil, 3 to 14 June 1992, 351 pp.; https://sustainabledevelopment.un.org/content/documents/Agenda21.pdf.

Blank back page

It is recommended to search the market and find other products, such as the Oakton PCTSTestr™ 50, which can measure pH/Conductivity/Total Dissolved Solids/Salinity.

Mercury can be accumulated in distilled water.