International Union of Geological Sciences

Manual of Standard Methods

for

Establishing the Global Geochemical Reference Network

Chapter 3

Sampling Methods: Introduction

Alecos Demetriades1,5, Christopher C. Johnson2,5, David B. Smith5,

Timo Tarvainen3,5, Maria João Batista4,5

1 Institute of Geology and Mineral Exploration, Athens, Hellenic Republic

2  GeoElementary, Derby, United Kingdom

3 Geological Survey of Finland, Espoo, Finland

4  Laboratório Nacional de Energia e Geologia, Amadora, Portugal

5 IUGS Commission on Global Geochemical Baselines

2022

Published by

The International Union of Geological Sciences

Commission on Global Geochemical Baselines

It is recommended that reference to this part of the Manual should be made in the following way:

Demetriades, A., Johnson, C.C., Smith, D.B., Tarvainen, T. & Batista, M.J., 2022. Sampling Methods: Introduction. Chapter 3 In: Demetriades, A., Johnson, C.C., Smith, D.B., Ladenberger, A., Adánez Sanjuan, P., Argyraki, A., Stouraiti, C., Caritat, P. de, Knights, K.V., Prieto Rincón, G. & Simubali, G.N. (Editors), International Union of Geological Sciences Manual of Standard Methods for Establishing the Global Geochemical Reference Network. IUGS Commission on Global Geochemical Baselines, Athens, Hellenic Republic, Special Publication, 2, 61−70.

Published by

The International Union of Geological Sciences

Commission on Global Geochemical Baselines

P.O. Box 640 37, Zografou

GR-157 10 Athens

Hellenic Republic

http://www.iugs.org/

http://www.globalgeochemicalbaselines.eu/

National Library of Greece Cataloguing in Publication Data

A catalogue record for this electronic book is available from the

National Library of Greece

ISBN: 978-618-85049-1-2

Contents

3A. Introductory notes	65

3B. Equipment	65

3C. Preparation of equipment for sampling	66

3D. Sample number flip card system for photographs	67

3E. Plastic laminated scalebar for photographs	68

3F. Photographs to be taken	68

3G. Photograph identifiers	68

References	69

Blank back page

## 3A. Introductory notes

In Chapter 2, the Global Terrestrial Network (GTN) grid cells of 160x160 km, and the selection of sample sites in second- and third-order catchment basins, according to the stream magnitude order classification system by Strahler (1957, 1969), are described. Further, identifiers of the different sample types are given. It is strongly recommended to read Chapter 2 a few times because it is important to understand the philosophy of the GTN sampling design, the selection of sample sites, and the use of sample identifiers before proceeding to study this Chapter where the sampling procedure is described in detail.

Sampling is the most important and costly stage of any geochemical mapping project, and it is the most difficult to repeat if not carried out properly (Demetriades, 2014, 2021; Demetriades et al., 2018). Consequently, the applied geochemists involved in sampling must be well-trained in the collection of samples of:

- Rock,

- Residual soil (Top and Bottom),

- Humus (where present),

- Stream water (where present),

- Stream sediment, and

- Overbank sediment (Top and Bottom)

from second-order catchment basins with an area of <100 km2, and

- Floodplain sediment (Top and Bottom)

from third-order catchment basins with an area between 1000 and 6000 km2.

It is strongly stressed that untrained personnel must NOT be allowed to go out in the field for sampling, if not accompanied and supervised by an experienced field applied geochemist until they become proficient.

The stepwise sampling instructions are designed for novices but also experienced field applied geochemists for teaching purposes. They must be followed precisely because the aim of the project for ‘Establishing the Global Geochemical Reference Network’ is to collect all sample types from all countries with a standardised methodology. Further, the wide-spaced sampling density of 1 site/5120 km2 in GTN grid cells of 160x160 km is exceedingly demanding because all collected samples will constitute the Standardised Global Geochemical Reference Network for each sample type.

The sampling of residual soil is the most difficult, even for experienced field applied geochemists. Therefore, it is strongly recommended that all national sampling teams must be trained by an experienced soil scientist for the recognition of soil horizons.

The sampling methods described in this Chapter can be applied also to regional geochemical surveys by changing the sampling density according to the area of the country and available funds.

## 3B. Equipment

In the following Chapters, describing the standardised methodology for sampling Rock, Residual soil, Humus, Stream water, Stream sediment, Overbank sediment and Floodplain sediment, there are two lists of equipment:

- Equipment to be provided by the Project Coordinator, and

- Equipment to be purchased by each participating country.

The first list of equipment to be provided by the Project Coordinator is important because it secures the standardised methodology and, therefore, needs to be purchased centrally and distributed to all participating countries. Thus, each national coordinator should inform the Project Coordinator of the number of sampling teams that will be involved in the sampling campaigns in each country.

The second list of equipment is, of course, just as important and could easily be purchased in each country. In this case, the national coordinator should ensure that all sampling teams in the country have the same equipment.

Aluminium and Brass equipment and Coloured plastic scoops and buckets must be avoided

## 3C. Preparation of equipment for sampling

Most equipment that will be purchased by each participating country is ready for use. However, some equipment may require attention. New spades and mattock cutters are usually painted, and the wooden handle may be varnished or painted (Fig. 3.1a). Previously used spades and mattock cutters, such as those shown in Figure 3.1c, may be rusted and, in this case, the rust must be removed (Fig. 3.1d). Similarly, new sledgehammers and chisels are painted, and used geological hammers may be slightly dirty-rusted (Fig. 3.1e). The paint and rust must be removed before the equipment is used for sampling, and this can be achieved quite easily by sandblasting (Figs. 3.1b, d & f). The varnish and paint on wooden handles can be removed again by sandblasting.

| (a) | (b) |
| --- | --- |
| (c) | (d) |
| (e) | (f) |

Figure 3.1. (a) Painted spade and mattock cutter. (b) Paint removed by sandblasting. (c) Rusted spade and mattock cutter. (d) Rust removed by sandblasting. (e) Painted 2 and 5 kg sledgehammers and chisel, and slightly dirty-rusted geological hammers. (f) Paint and dirt-rust removed by sandblasting. Photographs by Alecos Demetriades, Hellenic Institute of Geology and Mineral Exploration (IGME) & IUGS Commission on Global Geochemical Baselines (IUGS-CGGB).

## 3D. Sample number flip card system for photographs

It is recommended that each country makes a sample number flip card system, according to the GTN number code used in the Global Geochemical Reference Network project. This will be used to set up the sample site number, and sample type for taking the first photograph at each site. Figure 3.2 shows the sample number flip card system for a country situated in the northern (N) hemisphere and to the east (E) of the Greenwich meridian. Countries that are in the southern hemisphere and to the east of the Greenwich meridian should make a flip card system with ‘S’ and ‘E’, respectively. Accordingly, countries that are to the west of the Greenwich meridian should exchange the ‘E’ with ‘W’. Countries that happen to be on both sides of the equator and the Greenwich meridian should have both ‘N’ and ‘S, and ‘E’ and ‘W’, respectively.

Figure 3.2 shows the photographs with the site identification codes together with the letters and numbers that are necessary for making the sample number flip card system.

The letters and numbers should be printed on a thick paper or a thin card, cut into rectangles, then plasticised, perforated and mounted in a plastic or plasticised metal ring binder. The latter is preferred, if available.

It is strongly recommended that each national sampling team should have, as a safety precaution, at least two sample number flip card systems during the field sampling campaign.

| Site photograph identification code |  |  |  |  |  |  |  |  |  |  | Explanation of the first photograph at each site |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  | Identification code (ID) of a rock (R) sample site photograph. |  |  |
|  |  |  |  |  |  |  |  |  |  |  | Identification code of a field duplicate (D) rock sample site photograph. This is a special case for all sample types if the duplicate field site is very different from the routine sample site. |  |  |
|  |  |  |  |  |  |  |  |  |  |  | Identification code of a residual soil sample site photograph. The ID code of the Topsoil (T) sample is used. |  |  |
|  |  |  |  |  |  |  |  |  |  |  | Identification code of a humus (H) sample site photograph. This is for special cases, as normally the humus horizon is included in the residual soil photograph. |  |  |
|  |  |  |  |  |  |  |  |  |  |  | Identification code of a Stream water/Stream sediment sample site photograph, as these two samples are collected from the same site, the Stream water ID is used. |  |  |
|  |  |  |  |  |  |  |  |  |  |  | Identification code of an overbank sediment sample site photograph. The ID code of the top (K) overbank sediment sample is used. |  |  |
|  |  |  |  |  |  |  |  |  |  |  | Identification code of a floodplain sediment sample site photograph. The ID code of the top (F) floodplain sediment sample is used. |  |  |
|  | N | 0 | 0 | E | 0 | 0 | R | 1 |  |  |  | R = Rock |  |
|  | S | 1 | 1 | W | 1 | 1 | T | 2 | D |  |  | T = Residual soil (Top) |  |
|  |  | 2 | 2 |  | 2 | 2 | H | 3 |  |  |  | H = Humus |  |
|  |  | 3 | 3 |  | 3 | 3 | W | 4 |  |  |  | W = Stream water/Stream sediment |  |
|  |  | 4 | 4 |  | 4 | 4 | K | 5 |  |  |  | K = Overbank sediment (Top) |  |
|  |  | 5 | 5 |  | 5 | 5 | F |  |  |  |  | F = Floodplain sediment (Top) |  |
|  |  | 6 | 6 |  | 6 | 6 |  |  |  |  |  |  |  |
|  |  | 7 | 7 |  | 7 | 7 |  |  |  |  |  |  |  |
|  |  | 8 | 8 |  | 8 | 8 |  |  |  |  |  |  |  |
|  |  | 9 | 9 |  | 9 | 9 |  |  |  |  |  |  |  |

Figure 3.2. A sample number flip-card system is used to set up the sample number for taking the first photograph at each sample site. For an explanation see the text. Photographs by Alecos Demetriades (IGME/IUGS-CGGB).

## 3E. Plastic laminated scalebar for photographs

It is recommended to make a plastic laminated 12-cm scalebar to be used as a scale for photographs in cases where the two-metre measure is not displayed. Figure 3.3 shows such a scalebar.

Figure 3.3. Scalebar for photographs: 12-cm scalebar with the logo and name of the IUGS Commission on Global Geochemical Baselines to be printed on both sides of a card and plasticised for protection. Designed by Alecos Demetriades (IGME/IUGS-CGGB) with Microsoft™ PowerPoint.

## 3F. Photographs to be taken

The minimum number of digital photographs, which should be taken at each rock, residual soil, stream water, stream sediment, overbank sediment and floodplain sediment sampling site, is given in each Chapter that describes the sampling procedure.

## 3G. Photograph identifiers

Table 3.1 provides the photograph codes that should be used for the digital photograph archive.

Table 3.1. Photograph identifier codes. Random site 3 of GTN grid cell N26E14 is used as an example (see Fig. 2.8 in Chapter 2). Notation: The star (*) refers to the random GTN point number 1, 2, 3, 4 or 5.

| Catchment basin | Sample type photograph |  | Photograph code |  | Example |
| --- | --- | --- | --- | --- | --- |
| Second-order (<100 km2) | Rock: |  | R |  |  |
|  |  | Sample number |  | R*N | N26E14R3N |
|  |  | Landscape photograph |  | R*L | N26E14R3L |
|  |  | Site photograph (natural light) |  | R*S | N26E14R3S |
|  |  | Site photograph (fill-in flash) |  | R*F | N26E14R3F |
|  |  | Close-up |  | R*C | N26E14R3C |
|  | Residual soil: |  | T |  |  |
|  |  | Sample number |  | T*N | N26E14T3N |
|  |  | Landscape photograph |  | T*L | N26E14T3L |
|  |  | Ground surface |  | T*G | N26E14T3G |
|  |  | Site photograph (natural light) |  | T*S | N26E14T3S |
|  |  | Site photograph (fill-in flash) |  | T*F | N26E14T3F |
|  |  | Close-up |  | T*C | N26E14T3C |
|  |  | Photograph of the fill-up pit (a must at all sites) |  | T*P | N26E14T3P |
|  | Humus (where present): |  | H |  |  |
|  |  | Site photograph (natural light) |  | H*S | N26E14H3S |
|  |  | Site photograph (fill-in flash) |  | H*F | N26E14H3F |
|  |  | Close-up |  | H*C | N26E14H3C |
|  | Stream water/Stream sediment: |  | W |  |  |
|  |  | Sample number |  | W*N | N26E14W3N |
|  |  | Landscape photograph |  | W*L | N26E14W3L |
|  |  | Site photograph (natural light) |  | W*S | N26E14W3S |
|  |  | Site photograph (fill-in flash) |  | W*F | N26E14W3F |
|  |  | Close-up |  | W*C | N26E14W3C |
|  | Overbank sediment: |  | K |  |  |
|  |  | Sample number |  | K*N | N26E14K3N |
|  |  | Landscape photograph |  | K*L | N26E14K3L |
|  |  | Site photograph (natural light) |  | K*S | N26E14K3S |
|  |  | Site photograph (fill-in flash) |  | K*F | N26E14K3F |
|  |  | Close-up |  | K*C | N26E14K3C |
|  |  | If a pit is dug, take a photograph of the fill-up pit |  | K*P | N26E14K3P |
| Third-order (1000-6000 km2) | Floodplain sediment: |  | F |  |  |
|  |  | Sample number |  | F*N | N26E14F3N |
|  |  | Landscape photograph |  | F*L | N26E14F3L |
|  |  | Site photograph (natural light) |  | F*S | N26E14F3S |
|  |  | Site photograph (fill-in flash) |  | F*F | N26E14F3F |
|  |  | Close-up |  | F*C | N26E14F3C |
|  |  | If a pit is dug, take a photograph of the fill-up pit |  | F*P | N26E14F3P |

## References

Note: All hyperlinks were checked on the 12th of February 2022.

Demetriades, A., 2014. Basic considerations: Sampling, the Key for a Successful Applied Geochemical Survey for Mineral Exploration and Environmental Purposes. Chapter 15.1 In: W.F. McDonough (volume Editor), Analytical Geochemistry/Inorganic Instrument Analysis. In: H.D. Holland & K.K. Turekian (Executive Editors), Treatise on Geochemistry. Elsevier, Oxford, vol. 15, 1−31; https://doi.org/10.1016/B978-0-08-095975-7.01401-7.

Demetriades, A., 2021. Geochemical Mapping. In: D. Alderton & S.A. Elias (Editors), Encyclopedia of Geology, 2nd edition. Academic Press, United Kingdom, 6, 267−280; https://doi.org/10.1016/B978-0-08-102908-4.00059-X.

Demetriades, A., Smith, D.B. & Wang, X., 2018. General concepts of geochemical mapping at global, regional, and local scales for mineral exploration and environmental purposes. In: O.A.B. Licht (Guest Editor), Geochemical Mapping. Special Issue, Geochimica Brasiliensis, 32(2), 136−179; https://doi.org/10.21715/GB2358-2812.2018322136.

Strahler, A.N., 1957. Quantitative analysis of watershed geomorphology. American Geophysical Union Transactions, 38(6), 912−920; https://doi.org/10.1029/TR038i006p00913.

Strahler, A.N., 1969. Physical Geography. Wiley & Sons, Inc., New York and London, 733 pp.