International Union of Geological Sciences

Manual of Standard Methods

for

Establishing the Global Geochemical Reference Network

Chapter 8

Data Conditioning Methods:

Generating Time Independent Geochemical Data

Christopher C. Johnson1, T. Robert Lister2

1 GeoElementary, Derby, United Kingdom

2 British Geological Survey, Keyworth, Nottingham, United Kingdom

2022

Published by

The International Union of Geological Sciences

Commission on Global Geochemical Baselines

It is recommended that reference to this part of the Manual should be made in the following way:

Johnson, C.C. & Lister, T.R., 2022. Data Conditioning Methods: Generating Time Independent Geochemical Data. Chapter 8 In: Demetriades, A., Johnson, C.C., Smith, D.B., Ladenberger, A., Adánez Sanjuan, P., Argyraki, A., Stouraiti, C., Caritat, P. de, Knights, K.V., Prieto Rincón, G. & Simubali, G.N. (Editors), International Union of Geological Sciences Manual of Standard Methods for Establishing the Global Geochemical Reference Network. IUGS Commission on Global Geochemical Baselines, Athens, Hellas, Special Publication, 2, 429−456.

Published by

The International Union of Geological Sciences

Commission on Global Geochemical Baselines

P.O. Box 640 37, Zografou

GR-157 10 Athens

Hellenic Republic

http://www.iugs.org/

http://www.globalgeochemicalbaselines.eu/

National Library of Greece Cataloguing in Publication Data

A catalogue record for this electronic book is available from the

National Library of Greece

ISBN: 978-618-85049-1-2

Contents

8.1. Introduction	433

8.2. Data levelling	434

8.2.1. Preparation of geochemical data prior to testing whether levelling is required	434

8.2.2. Testing to see whether an analytical batch requires levelling	435

8.2.2.1. Parametric levelling	436

8.3. Worked examples	439

8.3.1. Worked Example 1: Levelling stream sediment results for K2O from the G−BASE central  and eastern England area	439

8.3.2. Worked Example 2: Levelling stream sediment results for lanthanum (La) from northern Scotland determined by DC-OES and ICP-MS	445

8.4. Other strategies required to future-proof data over long periods	449

8.4.1. Improved lower detection limit	449

8.4.2. Monitoring changes to the geochemical baseline over long periods	452

8.5. Recommendations	452

Supplementary material	454

References	454

Blank back page

## 8.1. Introduction

A geochemical project can generate data sets of varying sizes whether it be the results of twenty samples from an undergraduate university thesis or a national mapping programme completing chemical analyses of tens of thousands of samples. The former project may take a couple of weeks and the latter could span many decades. If data from a single long-duration programme or multiple smaller projects carried out over a long period are to be compared, or used to produce seamless geochemical maps, then strategies need to be adopted to mitigate any changes that may occur to the methodology over a long time. Figure 8.1 from the original Geochemical Atlas of Alaska (Weaver et al., 1983) illustrates the problem of combining disparate data sets. The prominent geochemical features visible relate to map sheet boundaries rather than any real natural geochemical variation. The results from the different map sheets need to be levelled such that a seamless geochemical map can be produced, irrespective of field campaign boundaries.

Figure 8.1. Cobalt distribution map from the original Geochemical Atlas of Alaska (Weaver et al., 1983) modified by Smith et al. (2013, Fig. 4, p.172). Large blocks of unlevelled data, identifiable by their straight-edge boundaries (map sheet boundaries), show the effects of uncorrected bias in the analytical results.

This chapter discusses the strategies and methods available to make existing geochemical data sets more compatible and recommends how we should future proof our geochemical data. The primary method described is that of parametric data levelling with real examples from the British Geological Survey’s (BGS) G-BASE project (Johnson et al., 2005; Everett et al., 2019).

Before the advent of global recommendations to standardise methods of collecting and analysing geochemical samples (Darnley et al., 1995), it was not easy to combine or compare the geochemical results generated by different projects and organisations at local through to global scales. Better coordination of procedures has meant that it is now easier to compare directly regional geochemical data sets from around the globe (e.g., Reimann et al., 2012). Hence, useful and meaningful variations in the distribution of chemical elements can be described and discussed with confidence. A particularly important part of the recommendations is the use of control samples in the chemical analysis. The primary and secondary reference materials (abbreviated to PRMs and SRMs and described in Chapter 5 of this Manual) can be used to level chemical results generated over a long time. It is necessary that the reference samples have been regularly submitted for analysis over the period the project lasts, and that the reference materials cover the range of concentrations for the elements being determined.

## 8.2. Data levelling

All the procedures applied to geochemical results after receipt from the analytical laboratory to make them fit for the purpose of their intended use are collectively referred to as data conditioning (Johnson et al., 2018). The process of adjusting a set of results to combine it with another existing set to give a single seamless geochemical map is called data levelling. It should be noted that data conditioning includes procedures to assess whether data levelling is needed. Levelling is akin to data normalisation in which results, measured under different conditions, are adjusted to a notionally common scale or references. The more general term ‘levelling’ is preferred here as the process can be required whether or not the conditions of analysis are the same (see also Annexes A2.1 & A2.2 in this Manual).

It must be emphasised that, if the standard methods for sampling and analysis, as described in this Manual are used (see Section §8.2.1 below), the process required is initially one of inspection to see whether the results need levelling. It is only after exceptional circumstances that the results will need to be levelled.

Such exceptional circumstances may arise if there is a change during a project that spans many years. For example, the analytical method may change (including subtle changes to the extraction procedure) or the analytical instrument used for the analyses undergoes significant modification, like a replacement of an X-ray fluorescence tube. Upon the completion of a project that has spanned many years or even decades, a final levelling process to ensure the data fits with other international data sets may also be required. The high-resolution baseline geochemical mapping of the United Kingdom (UK) using stream sediment samples (the G-BASE project) spanned more than forty years during which time analytical methods improved and were changed. This has required several periods of levelling that are summarised in Table 8.1. The final phase of levelling was to ensure the UK stream sediment geochemical atlas (Everett et al., 2019) is compatible with other national stream sediment data sets from around the globe.

### 8.2.1. Preparation of geochemical data prior to testing whether levelling is required

Before establishing whether a data set needs to be levelled relative to another, certain conditions need to be satisfied regarding the results:

- High-quality geochemical results have been received from the chemical laboratory and have been accepted as satisfactory. This is achieved by ensuring the analytical methodology (as described in Chapter 6 of this Manual) has been implemented, particularly concerning the inclusion of reference control samples (see Chapter 7 of this Manual).

- The control sample data are available as a supplement to the sample results and should also be stored in the geochemical database. All too often control samples are omitted from geochemical data sets, so that users do not misuse the control data as ‘real samples’. But this omission makes it more difficult to undertake any levelling of the results many years in the future by those not involved in the original project.

Important note 1: Therefore, an important prerequisite, during the design of a project geochemical database, is that the control samples must be an integral part of the database.

- Note that it should be the laboratory’s responsibility to ensure the accuracy of results and that any observed analytical bias is removed before it is accepted by the applied geochemist. This can be verified by repeated analyses of primary reference standards and internal laboratory procedures to correct for analytical bias. The results received from a specific laboratory should be seen, therefore, as consistent and initially requiring no further levelling.

It will be the applied geochemist’s responsibility to routinely carry out checking of the results for any analytical bias. This is achieved by inserting a range of SRMs that are ‘blind’ to the analyst to provide results that can be used in the parametric levelling procedure described below. Sufficient quantities of PRMs and SRMs need to be available to service long contracts with an analytical laboratory or laboratories as the analytical requirements (that is, analytical method and laboratory) are likely to change over a long period of many decades.

Table 8.1. A summary of the levelling reference points for the British Geological Survey’s G-BASE stream sediment data.

| Reference Point | Date of first levelling | Method |
| --- | --- | --- |
| North Scotland DC-OES | 1970s | Parametric levelling using internal reference materials relative to the first batch of results. |
| Wales XRFS | 1993 | All samples previously analysed by DC-OES levelled to the new XRFS method, using parametric levelling of internal reference materials repeatedly analysed by both methods. Some complete sample batches were also analysed by both methods to confirm the suitability of levelling. |
| United Kingdom geochemical atlas | 2015 | All stream sediment results were levelled to an international reference point using international reference materials. |

### 8.2.2. Testing to see whether an analytical batch requires levelling

Testing to see whether a newly received batch of analytical results requires any levelling can be done by two methods:

- Plotting spatial data to generate geochemical images to inspect whether any artefacts can be seen between the boundaries of analytical batches, and

- Plotting X-Y graphs for each element for three or more reference standards against their accepted reference values to determine any deviation from a straight line of gradient 1. This is referred to as parametric data levelling.

The first method is most applicable if the new analytical batch is spatially adjacent to existing results. Generally, the process of data levelling will be investigated when complete map sheets or large regions have been sampled and analysed. It is when complete map sheet areas are plotted together that any problems arising between map sheet boundaries can be tested (see Figs. 8.1 & 8.2).

| (a) | (b) |
| --- | --- |

Figure 8.2. (a) A map of the United Kingdom showing the different G-BASE sampling campaigns from 1969 –2013. Each colour represents a different year of sampling, which progressed from the north of Scotland to the south of England. This figure shows how most field campaigns were carried out within map sheet boundaries. (b) A map of chromium (Cr) in stream sediments (levelled data) showing no analytical bias associated with any of the map sheet boundaries or field campaigns.

#### 8.2.2.1. Parametric levelling

The method of X-Y plots is referred to as parametric, which is a term in statistics that refers to the fact that independent variables (X and Y) can be defined by an equation. The simplest and most common of these is the parametric linear equation:

Y = B*X + A

where ‘B’ is the gradient of the line, and ‘A’ is the intercept on the Y-axis at X equals zero concentration (Fig. 8.3).

Levelling adjusts the results to a common level, which in the subsequent real examples is the accepted element value for a secondary reference material (plotted as ‘Y’ on the X-Y plot). The two data sets being levelled can be referred to as disparate because, as a consequence of changes in methodology (planned or otherwise), the results contain an analytical bias that needs to be removed. The parametric linear equation is the simplest model to work with, and it may be necessary to logarithmically transform the data prior to levelling, and this can be determined from an initial X-Y plot. A good guide is, if when plotted without a logarithmic transformation the data ‘fan out’ at higher levels, then the data should be logarithmically transformed for reasons related to the homogeneity of variance and the subsequent numerical steps (Darnley et al., 1995; see also Annexe A2.2 in this Manual).

Figure 8.3. Parametric linear equation Y = B*X + A shown graphically. Note that A is the intercept on the Y-axis at X0 = zero. The green line indicates the 45-degree (1:1) diagonal. Plotted with Golden Software’s GrapherTM v20.

Firstly, the X-Y plots of the SRMs from the analytical batches to be levelled are plotted.  Where ‘X’ is the measured value and ‘Y’ is the SRM accepted reference value, determined by taking the mean value of many earlier analyses (refer to Chapter 5 in this Manual; Heydorn, 2006). A selection of SRMs showing a good range of elemental values is required for each plot and each element determined has to be processed individually. This means a large number of plots are required, so it is useful to automate the process using graphical plotting software for the X-Y plots and regression analysis to determine ‘B’ and ‘A’. This can be done in MicrosoftTM (MS) Excel, but other software packages are available which can offer greater automated customisation and consistency, such as R–scripts (Reimann et al., 2008). The more SRMs that are available the better the fit of the modelling.

Figure 8.4 shows various scenarios for how the X-Y plot may look. The situation shown in Figure 8.4a has points plotting on or close to the line of gradient 1 (i.e., value of B = 1), and the regression line passes through the origin of the graph (i.e., value of A = 0), referred to subsequently here as the reference line. In this instance, no levelling of the data is required.  Figure 8.4f shows the scenario that may arise where no correlation - linear or non-linear − can be seen between the data sets to be levelled. In this instance, levelling using this procedure must be considered impossible. In cases of non-linearity some form of linearisation, polynomial or non-linear curve fitting might be considered but modelling of this complexity may bring into question the credibility of the modified results.

A generally straight line, parallel to the reference line, indicates a constant positive or negative shift in the results to be levelled (as shown in Figs. 8.4b & 8.4c). If the slope of the regression line is greater or lesser than that of the reference line combined with a positive or negative shift (scenarios shown in Figs. 8.4d & 8.4e), then a correction factor (determined by ‘B’) and constant (subtraction or addition determined by ‘A’) will be required to level the element concentration ‘X’.

Figure 8.4. Various scenarios are shown by X-Y plots that may occur during parametric levelling. For descriptions of (a) to (f) see text. The green line indicates the 45-degree (1:1) diagonal. Plotted with Golden Software’s GrapherTM v20.

## 8.3. Worked examples

### 8.3.1. Worked Example 1: Levelling stream sediment results for K2O from the G−BASE central and eastern England area

The preceding discussion of the X-Y plotting procedure is best explained by using a real example taken from the BGS G-BASE project, as documented in detail in the G-BASE data conditioning procedures manual (Lister and Johnson, 2005). This example shows levelling arising out of routine inspection of control data before the preparation of a map sheet area sampled and analysed over several years. The data used in this example for K2O results are given in Table 8.2, categorised by laboratory batch number.

Table 8.2. K2O results of SRM S15B determined by WD-XRFS used for plotting the time series (Shewhart) chart of Figure 8.5. Note that the different aliquots of S15B are given regular sample numbers and are inserted in the analytical batches as routine samples not recognisable by the laboratory. The laboratory must provide the applied geochemist with the recorded results as extracted from the analytical instrument with the laboratory batch number and date of analysis. The colours denote the different batches analysed at different dates.

| Sample No. | SRM | Laboratory batch No. | Date | K2O% WD-XRFS |
| --- | --- | --- | --- | --- |
| 440022 | S15B | 10372-0010 | 26/6/2003 | 3.04 |
| 440687 | S15B | 10372-0225 | 6/7/2003 | 3.03 |
| 440822 | S15B | 10372-0260 | 7/7/2003 | 3.02 |
| 441087 | S15B | 10372-0341 | 12/7/2003 | 3.04 |
| 441622 | S15B | 10377-0052 | 22/7/2003 | 2.84 |
| 441887 | S15B | 10377-0109 | 25/7/2003 | 2.86 |
| 442128 | S15B | 10377-0196 | 29/7/2003 | 2.83 |
| 440287 | S15B | 10372-0102 | 9/10/2003 | 3.03 |
| 440422 | S15B | 10372-0150 | 13/10/2003 | 3.03 |
| 444022 | S15B | 10587-0022 | 25/12/2003 | 2.75 |
| 442422 | S15B | 10587-0026 | 25/12/2003 | 2.83 |
| 444217 | S15B | 10587-0064 | 28/12/2003 | 2.79 |
| 442928 | S15B | 10586-0190 | 15/1/2004 | 2.74 |
| 443196 | S15B | 10586-0280 | 23/1/2004 | 2.77 |
| 443417 | S15B | 10586-0346 | 28/1/2004 | 2.78 |
| 443668 | S15B | 10586-0417 | 2/2/2004 | 2.76 |
| 443947 | S15B | 10586-0509 | 16/2/2004 | 2.76 |
| 444617 | S15B | 10719-0109 | 27/5/2004 | 2.74 |
| 445087 | S15B | 10719-0373 | 16/6/2004 | 2.74 |
| 445622 | S15B | 10721-0097 | 2/7/2004 | 2.73 |
| 445828 | S15B | 10994-0048 | 27/12/2004 | 2.72 |
| 446447 | S15B | 10994-0323 | 19/1/2005 | 2.74 |
| 447028 | S15B | 10995-0074 | 9/2/2005 | 2.68 |

The British Geological Survey (BGS) carried out a high-resolution geochemical mapping programme of the United Kingdom (Johnson et al., 2005) that commenced in 1969 and was finally completed in 2013, a span of 44 years. Figure 8.2a shows a map of the annual G-BASE field campaigns from 1969 to 2013, each colour representing a different field campaign year as the sampling progressed generally from north to south during the summer months. Within an annual field campaign, the 1000s of samples collected would have been analysed in more than one analytical batch and these are not distinguished on the field campaign map. Figure 8.2a shows how the sampling campaigns were often carried out constrained by map sheet boundaries. Prior to the production of each map sheet geochemical atlas, data levelling was undertaken to give seamless geochemical maps and to avoid the analytical bias relating to map-sheet boundaries, as is seen in Figure 8.1. This was achieved by studying the results from the SRMs submitted blind to the laboratories for each element in turn to identify any analytical bias introduced during a field campaign or analytical batch. The G-BASE analytical batch consisted of 500 samples, comprising five batches of one hundred samples submitted from the field (field batch) and including two SRMs per hundred samples (Johnson et al., 2018). The SRMs chosen for inclusion were selected to be of similar chemical composition to the samples being submitted and a number of different SRMs per laboratory batch were inserted to give a range of element concentrations to determine any analytical bias.

The data levelling was the last of the G-BASE data conditioning procedures (Lister and Johnson, 2005), performed before preparing the final atlas sheet data sets. The levelling proceeded as follows:

- Gather the geochemical results for inspection. Analytical batches for all the samples of the map sheet to be levelled are downloaded from the laboratory results database. From this, all the SRM results are extracted. The SRM samples, when submitted for analysis, were assigned normal sample numbers to be kept blind from the analyst and, thus, the SRM identity has to be reassigned, a process helped when the analytical results are merged with the field database (which identifies control samples).

- Plot time series charts. Time series charts (Shewhart plots) like that shown in Figure 8.5 (Table 8.2) are plotted for each SRM for every element determined. With more than forty elements and five SRMs, this gives two hundred time series charts. So, this plotting needs to be automated with a systematic naming convention for the computer plot files generated. Such software was not available for much of G-BASE’s history and plots were done individually initially by hand and later in MS Excel.

Figure 8.5. A time series (Shewhart) plot showing the K2O% results (Y-axis) for secondary reference material S15B plotted against the date of analysis (refer to Table 8.2 for results). The blue dashed horizontal line represents the accepted value for SRM S15B. These results were generated when the BGS XRF laboratory was undergoing major changes during the latter half of 2003. Analytical differences can be seen in three groups, namely, batches 10372 and 10377 and the remaining batches. The chart indicates clearly some analytical bias between batches during 2003. Plotted with Golden Software’s GrapherTM v20.

- Look for significant changes in the time series charts. Each chart is inspected for significant changes from a horizontal trend and if deviations are seen in several or more elements and for the different SRMs, then there is clearly an analytical problem introducing analytical bias. Figure 8.5 shows an example of this procedure. Further investigation revealed that in the second half of 2003 the XRF machine started to develop problems, which eventually led to the X-ray tube being replaced and the instrument being recalibrated. The laboratory batches affected are identified and the SRMs from these batches are used to level the results.

- Make X-Y plots for the SRMs for each element requiring levelling. For each affected laboratory batch X-Y plots are plotted and inspected with reference to Figure 8.4 to assess the nature of the levelling required or if indeed levelling can be done. In the worked example given here, three groups of affected samples are identified, namely, batch 10372, batch 10377 and batches from after 26/12/2003 (Fig. 8.5). X-Y plots for the three groups, tabulated in Table 8.3, are given in Figures 8.6 to 8.8 (prepared initially by using the MS Excel scatterplot graph option with inserted trend line and equation; the charts were subsequently plotted with Golden Software’s Grapher v20 for this Manual). All three plots show a very close fit of the data to the regression line. The coefficient of determination, R2, is very close to 1.0. The closeness to the 45-degree (1:1) reference line on the X-Y plots shows the levelling to the results will be small but is subtly different for the different batches.

Table 8.3. Results for different SRMs were extracted from analytical batches before the commencement of levelling of the central and eastern England map sheets. The table below represents the batches identified on the Shewhart charts of Figure 8.6: (a) Laboratory Batch 10372 (Fig. 8.6). (b) Laboratory Batch 10377(Fig. 8.7). (c) Laboratory batches after 26/12/2003 (Fig. 8.8), all charts plotted against (d) The Accepted SRM value.

| SRM ID | (a) Batch 10372 | (b) Batch 10377 | (c) Batches after 26/12/2003 | (d) Accepted SRM value |
| --- | --- | --- | --- | --- |
|  | X-axis K2O % values in different batches |  |  | Y-axis values |
| S13 | − | 2.078 | 2.006 | 2.170 |
| S13B | 2.324 | 2.283 | 2.227 | 2.290 |
| S15B | 3.030 | 2.843 | 2.752 | 2.940 |
| S23B | 3.790 | 3.790 | 3.784 | 3.860 |
| S58S | 2.144 | 1.993 | 1.896 | 2.040 |

- Apply the proposed correction factors. The regression equations determined for each of the three groups of samples (given in Figs. 8.6 to 8.8) are used to correct the data from the respective analytical batches. All the results in the batch (control samples included) for every element identified as requiring correction. The following shows how the SRM sample S15B is corrected in each of the three groups given in this worked example. All the sample K2O results from each analytical batch will be similarly corrected in a similar manner. It is stressed that this is a tedious procedure and must be carried out with the utmost care.

Batch 10372: K2O = 3.030%, regression equation Y = 1.0811 * (X – 0.2685);

corrected value = (1.0811 * 3.030) – 0.2685 = 3.007% K2O

Batch 10377: K2O = 2.843%, regression equation Y = 1.0455 * (X – 0.0796);

corrected value = (1.0455 * 2.843) – 0.0796 = 2.893% K2O

Batches after 26/12/2003 (from new, recalibrated X-Ray tube):

K2O = 2.752%, regression equation Y = 1.0062 * (X + 0.0872);

corrected value = (1.0062 * 2.752) + 0.0872 = 2.856% K2O

Figure 8.6. Regression analysis of the K2O results for SRMs from laboratory batch 10372 (using results in Table 8.3a initially in MS Excel to plot X-Y values and fit a linear trend line and regression equation). The green line indicates the 45-degree (1:1) diagonal, and the red line is the fitted linear correlation line. R is the linear correlation coefficient, and R2 is a statistical measure of how close the data are to the fitted regression line. Plotted with Golden Software’s GrapherTM v20.

Figure 8.7. Regression analysis of the K2O results for SRMs from laboratory batch 10377 (using results in Table 8.3b initially in MS Excel to plot X-Y values and fit a linear trend line and regression equation). The green line indicates the 45-degree (1:1) diagonal, and the red line is the fitted linear correlation line. R is the linear correlation coefficient, and R2 is a statistical measure of how close the data are to the fitted regression line. Plotted with Golden Software’s GrapherTM v20.

Figure 8.8. Regression analysis of the K2O results for SRMs from all batches after 26/12/2003 (using results in Table 8.3c initially in MS Excel to plot X-Y values and fit a linear trend line and regression equation). The green line indicates the 45-degree (1:1) diagonal, and the red line is the fitted linear correlation line. R is the linear correlation coefficient, and R2 is a statistical measure of how close the data are to the fitted regression line. Plotted with Golden Software’s GrapherTM v20.

- Replot the time series chart with the corrected results. The time series chart can be replotted for each SRM and element. Figure 8.9 visually shows the extent to which the various batches have been corrected. The most significant change is the ca. 0.1% K2O raising of concentrations for samples analysed after 26/12/2003 when the new XRF instrument was recalibrated.

- Plot geochemical map sheets with corrected results. Once the results for a map sheet area have been corrected by levelling to the satisfaction of the applied geochemist, the geochemical maps can be produced. Figure 8.10 shows the regional geochemical map of K2O for the area covered by the worked example above. Here the changes are subtle such as the raising of areas to the next percentile level as shown within the red circle. For environmentally sensitive elements, such as As, this raising to a higher percentile level could have significant implications for the area affected. The impact of the levelling procedure shown here, arising from anomalies as seen in a time series chart, and caused by instrument recalibration, is not as striking as the changes observed when levelling across campaign/map boundaries as shown in Figure 8.1. Plotting the geochemical images is an important part of the iterative process to level the data. It is important to look at the images in the context of other parameters (e.g., sampling campaign boundaries or analytical batches) that may cause analytical artefacts.

Figure 8.9. Time series plot (Shewhart plot) for SRM S15B for K2O% showing the original and corrected results from Table 8.4 (from Johnson et al., 2018, Fig. 5.13, p.96). Plotted with Golden Software’s GrapherTM v20.

Table 8.4. The original and modified K2O% results of SRM S15B are displayed in the time series (Shewhart) plot of Figure 8.9. Note that SRM S15B was inserted as a routine sample in the analytical batches and blind to the laboratory. The results were subsequently extracted from the different batches, as indicated by the Laboratory No. and date. Refer to text for explanations.

| Sample No. | SRM | Laboratory No. | Date | K2O% WD-XRFS | Modified K2O |
| --- | --- | --- | --- | --- | --- |
| 440022 | S15B | 10372-0010 | 26/6/2003 | 3.04 | 3.02 |
| 440687 | S15B | 10372-0225 | 6/7/2003 | 3.03 | 3.01 |
| 440822 | S15B | 10372-0260 | 7/7/2003 | 3.02 | 3.00 |
| 441087 | S15B | 10372-0341 | 12/7/2003 | 3.04 | 3.02 |
| 441622 | S15B | 10377-0052 | 22/7/2003 | 2.84 | 2.89 |
| 441887 | S15B | 10377-0109 | 25/7/2003 | 2.86 | 2.91 |
| 442128 | S15B | 10377-0196 | 29/7/2003 | 2.83 | 2.88 |
| 440287 | S15B | 10372-0102 | 9/10/2003 | 3.03 | 3.01 |
| 440422 | S15B | 10372-0150 | 13/10/2003 | 3.03 | 3.01 |
| 444022 | S15B | 10587-0022 | 25/12/2003 | 2.75 | 2.85 |
| 442422 | S15B | 10587-0026 | 25/12/2003 | 2.83 | 2.93 |
| 444217 | S15B | 10587-0064 | 28/12/2003 | 2.79 | 2.89 |
| 442928 | S15B | 10586-0190 | 15/1/2004 | 2.74 | 2.84 |
| 443196 | S15B | 10586-0280 | 23/1/2004 | 2.77 | 2.87 |
| 443417 | S15B | 10586-0346 | 28/1/2004 | 2.78 | 2.88 |
| 443668 | S15B | 10586-0417 | 2/2/2004 | 2.76 | 2.86 |
| 443947 | S15B | 10586-0509 | 16/2/2004 | 2.76 | 2.86 |
| 444617 | S15B | 10719-0109 | 27/5/2004 | 2.74 | 2.84 |
| 445087 | S15B | 10719-0373 | 16/6/2004 | 2.74 | 2.84 |
| 445622 | S15B | 10721-0097 | 2/7/2004 | 2.73 | 2.83 |
| 445828 | S15B | 10994-0048 | 27/12/2004 | 2.72 | 2.82 |
| 446447 | S15B | 10994-0323 | 19/1/2005 | 2.74 | 2.84 |
| 447028 | S15B | 10995-0074 | 9/2/2005 | 2.68 | 2.78 |

Figure 8.10. Percentile geochemical maps for K2O in stream sediment samples from central and eastern England. The original results are shown on the top map, the bottom map includes the levelled results. The changes are subtle such as the raising of areas to the next percentile level as shown within the red circle (from Lister and Johnson, 2005, Fig. 39, p.43).

### 8.3.2. Worked Example 2: Levelling stream sediment results for lanthanum (La) from northern Scotland determined by DC-OES and ICP-MS

Regional geochemical mapping of northern Britain began in 1969 and the analyses of stream sediment samples were carried out by direct current optical emission spectroscopy (DC-OES). Lanthanum (La) was not in the original suite of elements analysed though it was added to the analytes determined several years into the project. Some thirty years later, excess material from these original stream sediment samples was analysed by inductively coupled plasma mass spectrometry (ICP-MS) and La was one of the reported analytes. In Figure 8.11, the area of northern Scotland where La in the stream sediment samples was determined by ICP-MS is put into the context of the large area of northern Britain where La was determined by DC-OES.

The results by the ICP-MS method were reported in two discrete laboratory batches, namely Consignment A and Consignment E.

To align the ICP-MS data to the existing error-controlled and conditioned DC-OES data, an alternative levelling methodology, using cumulative percentiles was employed.

- The percentiles of existing DC-OES data from the area to the south of the ICP-MS data area were estimated for the 5th, 10th, 20th, 30th, 40th, 50th, 60th, 70th, 80th, 90th and 95th percentiles. These were then plotted against the ICP-MS data for the same percentiles to determine levelling factors based on comparative percentile distributions over like lithologies (Everett et al., 2019). This was done independently for data from consignments A and E to give two separate regression equations. This is, therefore, an example of parametric data levelling using percentile values from old (accepted) data and new data to be levelled rather than based on reference material results. The percentile values determined are shown in Table 8.5.

| (a) | (b) |
| --- | --- |

Figure 8.11. Map of northern Britain showing the areas where La has been determined in stream sediment samples by different analytical methods with a 30-year time gap (dark shading). (a) La was determined by DC-OES, and (b) La was determined by ICP-MS.

Table 8.5. Percentiles were calculated for the new ICP-MS La results (consignments A and E) and the original DC-OES results from northern Britain. Consignments A and E contained 2,822 and 1,824 samples, respectively. The original DC-OES data is for 64,530 samples.

| Percentile | Consignment A ICP-MS La (mg/kg) | Consignment E ICP-MS La (mg/kg) | DC-OES La (mg/kg) |
| --- | --- | --- | --- |
| 5 | 3 | 7 | 34 |
| 10 | 6 | 9 | 37 |
| 20 | 9 | 12 | 41 |
| 30 | 11 | 15 | 44 |
| 40 | 14 | 17 | 45 |
| 50 | 16 | 19 | 48 |
| 60 | 19 | 21.2 | 50 |
| 70 | 23 | 25 | 53 |
| 80 | 30 | 30 | 59 |
| 90 | 41 | 43.8 | 69 |
| 95 | 59 | 62.6 | 79 |

- X-Y plots of the data from Table 8.5 are shown in Figure 8.12 for the ICP-MS consignments A and E (prepared initially by using MS Excel scatterplot graph option with inserted trend line and equation; the charts were subsequently plotted with Golden Software’s Grapher v20 for this Manual). From the regression equations, a correction factor was calculated to level the ICP-MS La results with respect to the DC-OES data.

- Where Y is the revised La result and X is the original ICP-MS, the levelling can be carried out for all the newly determined ICP-MS samples, namely:

Consignment A: Y = 0.8089X + 33.831

Consignment E: Y = 0.8135X + 31.471

Figure 8.12. Regression analysis (X-Y plots) for the two ICP-MS consignments (A & E) comparing percentiles with the original DC-OES results from northern Britain (refer to Table 8.5). The green line indicates the 45-degree (1:1) diagonal, and the blue in (a) and red in (b) the fitted linear regression lines. R is the linear correlation coefficient, and R2 is a statistical measure of how close the data are to the fitted regression line. Plotted with Golden Software’s GrapherTM v20.

- The levelled data can be plotted as a seamless geochemical image (Figure 8.13) based on cumulative percentiles regressions of the two discrete batches of ICP-MS data against the pre-existing error controlled and conditioned DC-OES data.

The worked examples show a procedure to level data when no adequate control samples are available to use the more straightforward method given in Example 1. In such instances, it is necessary to try a range of levelling techniques and have a good understanding of the possible constraints associated with the results being merged. The percentile classification in Table 8.5 shows the higher concentrations of La resulting from the use of a ‘total’ analytical method (DC-OES) rather than a ‘partial’ method in which the sample requires a digestion process before its determination is done (ICP-MS). Furthermore, the levelling method assumes similar geological constraints on the distribution of La on either side of the boundary of the two sets of data. In this example, the seamless fit achieved across the joining boundary and the close resemblance of the La and uranium (U) geochemical distribution in northern Scotland (Everett et al., 2019) is satisfactory evidence that the levelling procedure was appropriate.

| (a) | (b) |
| --- | --- |

Figure 8.13. Lanthanum in stream sediment samples from northern Britain based on combined data sets from La determined by DC-OES and ICP-MS. Maps with (a) Unlevelled ICP-MS La results, and (b) Levelled ICP-MS La results.

Other levelling procedures are described in Darnley et al. (1995, Section §8.4, p.75-78), and more recently by Amor et al. (2019) in an example using lake-sediments from western Labrador and north-eastern Québec, Canada. Amor et al. (2019) describe a data set of 21,678 samples with disharmony in the original database arising from analyses employing digestions of differing strengths. Nearest-neighbour regression, quantile regression, and moving-median smoothing levelling methods are described by Amor et al. (2019). Another detailed example of quantile levelling is given by Daneshfar and Cameron (1998). Here the method is more refined using a weighted regression so the outer quantiles, which have the greater error, have less influence in estimating the regression coefficients. Note that the quantile is the value corresponding to a given fraction of the data and is an alternative expression for percentile when the result is expressed in fractions.

Apart from the normal statistical levelling methods discussed above, there are two additional methods in the literature, the first using geostatistics and the second the compositional nature of geochemical data. Lark et al. (2019) describe a multivariate geostatistical model, the linear model of coregionalisation (LMCR), to join the spatial distribution of two data sets, namely the English Representative Soil Sampling Scheme with 6620 samples (Church and Skinner, 1986) and the National Soil Inventory of England and Wales with a total of 5586 samples (McGrath and Loveland, 1992). Williams (2021) proposes a method for levelling multi-element geochemical data sets by levelling all elements and their covariances, simultaneously across all component map sheets by taking full account of the compositional nature of the data. The data sets used were the 28 elements determined by ICP-MS on 23,517 stream sediment samples, which are located in 34 separate Canadian 1:250,000 map sheets (Mackie et al., 2017). It has been shown that this method is computationally feasible, requiring manageable execution time even for large numbers of elements and large numbers of component surveys.

Most likely other statistical or geostatistical levelling and normalisation methods of existing data sets will be published in the future, and the applied geochemist should be careful, as pointed out by Darnley et al. (1995, p.75), that these are ‘geochemically blind’ because they simply manipulate numbers.

## 8.4. Other strategies required to future-proof data over long periods

### 8.4.1. Improved lower detection limit

A significant problem in combining geochemical data sets generated several decades apart is dealing with changes in how missing, semi-quantitative and unreliable results are dealt with (Johnson et al., 2018). This is particularly the case for the semi-quantitative (‘less than’) reporting of results falling below an analytical method’s lower detection limit (LDL).

Firstly, it is important to be aware of the distinction between recorded and reported results (AMC, 2001a, b). Recording of results is a listing of the data as produced by the analytical instrument, including negative values and results that may be below the laboratories cited LDL (uncensored analytical data). The reporting of results is a contractual matter between the analytical laboratory and the customer receiving the results. For example, the results may be edited by the laboratory, so all recorded results of an element that fall below the cited LDL are reported as ‘<LDL’ (censored data), and if a quantitative value is required for statistical analysis, a value ‘half the LDL’ may be substituted. This introduces a significant statistical bias at the lower end of the analyte’s data distribution. A similar analytical bias is introduced if results are rounded, say to the nearest mg/kg, giving a stepped data distribution. A geochemical database must comprise entirely of recorded results and not a mixture of recorded and reported data sets. Sufficient metadata needs to be attached to the data set so that, over a long period, the derivation of the reported results from the recorded results can be understood, reproduced or even modified. This helps to accommodate improvements to analytical precision, and results from areas produced over different periods may have different LDLs and methods of representing semi-quantitative data, described in the associated metadata.

An example from the BGS G-BASE programme is given in Figure 8.14 showing how the changes in the analytical method for the determination of Al in stream water samples greatly reduced the analytical bias at the lower concentrations of the Al data distribution (see other examples in Chapter 7 of this Manual). The change from the ICP-AES to ICP-MS analytical method resulted in a more than ten-fold improvement in the LDL. This meant that, over a long time, when all the results were put together into a single data set to plot a regional geochemical map, a strategy was required to produce a meaningful geochemical image (see Fig. 8.15). It is not possible to generate a seamless geochemical map for this situation, but a common percentile classification can be used for the higher Al results. However, the lower (below detection limit results) for the ICP-AES method have to be distinguished as a different classification on the map giving a non-seamless but honest representation of the results. The consequence is that the Al stream water results for Wales and northern England cannot be reported with the same high degree of resolution as the results for central and eastern England. A seamless map could be produced by using the poorer ICP-AES LDL for all the data giving a consistent lowest percentile class across the entire region. This would be at the expense of seeing meaningful patterns in the lower end of the Al data distribution in the central and eastern England area and degrading the high-quality modern data.

A similar non-seamless map would be generated using early geochemical results reported with less resolution, for example, to the nearest mg/kg, and later reported results with better resolution, say, for example, to one or more decimal places. A seamless geochemical map could be generated by reducing the precision of the later data (i.e., rounding to 0 decimal places) but again this would result in the loss of recognition of meaningful element distributions. Alternatively, without any degradation of the data set with better precision, a careful choice of percentile class thresholds can be used to minimise the effect of the ‘stepped’ data generated from the older and poorer analytical methodology.

Figure 8.14. Cumulative probability plot indicating true detection limits for samples determined by two different analytical methods. Such plots can demonstrate that the real detection limits (i.e., the point at the lower end of the cumulative distribution curve where the curve becomes horizontal) are often much lower than those cited by the analyst (the cited detection limits are marked on the plot). Recorded data for 10,000 samples from part of central and eastern England (from Johnson et al., 2018, Fig. 5.5, p.88).

It should also be noted here that the analysts generally report more conservative LDLs than the real LDLs that can be determined from the distribution plots or estimated as the practical detection limit (see Chapter 7 of this Manual). From the cumulative probability plot shown in Figure 8.14, the real LDL for the two analytical methods is the point where the lower end of the curves becomes flat, and the data distribution is truncated. Figure 8.14 shows that there is a significant structure to be found in the results reported between the cited LDL and the real LDL and this emphasises the importance of using the recorded rather than the reported analytical results. In this example, the reported data (for both ICP-AES and ICP-MS) would have been given as less than (<) the cited LDL losing a lot of interesting distribution information at the lower concentrations of Al in stream water.

Figure 8.15. Aluminium in stream water samples from England and Wales from the G-BASE project illustrates how data sets with different lower detection limits can be combined (from Johnson et al., 2018, Fig. 5.14, p.98).

Important note 2: The laboratory MUST provide the applied geochemist with the recorded results as produced by the analytical instrument, including negative values (uncensored data). The geochemical database MUST NOT include a mixture of recorded and reported results.

Important note 3: Secondary Reference Materials (SRMs) of different chemical composition must always be introduced systematically in the analytical batches of geochemical surveys as their results can be used in the future for levelling disparate data sets. At least five different SRMs will be needed for each sample type to cover the concentration ranges of all elements. Hence, large quantities of each SRM are required to be made to cover the duration of long-running surveys such as the Global Geochemical Reference Network project, and enough material should remain for future use and for standardising new SRMs.

### 8.4.2. Monitoring changes to the geochemical baseline over long periods

It is important to distinguish between changes in the geochemical baseline arising from different methodologies used to quantify that baseline, and the real environmental changes that occur over time, both natural and anthropogenic. For example, the cessation of domestic coal burning has significantly reduced the airborne input of some metals to soil (Haygarth, 1994; McConnell and Edwards, 2008). Long term monitoring of key national sites is, therefore, an important part of future-proofing regional geochemical mapping programmes. This is something that is relevant to a wide range of environmental sciences and needs to be coordinated nationally between environmental institutions and organisations. In the United Kingdom, the Park Grass experiment at Rothamsted Research, Harpenden, Hertfordshire, was initiated in 1856 and is the longest-running experiment on permanent grassland in the world (Silvertown et al., 2006). The experiment was originally designed to investigate the impacts of agriculture but has provided a soil sample archive that has helped to record changes to the geochemistry of the soil (e.g., Zhao et al., 1998; Warneke et al., 2002; Blake et al., 1999; Hartley et al., 2013; Bowley et al., 2017).

With the establishment of standards for the global geochemical baseline mapping of the environment since the mid-1990s (Darnley et al., 1995) more long-term monitoring sites need to be established globally to monitor changes to the soil, stream sediment and stream water. This is particularly relevant to monitoring the changes caused by global-scale industrialisation and urbanisation and accompanying natural climate change. Only by adding a time dimension to the geochemical baseline can the long term future-proofing of global baseline data be achieved.

## 8.5. Recommendations

This chapter demonstrates the value of routinely and systematically including SRM samples in analytical batches to enable sets of geochemical results produced over a long period to be combined to yield seamless geochemical maps. Methods of chemical analysis will evolve during the lifetime of a long-running project to achieve better precision and accuracy in the measurement of the natural geochemical variability. This will generate some incompatibility with earlier data sets that can be rectified with the use of SRMs. If a national or a global geochemical data set has been rapidly generated, using standardised and consistent methodology throughout, no ‘internal’ levelling is necessary. It may well be used, however, in the context of other data sets of lesser quality, and in such cases, SRMs from the high-quality data set are still required for the levelling process.

A further very important consideration in any long-running geochemical survey is the financial budget. A fluctuating budget over the lifetime of a project will have an impact on the consistency of data quality. In the case of the BGS’s G-BASE project, there were many changes to the annual budget over four decades. A limitless budget is a rare occurrence for any geochemical survey and the analytical scheme is usually a compromise to achieve the best value for the available budget. The inclusion of a sufficient number of SRMs throughout a project’s duration is an insurance policy that ensures the final product is universally applicable and defensibly consistent. This is particularly important if data are to be applied in a legislative or regulatory context.

Regarding the data levelling procedure, it is recommended that:

- Sufficient time must be allocated for the data conditioning of the geochemical results. The final part of this process is testing whether the results need to be levelled to make them consistent with earlier analytical batches or with other external data with which they may be combined or compared, provided the same SRMs have been used. It is our experience from over four decades of generating large regional geochemical baseline data sets, for many countries around the globe, that the data conditioning process is generally under-resourced and not understood by project managers who have no geochemical experience. Making geochemical maps and X-Y plots for a data set, which may contain results for more than fifty elements, can be very time-consuming. Consistent and rapid output is best achieved by automating the plotting procedures using application software on standard formatted data files containing SRMs results and their accepted values (e.g., R scripts (Reimann et al., 2008) or MS Excel macro routines) – see the example MS Excel file in the Supplementary material.

- The SRMs should be of a similar material to the sample types being determined. In some analytical methods, high concentrations of elements can lead to interference in the determination of other elements so these control samples should experience the same interferences as the real samples. Any national or global geochemical survey should consider at the commencement of a large geochemical and long-running project the need to develop a set of SRMs (see Chapter 5 of this Manual). Experience from past geochemical mapping projects would suggest that at least five different SRMs will be needed for each sample type to cover the concentration ranges of all elements (e.g., see the range of different SRMs used by the G-BASE project in Table 8.3).

- A number of different SRMs should be routinely analysed in every batch of samples so that the X-Y plots, such as those shown in Figures 8.6 to 8.8, provide a meaningful regression analysis over a range of concentrations for all the elements determined. The number of SRMs inserted into a batch of 100 samples should be a minimum of two if the active project is collecting 1000s of samples. If the project is collecting 100s of samples then this should be increased to four. If the number of samples collected is small (<100) then between 1 in 20 samples and 1 in 10 samples should be an SRM. The number of different SRMs (see point 2 above) submitted in an entire analytical batch of samples must be sufficient to produce satisfactory regression analysis plots.

- The process of submitting SRMs in batches of samples for analysis should be done independently of the analytical laboratory with the SRMs being submitted to the laboratories in the guise of normal samples and hence ‘blind’ to the analyst. The analytical contract should include a requirement for the laboratory to routinely determine primary reference materials (PRMs) usually at a rate of four PRMs at the start and end of each analytical batch. This is a procedure the laboratory must do to confirm for itself, as well as to its customer, that the results generated are of acceptable quality. The results of the PRMs can also be used to ensure the final levelling of a completed data set is done to an international standard reference point (e.g., Everett et al., 2018), provided that the same PRMs have been used throughout a long-running project.

- The geochemistry database storing the geochemical results needs to be carefully designed so it captures all versions of the geochemical results a project will generate. A project will initially receive laboratory results that will be subjected to data conditioning to produce a project geochemical database, which may be subjected to one or more phases of levelling. It needs to be able to store the original results as received from the laboratory plus the results after they have been verified and conditioned to deal with missing or semi-quantitative results (Johnson et al., 2018). Then for each phase of data levelling, database tables associated with version control need to contain the levelling factors determined by the parametric levelling. In this way, a single geochemical database can be used to access all the versions of the geochemical results from those received from the laboratory to the latest version of levelled data. Most importantly, control data (duplicates, replicates, blanks, PRMs and SRMs) and recommended levelling factors should also be included in the database so future researchers can carry out any further levelling independently of the originators of the original results. This will give a clear audit of any data modifications.

- National monitoring sites for all types of environmental samples should be set up and coordinated internationally for recording changes to the geochemical baseline over time. As global geochemical baseline mapping becomes established and the recorded baselines covers many decades, applied geochemists must be in a position to monitor changes to the baseline with time. The data levelling procedures described here to combine results collected over a long period assume that the geochemical baseline has remained the same. However, urbanisation and industrialisation do change the natural geochemical baseline (Johnson et al., 2011), and more rapid climate change will also cause significant changes in the foreseeable future. It is, therefore, important that long-term national monitoring sites covering a range of environmental materials are set up and coordinated on a global scale. In this way, the process of levelling data between projects can be refined to include real changes to the geochemical baseline over time.

## Supplementary material

The following MS Excel file can be downloaded from the Publications web page of the IUGS Commission on Global Geochemical Baselines:

- IUGS-CGGB_Chapter-8_Examples.xlsx.

The MS Excel file contains the data for plotting Figures 8.5, 8.6, 8.7, 8.8, 8.9 and 8.12.

The MS Excel file with the DUPREPPLOT routine is included in the Supplementary material of Chapter 7.

## References

Note: All hyperlinks were checked on the 30th of January 2022.

AMC, 2001a. What should be done with results below the detection limit? Mentioning the unmentionable. AMC (Analytical Methods Committee) Technical Brief No 5. Royal Society of Chemistry, United Kingdom, 2 pp.; https://www.rsc.org/images/results-below-detection-limit-technical-brief-5_tcm18-214854.pdf.

AMC, 2001b. Measurement of near zero concentration: recording and reporting results that fall close to or below the detection limit. Analyst, 126, 256−259; https://doi.org/10.1039/b009590g.

Amor, S., McCurdy, M. & Garrett, R., 2019. Creation of an atlas of lake-sediment geochemistry of Western Labrador and Northeastern Québec. Geochemistry: Exploration, Environment, Analysis, 19, 369−393; https://doi.org/10.1144/geochem2018-061.

Blake, L., Goulding, K.W.T., Mott, C.J.B. & Johnston, A.E., 1999. Changes in soil chemistry accompanying acidification over more than 100 years under woodland and grass at Rothamsted Experimental Station, UK. European Journal of Soil Science, 50, 401−412; https://doi.org/10.1046/j.1365-2389.1999.00253.x.

Bowley, H.E., Mathers, A.W., Young, S.D., Macdonald, A.J., Ander, E.L., Watts, M.J.,  Zhao, F.J., McGrath, S.P., Crout, N.M.J. & Bailey, E.H., 2017. Historical trends in iodine and selenium in soil and herbage at the Park Grass Experiment. Soil Use and Management, 33(2), 252−262; https://doi.org/10.1111/sum.12343.

Church, B.M. & Skinner, R.J., 1986. The pH and nutrient status of agricultural soils in England and Wales, 1969–1983. Journal of Agricultural Science, 107(1), 21–28; https://doi.org/10.1017/S0021859600066752.

Daneshfar, B. & Cameron, E., 1998. Leveling geochemical data between map sheets. Journal of Geochemical Exploration. 63, 189−201; https://doi.org/10.1016/S0375-6742(98)00015-6 .

Darnley, A.G., Bjorklund, A., Bølviken, B., Gustavsson, N., Koval, P.V., Plant, J.A., Steenfelt, A., Tauchid, M., Xuejing, X., Garrett, R.G. & Hall, G.E.M., 1995. A Global Geochemical Database for Environmental and Resource Management. Recommendations for International Geochemical Mapping – Final Report of IGCP Project 259. Earth Science Report 19.UNESCO Publishing, Paris, 122 pp.; http://globalgeochemicalbaselines.eu.176-31-41-129.hs-servers.gr/datafiles/file/Blue_Book_GGD_IGCP259.pdf.

Everett, P.A., Lister, T.R., Fordyce, F.M., Ferreira, A.M.P.J., Donald, A.W., Gowing, C.J.B.& Lawley, R.S., 2019. Stream sediment geochemical atlas of the United Kingdom.  British Geological Survey, OR/18/048, Keyworth, Nottingham, 94 pp.; http://nora.nerc.ac.uk/id/eprint/524956/.

Hartley, T.N., Macdonald, A.J., McGrath, S.P. & Zhao, F.J., 2013. Historical arsenic contamination of soil due to long-term phosphate fertiliser applications. Environmental Pollution, 180, 259−264; https://doi.org/10.1016/j.envpol.2013.05.034.

Haygarth, P.M., 1994. Global importance and cycling of selenium.  In: W.T. Frankenberger & S. Benson (Editors), Selenium in the Environment. Marcel-Dekker, New York, 1−28.

Heydorn, K., 2006. The determination of an accepted reference value from proficiency data with stated uncertainties. Accreditation and Quality Assurance, 10, 479−484; https://doi.org/10.1007/s00769-005-0043-2.

Johnson, C.C., Breward, N., Ander, E.L. & Ault, L., 2005. G-BASE: Baseline geochemical mapping of Great Britain and Northern Ireland. Geochemistry: Exploration-Environment-Analysis 5(4), 347−357; https://doi.org/10.1144/1467-7873/05-070.

Johnson, C.C., Demetriades, A., Locutura, J., & Ottesen, R.T. (Editors), 2011. Mapping the Chemical Environment of Urban Areas. Wiley-Blackwell, John Wiley & Sons Ltd., Chichester, West Sussex, United Kingdom, 616 pp.; https://doi.org/10.1002/9780470670071.

Johnson, C.C., Ander, E.L., Lister, T.R. & Flight, D.M.A., 2018. Data Conditioning of Environmental Geochemical Data: Quality Control Procedures Used in the British Geological Survey's Regional Geochemical Mapping Project. Chapter 5 In: B. de Vivo, H.E. Belkin & A. Lima (Editors), Environmental Geochemistry: Site Characterization, Data Analysis and Case Histories (Second Edition). Elsevier, 79−101; https://doi.org/10.1016/B978-0-444-63763-5.00006-9.

Lark, R.M., Ander, E.L. & Broadley, M.R., 2019. Combining two national-scale datasets to map soil properties, the case of available magnesium in England and Wales. European Journal of Soil Science, 70, 361−377; https://doi.org/10.1111/ejss.12743.

Lee, G.K., Yager, D.B., Mauk, J.L, Granitto, M., Denning, P.D., Wang, B. & Werdon, M.B., 2016. The Geochemical Atlas of Alaska, 2016. U.S. Geological Survey Data Series 908, 25 pp., 272 sheets (11” × 17”), GIS database; http://dx.doi.org/10.3133/ds908.

Lister, T.R. & Johnson, C.C., 2005. G-BASE data conditioning procedures for stream sediment and soil chemical analyses. British Geological Survey Internal Report, IR/05/150, 85 pp.; http://nora.nerc.ac.uk/id/eprint/11170/1/IR05150.pdf.

Mackie, R.A., Arne, D.C. & Pennimpede, C., 2017. Assessment of Yukon regional stream sediment catchment basin and geochemical data quality. Yukon Geological Survey, Open File 2017-4, 29 pp. and digital files; http://data.geology.gov.yk.ca/Reference/79430.

McConnell, J. R. & Edwards, R., 2008. Coal burning leaves toxic heavy metal legacy in the Arctic. Proceedings of the National Academy of Sciences of the Unites States of America, 105(34), 12140−12144; https://doi.org/10.1073/pnas.0803564105.

McGrath, S.P. & Loveland, P.J., 1992. The Soil Geochemical Atlas of England and Wales. Blackie Academic and Professional, Glasgow, 101 pp.

Reimann, C., Filzmoser, P., Garrett, R.G. & Dutter, R., 2008. Statistical Data Analysis Explained. Applied Environmental Statistics with R. John Wiley & Sons, Ltd., Chichester, England, 343 pp.

Reimann, C., Caritat, P. de, GEMAS Project Team & NGSA Project Team, 2012. New soil composition data for Europe and Australia: Demonstrating comparability, identifying continental-scale processes and learning lessons for global geochemical mapping. Science of the Total Environment, 416, 239−252; https://doi.org/10.1016/j.scitotenv.2011.11.019.

Silvertown, J., Poulton, P., Johnston, E., Edwards, G., Heard, M. & Biss, P.M., 2006. The Park Grass Experiment 1856-2006: its contribution to ecology. Journal of Ecology, 94(4), 801−814; https://doi.org/10.1111/j.1365-2745.2006.01145.x.

Smith, D.B., Smith, S.M. & Horton, J.D., 2013. History and evaluation of national-scale geochemical data sets for the United States. Geoscience Frontiers 4(2), 167−183. https://doi.org/10.1016/j.gsf.2012.07.002.

Warneke, T., Croudace, I.W., Warwick, P.E. & Taylor, R.N., 2002. A new ground-level fallout record of uranium and plutonium isotopes for northern temperate latitudes. Earth and Planetary Science Letters, 203(3-4), 1047−1057; https://doi.org/10.1016/S0012-821X(02)00930-5.

Weaver, T.A., Freeman, S.H., Broxton, D.E. & Bolivar, S.L., 1983. The Geochemical Atlas of Alaska: Los Alamos, New Mexico, U.S. Department of Energy, Los Alamos National Laboratory, Geochemistry Group, Earth and Space Sciences Division, Report nos. LA–9897–MS and GJBX–32 (83), DOE contract no. W–7405–ENG–36, 57 pp.

Williams, P.M., 2021. Statistical levelling of multi-element geochemical data. Applied Computing and Geosciences, 10, 100060; https://doi.org/10.1016/j.acags.2021.100060.

Zhao, F.J., Spiro, B., Poulton, P.R. & McGrath, S.P., 1998. Use of sulfur isotope ratios to determine anthropogenic sulfur signals in a grassland ecosystem. Environmental Science and Technology, 32(15), 2288–2291; https://doi.org/10.1021/es980157f.

Subsequently updated and revised by Lee et al. (2016)